3GPP TSG-RAN WG4 Meeting # 112-bis														R4-2416004

Hefei, China, 14 – 18 October 2024

Agenda item:			6.17.1

Source:	Samsung

Title:	Views on general aspect of AI/ML for NR air interface

Document for:	Discussion

1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item was approved to introduce the specification support for the aspects of AI/ML general framework, two confirmed use cases (i.e., beam management and positioning accuracy enhancements) and necessary RAN4 core requirements for the above two use cases and LCM procedure including performance monitoring [2].

Furthermore, RAN plenary confirmed again the two approaches for RAN4 in RAN#105. According to work item objectives [2], RAN4 is required to further study the interoperability and testability aspects as below:

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

Some progress has been made on the above-highlighted general aspects for testability and interoperability during the past meetings although they still have FFS or waiting for feasibility or other WGs’ discussion. Based on the progress and the agreement captured in [3], in this contribution, we would like to discuss the remaining issues focusing on the remaining topic in general agenda.

2. Discussion

The post-deployment validation for model change/drift are proposed and discussed to be considered from the late phase of Rel-18 study item as below.

a.	The changes/updates to the ML-enabled Functionality/Feature are tested and declared by the device vendor against RAN4 requirements before any deployment to the UE is performed.

b.	After deployment to the UE and before changed/updated ML-enabled Functionality/Feature is activated in the UE, a post-deployment validation is performed, e.g., a sanity check test loop is run, e.g., using the functionality performance monitoring and LCM activation/deactivation/switching procedures,

c.	At least one fallback/default Functionality/Feature that passed conformance testing must always be present in the device.

However, in our view, the discussion has just been focused on the necessity of the consideration rather than the feasibility of the validation, and it makes RAN4 continue the inconclusive discussions back and forth from Rel-18. Here is the history of the post-deployment handling [4, 5, 6].

<RAN4#110>

Issue 1-2: Post deployment handling Agreement: - To ensure the AI performance after device deployment, discuss the following options further - Option 1: Conduct the conformance testing for AI model/functionality before deployment - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded

<RAN4#111>

Issue 1-1: Post deployment testing Interested companies are invited to bring analysis on the feasibility of the options proposed so far.

<RAN4#112>

Issue 1-1: Post deployment testing Tentative agreement: - Option 1: Conduct the conformance testing for new/updated AI model/functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in) - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring and proactive recovery from potential performance degradation - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Option 3: Capture model input during conformance testing for later testing of new models. - Other options are not precluded

Observation 1:	RAN4 discussion on the post deployment testing has just been focused on the necessity of the consideration rather than the feasibility of the validation, and the feasibility has not been provided yet.

As noted above, during the past meetings, a couple of options has been discussed for the post deployment testing such as Option 1 and 2. Option 1 is about testing for new/updated AI model/functionality before its deployment in already deployed UEs, and Option 2 is kind of verifying the performance monitoring and proactive recovery from potential performance degradation. However, both are still under tentative agreement since there was no consensus on the feasibility and other WGs’ progress as mentioned earlier.

Observation 2:	RAN4 could not reach a consensus on the post deployment testing due to lack of feasibility and other WGs’ progress.

For Option 1, in our view, it is a normal practice that UE vendors shall be responsible to guarantee the changes/updates to their device after the deployment, and this applies to the ML-based functionality/feature (just like any legacy feature) for sure. Such enough in-house validation before any deployment should guarantee the model change/drift as any other features now after the deployment. Moreover, on top of the feasibility, it should be noted that reducing test time/burden would be another important aspects RAN4 should consider as the test requirement. For example, if RAN4 finally introduces to conduct the conformance testing for AI model/functionality before deployment, i.e., it will incur huge testing burden if the conformance testing shall be conducted in certificated labs for every AI/ML model update.

Observation 3: 	Option 1 of post-deployment handling will incur huge testing burden if the conformance testing shall be conducted in certificated labs for every AI/ML model update/transfer.

The post deployment handling for AI/ML can be considered based on monitoring and LCM discussion in other WG, and for Option 2, it is more like performance monitoring that is pretty much dependent on other WGs. However, even if RAN4 defines a test to verify the performance monitoring and proactive recovery mechanism based on the monitoring, it cannot guarantee the ML-enabled Functionality/Feature from the model change/drift after the deployment, and UE should have the same performance regardless of the testing that is even not easy to be defined in the specification. It would be meaningless for RAN4 to discuss the test for monitoring performance or monitoring based solutions without monitoring and LCM discussion in other WG, which would be for the same issue in the end.

Observation 4:	It would be meaningless for RAN4 to discuss the test for monitoring performance or monitoring based solutions without monitoring and LCM discussion in other WG, which would be for the same issue in the end.

Therefore, in our view, the conformance testing should include the validation testing performed by UE vendors in the development phase of AI/ML model, rather than formal conformance tests in certificated laboratories. Otherwise, for the post-deployment validation, no option should be adopted at this stage given the lack of feasibility and progress in other WGs. We believe that RAN4 can pause the post deployment discussion until the progress in other WG while keeping the tentative agreement with Option 1 and Option 2.

Proposal 1: 		Option 1 of post-deployment handling is preferred only if

- The conformance testing includes the validation testing performed by UE vendors in the development phase of AI/ML model, rather than formal conformance tests in certificated labs.

Proposal 2: 		Otherwise, RAN4 should pause the post deployment discussion until the progress in other WG while keeping the previous tentative agreement with Option 1 and Option 2.

The same principle could apply to other important topics such as testing goal and channel modeling from general aspects. Following agreements shall be kept as they are until further progress can be achieved in RAN4 and other WGs with specific requirements and procedures for each use case, for instance [6].

Issue 1-2: Channel modeling Agreement: - Use synthetic channels as the default assumption for conformance requirements/test. - RAN4 may conduct an analysis for each use case to determine the reliability of using synthetic channels for test data in evaluating models trained on real data. - The field data can be considered for the analysis. Issue 1-3: Testing goals Agreement: - The testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved - FFS on whether and how many different test configurations/parameters are used for tests. - LCM would be tested

Proposal 3: 		The latest agreements for general aspects, e.g., testing goal and channel modelling, shall be kept as they are until further progress can be achieved in RAN4 and/or other WGs.

3. Conclusion

In this contribution, we provided our viewpoints on the general aspects for testability and interoperability focusing on the remaining issues. Following observations and proposals are obtained:

Observation 1:	RAN4 discussion on the post deployment testing has just been focused on the necessity of the consideration rather than the feasibility of the validation, and the feasibility has not been provided yet.

Observation 2:	RAN4 could not reach a consensus on the post deployment testing due to lack of feasibility and other WGs’ progress.

Observation 3: 	Option 1 of post-deployment handling will incur huge testing burden if the conformance testing shall be conducted in certificated labs for every AI/ML model update/transfer.

Observation 4:	It would be meaningless for RAN4 to discuss the test for monitoring performance or monitoring based solutions without monitoring and LCM discussion in other WG, which would be for the same issue in the end.

Proposal 1: 		Option 1 of post-deployment handling is preferred only if

- The conformance testing includes the validation testing performed by UE vendors in the development phase of AI/ML model, rather than formal conformance tests in certificated labs.

Proposal 2: 		Otherwise, RAN4 should pause the post deployment discussion until the progress in other WG while keeping the previous tentative agreement with Option 1 and Option 2.

Proposal 3: 		The latest agreements for general aspects, e.g., testing goal and channel modelling, shall be kept as they are until further progress can be achieved in RAN4 and/or other WGs.

4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-240774 (revised from RP-234039), “New WID on Artificial Intelligence (AI)/Machine Learning (ML)
 	for NR Air Interface”, Qualcomm, RAN#103, March 2023.

[3] R4-2402388, “General aspect of AI/ML for NR air interface”, Samsung, RAN#110, Feb. 2024.

[4] R4-2403712, “WF on AI/ML”, Qualcomm, RAN4#110, Feb. 2024.

[5] R4-2406617, “WF on AI/ML”, Qualcomm, RAN4#111, May. 2024.

[6] R4-2414323, “WF on AI/ML”, Qualcomm, RAN4#112, Aug. 2024