3GPP TSG-RAN WG4 Meeting #112bis	R4-2416005

Hefei, China, 14th – 18th October, 2024

Agenda item:	6.1.1.3.3

Source:            	Google

Title:                	Discussion on SRS antenna switching requirements for 6Rx UE

Document for:	Approval

# Introduction

In RAN#103, a new WID on RF enhancements for NR FR1/FR2 and EN-DC was approved as one of Release 19 work items [1]. One of the objectives in the WID is to discuss 6Rx requirements for handheld and FWA UEs, which is shown as follows. In this contribution, we will focus on the SRS antenna switching requirements for 6Rx handheld and FWA UE.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In RAN4#112, the SRS antenna switching requirements for 6Rx handheld and FWA UE has been discussed and some open issues are captured in the approved WF [2]. In WF, the remaining main issue is about how to specify ΔTRxSRS value corresponding to t1r6 and/or t2r6 on band n79 and n104 for handheld and FWA UEs. In this paper, we would like to share our view in the following.

# Topic 2: SRS antenna switching and ΔTRxSRS ## Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Way forward: Further discuss the following options. Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Issue 2-1-2: Whether to consider separate ∆TRxSRS values when the device is power class 2 in the band and ΔPPowerClass = 0dB and not indicating Tx diversity capability Agreement: Have additional 3dB for power class 2 when ΔPPowerClass = 0dB and not indicating Tx diversity capability. ## Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers. - Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n41, n77, n78; 4.0; 3.5; 4.5. - Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79, n104; 5.4; 4.7; 5.9. - Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79; 5.4; 4.7; 5.9. Band n104; 5.7; 5.0; 6.3.

# Discussion

In Rel-15, SRS Tx switching feature is introduced to obtain more accurate channel state information (CSI) by utilizing channel reciprocity for TDD bands. The motivation to introduce SRS Tx switching feature is that Tx antenna port number is usually less than Rx antenna port number from UE implementation perspective. In other words, in order to obtain more accurate CSI for DL-MIMO transmission at the base station side, the UE can be scheduled to perform antenna switching for SRS transmission in the corresponding Rx antenna port sequentially instead of simultaneous SRS transmission on the corresponding Rx ports.

From UE capability signalling perspective, SRS Tx switching feature can be indicated by the corresponding IE srs-TxSwitch/srs-TxSwitch-v1610 with “xTyR”, e.g., 1T2R/1T4R/ 2T4R/2T2R/4T4R/1T4R-2T4R, where “x” is the capable antenna port for SRS transmission and “y” means the total number of receive antenna where SRS transmission can be performed for the target band or the target band in the band combination. In Rel-17, the xTyR has been extended to support at most “1T8R/2T8R/4T8R” in the corresponding IE srs-AntennaSwitchingBeyond4Rx-r17.

From implementation perspective, if the UE is configured to perform SRS antenna switching, SRS Tx transmission may go through different number of RFFE antenna switch components, e.g., SPXT, DPXT where X=2,3,4,…8, at each associated Rx path on PCB board, which may cause different insertion loss for each SRS Tx path. In addition, the PCB tracing loss may also be different from each SRS Tx associated Rx path. Hence, in order to accommodate both the SRS insertion loss and PCB tracing loss for different UE hardware implementation, RAN4 define ΔTRxSRS requirement within the lower bound of PCMAX equation which is shown as follows in TS 38.101-1 sub clause 6.2.4. The ΔTRxSRS value in the inequation means the max allowed power reduction when the UE performing SRS Tx antenna switching. According to different “xTyR” capability, the corresponding ΔTRxSRS values are defined in TS 38.101-1 sub clause 6.2.4.

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

PCMAX_H,f,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass + ΔPPowerBoost}

In RAN4#112, SRS antenna switching for 6Rx handheld and FWA UEs has been discussed and two remaining open issue about ΔTRxSRS values for n79 and n104 are captured in the WF. The first remaining issue is to determine whether a new break point for ΔTRxSRS should be introduced when FUL_high is higher than the FUL_low of n104. In our view, the band frequency distance between n79 and n104 is at least 1425 MHz (6425MHz-5000MHz), and the insertion loss including PCB tracing loss and RFFE antenna switch loss may become higher for n104. Hence, it is proposed to introduce the additional break point for bands whose FUL_high is higher than the FUL_low of n104.

Proposal 1: Considering that the insertion loss including PCB tracing loss and RFFE antenna switch loss may become higher for n104, it is proposed to introduce the additional break point for bands whose FUL_high is higher than the FUL_high of n104.

The other open issue is how to determine the ΔTRxSRS value for 6Rx handheld and FWA UE on the example bands. In the WF, it is agreed to adopt the average values from companies to specify the values for ΔTRxSRS for n41, n77 and n78. For the bands whose FUL_high is lower than the FUL_high of n104, two options for ΔTRxSRS value are captured in the WF that Option 10 is not to consider the additional breakpoint for the band n79 and n104 and Option 11 is to consider the additional break point for the band n79 and n104. In our view, same as the above discussion, the additional insertion loss needed to be considered for n104. Hence, it is proposed to specify the values for ΔTRxSRS by considering an additional breakpoint for bands whose FUL_high is lower than the FUL_high of n104 in the following table.

Proposal 2: Considering that the insertion loss including PCB tracing loss and RFFE antenna switch loss may become higher for n104, it is proposed to specify the values for ΔTRxSRS by considering an additional breakpoint for bands whose FUL_high is lower than the FUL_high of n104 in the following table.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n104 | 5.7 | 5.0 | 6.3 |

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: Considering that the insertion loss including PCB tracing loss and RFFE antenna switch loss may become higher for n104, it is proposed to introduce the additional break point for bands whose FUL_high is higher than the FUL_high of n104.

Proposal 2: Considering that the insertion loss including PCB tracing loss and RFFE antenna switch loss may become higher for n104, it is proposed to specify the values for ΔTRxSRS by considering an additional breakpoint for bands whose FUL_high is lower than the FUL_high of n104 in the following table.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n104 | 5.7 | 5.0 | 6.3 |

# Reference

- RP-240828, “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, RAN4 chair (Huawei), RAN#103

- R4-2414285, “WF on 6Rx UE requirements”, AT&T, RAN4#112

- R4-2412939, “Discussion on SRS antenna switching requirements for 6Rx UE”, Google, RAN4#112