3GPP TSG-RAN WG4 Meeting #112bis	R4-2416008

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	5.18.2

Source:	MediaTek Inc.

Title:	Discussion on IoT-NTN B252 UE RF requirements

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for IoT-NTN B252 UE RF requirements is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new E-UTRA NTN FDD band for NB-IoT/eMTC NTN operation, with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario; - Support UE Categories NB1, NB2, M1; - Support UE Power Class 3 (+23dBm) and Power Class 5 (+20dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. The specification work of this WI shall leverage the studies and requirements for NR/LTE NTN n256/B256, n255/B255, n254/B254, where applicable.

2.2		UE RF system parameters

In RAN4#112, the approved WF [3] provided the guidance for following works. The partial contents are listed below for reference.

Issue 2-2-2: Band Plan Agreement: - Adopt the proposed band plan as follows E-UTRA operating bands for satellite access E-UTRA Operating Band; Uplink (UL) operating band BS receive UE transmit; Downlink (DL) operating band BS transmit UE receive; Duplex Mode. FUL_low – FUL_high; FDL_low – FDL_high. 256; 1980 MHz; –; 2010 MHz; 2170 MHz; –; 2200 MHz; FDD. 255; 1626.5 MHz; –; 1660.5 MHz; 1525 MHz; –; 1559 MHz; FDD. 254; 1610 MHz; -; 1626.5 MHz; 2483.5 MHz; -; 2500 MHz; FDD. 2532; 1668 MHz; -; 1675 MHz; 1518 MHz; -; 1525 MHz; FDD. 252; 2000 MHz; -; 2020 MHz; 2180 MHz; -; 2200 MHz; FDD. NOTE 1: Satellite bands are numbered in descending order from 256 NOTE 2: UE assigned to channels and allocated frequency resources in the lower portion of Band 253 may experience blocking or harmful interference from terrestrial networks in adjacent or nearby frequencies when operating in the proximity with terrestrial base stations.. Issue 2-2-5: Applicable TN Bands for Coexistence Agreement: - Focus on coexistence of S-band UL with B2/n2 and B25/n25 DL. - Capture the clarifications on the co-existence issues with B70/n70 and B66/n66 and that there is no 3GPP solution for them in the TR Issue 2-2-6: TN-NTN UE-to-UE Coexistence Offline Agreement: - Further study the UE-to-UE coexistence between NTN S-band UL and B2/n2, B25/n25 DL, considering: - Realistic usage scenarios - consider (NTN) UE antenna gain for smartphone of -5.5dBi - UE-to-UE separations distance - probability of the UE-to-UE coexistence scenarios occurring - Companies are encouraged to examine the previous work in TR 38.863 for UE-to-UE coexistence noting that the solution might need to be re-evaluated for this work. - Companies are encouraged to investigate the background scenario for applicability of the -50dBm/MHz requirement to assess its applicability to TN-NTN coexistence (e.g. reference R4-080710) - Companies are also encouraged to investigate other prior work in UE-to-UE coexistence where different values from -50 dBm/MHz have been adopted Issue 2-3-3: Default TX-RX Separation Tentative Offline Agreement: - Adopt default TX-RX frequency separation as fixed value as a starting point. Further study whether flexible TX-RX frequency separation can be supported, following conclusions from UE RF maintenance. Issue 2-3-4: General Views on Impact to UE RF Requirements Tentative Offline Agreement: - Use the following merged table with common views on identified specification impact as a starting point. - Companies to further assess focusing on the missing or TBC sections. Clause/Section; Requirement; COMMON VIEWS. Clause 6; Transmitter Characteristics. 6.2A.1; UE maximum output power; Some impact. 6.2B.1. 6.2A.1; UE maximum output power tolerance; Some impact. 6.2B.1. 6.2A.2; UE maximum output power reduction (MPR); No Impact. 6.2B.2. 6.2A.3; UE additional maximum output power reduction; Some impact. 6.2B.3. 6.2A.4; Configured transmitted Power for category M1; No Impact. 6.2B.4; Configured transmitted Power for category NB1 and NB2; No Impact. 6.3:; Output power dynamics; No Impact. 6.4:; Transmit signal quality; No Impact. 6.4.2; Transmit modulation quality; No Impact. 6.5A.3.2; Spectrum emission mask; No Impact. 6.5B.3.2. 6.5A.2; Occupied bandwidth for category M1; No Impact. 6.5B.2; Occupied bandwidth for category NB1 and NB2; No Impact. 6.5A.3.3; Additional Spectrum Emission Mask for category M1; TBC. 6.5B.3.3; Additional Spectrum Emission Mask for category NB1 and NB2. 6.5A.3.4; Adjacent Channel Leakage Ratio; No Impact. 6.5B.3.4. 6.5A.4.3; Spurious emissions for UE co-existence; Some impact. 6.5B.4.3. 6.5A.4.4/; Additional spurious emissions; Some impact. 6.5B.4.4. 6.6A; Transmit intermodulation for category M1; Not applicable.. 6.6:; Transmit intermodulation for category NB1 and NB2; No impact. Clause 7:; Receiver characteristics. 7.2:; Diversity characteristics; No impact. 7.3A; Reference sensitivity power level for UE category M1 HD-FDD; Some Impact. 7.3; Reference sensitivity power level for UE category NB1 and NB2; Some Impact. 7.4:; Maximum input level; No impact. 7.5:; Adjacent Channel Selectivity; No impact. 7.6A.2; In-band blocking requirements for category M1; TBC. 7.6B.2; In-band blocking requirements for category NB1 and NB2. 7.6A.3; Out-of-band blocking requirements for category M1; Some impact. 7.6B.3; Out-of-band blocking requirements for category NB1 and NB2. 7.6A.4; Narrow band blocking for category M1; TBC. 7.6B.4; Narrow band blocking for category NB1 and NB2. 7.7:; Spurious response; No impact. 7.8:; Intermodulation characteristics; No impact. 7.9:; RX spurious emission; TBC.

2.2.3	Tx-Rx frequency separation

It is observed that NR NTN bands n256, n255, n254 adopt wide CBW and these NR-NTN bands also support TX-RX frequency separation. Therefore, based on our evaluation, regarding IoT-NTN B252, to support variable Tx-Rx frequency separation is feasible.

Proposal 1: Based on the evaluation and experience of NR-NTN bands supporting the variable Tx-Rx frequency, regarding IoT-NTN B252, to support variable Tx-Rx frequency separation is feasible.

2.3		UE RF TX/RX requirements

2.3.1	UE maximum output power (MOP) and MOP tolerance

Regarding the eMTC/NB-IoT NTN UE RF requirements, the MOP requirements can be associated with RF HD-FDD operation.

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX maximum output power (MOP) simultaneously.

Observation 1: HD-FDD FE IL is lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

Observation 2: When Cat-M1 or Cat-NB1/NB2 UEs operate in HD-FDD mode for B252, it is observed that the HD-FDD TX MOP would be obviously better compared to FD-FDD TX MOP.

From EC Decision 2016/687[4], it is observed that a tolerance of up to + 2 dB is to take account of operation under extreme environmental conditions and production spread. Regarding UE TX output power, the power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be mobile or nomadic.

Observation 3: From EC Decision 2016/687[4], a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

From ETSI EN 301 908[5], it is observed that PC3 is specified as 23dBm +/-2.7dB or 23dBm +3/-4dB.

Observation 4: From ETSI EN 301 908[5], PC3 is specified with allowing higher power tolerance (i.e., 23dBm +/-2.7dB or 23dBm +3/-4dB).

In TS 38.521-1[6], it is observed that there is the additional FR1 UE TX output power test tolerance relative to the TX conductive output power upper bound. The partial contents are shown below for reference.

TS 38.521-1[6] Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR Band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |

TS 38.521-1[6] Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Observation 5: Based on TS 38.521-1 [6] conformance-test specification, the actual UE TX PC3 conductive output power can be higher than PC3 upper bound (e.g., 25dBm).

From WF [3] on Rel-19 FR1 NTN bands, it is observed that NTN antenna gain for smartphone is -5.5dBi for evaluating NTN UE A-MPR.

Proposal 2: Because NTN UE antenna gain for smartphone of -5.5dBi is considered for evaluating NTN A-MPR, the NTN antenna gain of -5.5dBi is also used when evaluating IoT-NTN UE TX MOP regulatory requirements.

Proposal 3: To increase IoT-NTN HD-FDD PC3 nominal power (e.g., change 23dBm to 23.8dBm) or tolerance (e.g., change +2dB to +2.8dB) because HD-FDD TX nominal output power is expected to be higher (e.g., 23.8dBm) and therefore there is less margin to meet the upper bound (i.e., 23 dBm + 2 dB) over PVT.

In our understanding, the regulatory requirements regarding in-band TX output power are also associated with SAR testing.

Observation 6: The UE TX output power upper bound would also be considered when performing SAR testing. Compared with FD-FDD TX SAR, HD-FDD TX SAR issues have been further alleviated.

Proposal 4: If necessary, regarding IoT-NTN HD-FDD MOP or MOP upper bound, consider capability reporting, or add the note to indicate applicable region where feasible when there is regional restriction from regulatory requirements.

2.3.2	IoT-NTN UE RF REFSENS for Cat-M1 and Cat-NB1/NB2

In TS 36.102 [2], IoT NTN Cat-M1 UE reference sensitivity is shown below for reference.

Table 7.3A-1: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -102.7 | FDD |
| 254 | -102.2 | FDD |
| 255 | -102.7 | FDD |
| 256 | -102.2 | FDD |

Table 7.3A-2: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 253 | -103.5 | HD-FDD |
| 254 | -103.1 | HD-FDD |
| 255 | -103.5 | HD-FDD |
| 256 | -103 | HD-FDD |

It has been observed that IoT-NTN B252 can have similar RX performance as IoT-NTN B256.

Proposal 5: Regarding B252 Cat-M1 UE REFSENS, to specify -102.2dBm/CBW and -103dBm/CBW for FD-FDD and HD-FDD modes, respectively.

Table: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.2 | FDD |

Table: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -103 | HD-FDD |

It has been observed that IoT-NTN UE RF REFSENS for Cat-NB1/NB2 is band agnostic.

Proposal 6: Regarding B252 Cat-NB1/NB2 REFSENS, to specify REFSENS of -108.2dBm/CBW because IoT-NTN UE RF REFSENS for Cat-NB1/NB2 is band agnostic.

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

3 Conclusion

In this paper, our views on IoT-NTN B252 UE RF requirements are provided.

Proposal 1: Based on the evaluation and experience of NR-NTN bands supporting the variable Tx-Rx frequency, regarding IoT-NTN B252, to support variable Tx-Rx frequency separation is feasible.

Observation 1: HD-FDD FE IL is lower compared to FD-FDD IL, which can improve HD-FDD TX MOP with 0.8dB compared to FD-FDD TX MOP.

Observation 2: When Cat-M1 or Cat-NB1/NB2 UEs operate in HD-FDD mode for B252, it is observed that the HD-FDD TX MOP would be obviously better compared to FD-FDD TX MOP.

Observation 3: From EC Decision 2016/687[4], a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

Observation 4: From ETSI EN 301 908[5], PC3 is specified with allowing higher power tolerance (i.e., 23dBm +/-2.7dB or 23dBm +3/-4dB).

Observation 5: Based on TS 38.521-1 [6] conformance-test specification, the actual UE TX PC3 conductive output power can be higher than PC3 upper bound (e.g., 25dBm).

Proposal 2: Because NTN UE antenna gain for smartphone of -5.5dBi is considered for evaluating NTN A-MPR, the NTN antenna gain of -5.5dBi is also used when evaluating IoT-NTN UE TX MOP regulatory requirements.

Proposal 3: To increase IoT-NTN HD-FDD PC3 nominal power (e.g., change 23dBm to 23.8dBm) or tolerance (e.g., change +2dB to +2.8dB) because HD-FDD TX nominal output power is expected to be higher (e.g., 23.8dBm) and therefore there is less margin to meet the upper bound (i.e., 23 dBm + 2 dB) over PVT.

Observation 6: The UE TX output power upper bound would also be considered when performing SAR testing. Compared with FD-FDD TX SAR, HD-FDD TX SAR issues have been further alleviated.

Proposal 4: If necessary, regarding IoT-NTN HD-FDD MOP or MOP upper bound, consider capability reporting, or add the note to indicate applicable region where feasible when there is regional restriction from regulatory requirements.

Proposal 5: Regarding B252 Cat-M1 UE REFSENS, to specify -102.2dBm/CBW and -103dBm/CBW for FD-FDD and HD-FDD modes, respectively.

Table: Reference sensitivity for FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -102.2 | FDD |

Table: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| NTN Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 252 | -103 | HD-FDD |

Proposal 6: Regarding B252 Cat-NB1/NB2 REFSENS, to specify REFSENS of -108.2dBm/CBW because IoT-NTN UE RF REFSENS for Cat-NB1/NB2 is band agnostic.

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.2B | - 108.2 |

4 Annex

TS 36.102

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 2532 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |

EC Decision 2016/687

The power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be fixed or installed and as total radiated power (TRP) (2) for terminal stations designed to be mobile or nomadic.

Total radiated power (TRP) is a measure of how much power the antenna actually radiates. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere.

ETSI EN 301 908

4.2.2 	Transmitter Maximum Output Power

4.2.2.1 	Transmitter maximum output power for Single Carrier

4.2.2.1.1 	Definition

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub-frame (1 ms).

4.2.2.1.2 	Limits

The UE maximum output power shall be within the shown value in table 4.2.2.1.2-1.

Table 4.2.2.1.2-1: UE power classes

| E-UTRA Band | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 1 | 23 | ±2,7 |
| 3 | 23 | ±2,7 (see note) |
| 7 | 23 | ±2,7 (see note) |
| 8 | 23 | ±2,7 (see note) |
| 20 | 23 | ±2,7 (see note) |
| 22 | 23 | +3,0/-4,5 |
| 28 | 23 | +2,7/-3,2 |
| 31 | 23 | ±2,7 |
| 33 | 23 | ±2,7 |
| 34 | 23 | ±2,7 |
| 38 | 23 | ±2,7 |
| 40 | 23 | ±2,7 |
| 42 | 23 | +3,0/-4,0 |
| 43 | 23 | +3,0/-4,0 |
| 65 | 23 | ±2,7 |
| 68 | 23 | ±2,7 |
| NOTE: For transmission bandwidths (ETSI TS 136 521-1 [1], clause 5) confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1,5 dB (tolerance = +2,7/-4,2). |  |  |

TS 38.521-1

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n85 |  |  |  |  | 23 | ±(23+TT) |
| n91 |  |  |  |  | 23 | ±(23, 5+TT) |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n106 |  |  |  |  | 23 | ±(2+TT) |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n8 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n75 | 31 | +2+TT/-3-TT |  |  |  |  |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n415 | 31 | +2+TT/-3-TT |  |  |  |  |
| n785 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

5 Reference

- RP-241662, “New WID on Introduction of IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- TS 36.102, “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access”

- R4-2414287, “WF on Rel-19 FR1 NTN bands”, Inmarsat

- EC Decision 2016/687

- ETSI EN 301 908-13 V13.2.0 (2021-11)

- TS 38.512-1, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 18)”