3GPP TSG-RAN WG4 Meeting #112bis	R4-2416017

Hefei, China, October 14-18, 2024

Source:	Nokia

Title:	On RRM Ph5 L3 BSF reduction

Agenda item:	6.15.2.1

Document for:	Discussion

Introduction

RAN4 has discussed L3 measurement delay enhancements through beam sweeping factor reduction over the past few meetings. The latest outcome is captured in RAN4#112 meeting’s way forward [1]. In this contribution we continue the discussion based on this WF.

Impacted RRM requirements

RAN4 agreed to prioritize the requirements for which BSF reduction applies according to the agreement under issue 1-3 [1]:

Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF Which scenarios are considered to use L3 measurement delay reduction by optimizing Rx BSF: - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 6: PSCell addition - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection - Scenario 8: SCell activation - Scenario 9: SCG activation - Scenario 10: CGI identification - Scenario 11: CSI-RS based intra-/inter-frequency measurements, the CSI-RS is configured associatedSSB. The discussion on CSI-RS configured with associatedSSB could be revisited if SSB based L3 measurement delay reduction is concluded. Agreement: - Scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

When looking at the list of prioritized requirements, the impact of faster beam sweeping would be the following:

Scenario 1-4 (measurement delay):

Beam sweeping is taken into account in the M-factors, where for PC3 the factor is defined as:

Mpss/sss_sync_w/o_gaps = 24

Mmeas_period_w/o_gaps = 24

Same values apply for intra- and inter-frequency measurements with and without gaps. The BSF 8 is hence merged in the M-values. When BSF is reduced, either the M value needs to be changed or an additional BSF reduction factor needs to be defined.

Scenario 5 (handover delay):

In handover requirements, beam sweeping is taken into account in the Tsearch component, where for an unknown intra- and inter-frequency target cell in FR3:

Tsearch = N* Trs ms (N=8)

Here the beam sweeping factor reduction can either be implemented similar to Rel-18 multi-Rx beam sweeping factor reduction by defining different values for N, or through an additional factor for a BSF reduction capable UE.

Scenario 7 (RRC re-establishment delay):

For RRC re-establishment, the impact of faster beam sweeping would apply for the definition of Tidentify_intra_NR and Tidentify_inter_NR, i. These are defined in tables 6.2.1.2.1-1 and 6.2.1.2.1-1.

Here RAN4 will need to discuss how to take into account BSF reduction in both values of the max-function. An additional factor for faster beam sweeping capable UE may be the simplest solution to capture the reduction.

If RAN4 wants to aim for a similar way to capture the beam sweeping factor reduction across the specification for all the applicable requirements, then we think an additional factor, such as Nreduced = NRel-19-BSF/8, where NRel-19-BSF is the UE reported N-factor, may be the clearest solution.

RAN4 to cover the impact of Rel-19 BSF reduction through an additional scaling factor across all supported requirements. This factor may depend on UE indicated N if different values for N are supported.

Such approach can also be applied to any other requirements impacted by the BSF reduction, if RAN4 agrees to support more than the agreed requirements for BSF reduction during the Rel-19 work item.

Value for reduced BSF

The proposals related to the value of the reduced beam sweeping factor are as listed under issue 1-4 in the last WF [1]:

Issue 1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Proposal 1 (CATT): - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Proposal 2 (Apple, CTC): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Proposal 3 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Proposal 4 (LGE): - If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4. - RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception - Proposal 5 (Ericsson): - As a result of FBS, RX beam sweep factor can be defined as [4]. - Proposal 6 (Ericsson): - RAN4 to study the requirements relevant to switching between different operations, including: - Transition time period when enabling and disabling FBS in Rel-19. - Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18. - Proposal 7 (Samsung): - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline - Whether to/How to define the other values can be based on SLS results (if necessary)

Logically, if the assumption is that a Rel-19 multi-Rx capable UE would be using 2 Rx to perform measurements, the BSF should be reduced from 8 to 4. Hence, at least N = 4 should be supported. However, Rel-18 multi-Rx BSF reduction supports also values N = 2 and N = 6. RAN4 should discuss if these (or some other) values should be considered also for Rel-19 L3 BSF reduction. If more than one reduced BSF value are supported, then related capability signalling should be introduced.

At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported.

The need for requirements for transition period during the switch between FBS and non-FBS depends on how switching between these states is to happen. We do not think RAN4 needs to discuss this before agreeing about the conditions under which FBS applies.

Conditions for BSF reduction

RAN4 has discussed power saving aspects related to BSF reduction in the past few meetings under issue 1-2-5. Based on the proposals given by the companies, in the way forward 3 directions for this discussion were listed [1]:

[Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) - Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. (3)“up to network indication” (use option 3 for discussion) - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. Discuss option 1/3/8 together, and then add details from other options if needed.

We acknowledge that the additional power consumption associated with FBS is an important factor that must be considered when determining whether and when to enable or disable this feature for a UE. To do so, several options for selectively enabling L3 BSF reduction have been proposed. These options include network-based configuration based on UE measurements and preferences, activation triggered by mobility events indicating cell edge proximity, and UE autonomous enabling of FBS. These options aim to optimize BSF reduction usage for improved performance while minimizing battery drain.

Looking at the options listed in the way forward, the simplified approach in each of them is:

- BSF reduction is triggered depending on (UE) conditions

- BSF reduction applies based on UE preference, such as Rel-18 UAI about single/multi-panel preference

- BSF reduction applies based on network indication

We think the necessity of these options depends on the requirements for which BSF is applied, as given under issue 1-3. Focusing on the prioritized scenarios, we now discuss how to take the power saving aspect into account in scenarios 1-4, 5 and 7.

Handover

Starting from the handover requirements (scenario 5), it is important to connect the UE to the new cell as fast as possible. Handovers in general do not happen all the time, and the UE would only need to use multi-Rx during the handover if the target cell is unknown. Hence, power saving aspects during handover are not so significant, and we think it would be reasonable that a Rel-19 multi-Rx capable UE would always be required to do FBS during handover. This would mean that for such UE, Tsearch is always defined with the reduced N.

For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

RRC re-establishment

The second mobility event to be discussed is the re-establishment process and when to enable FBS for this (scenario 7). Re-establishment takes place after the expiry of timer T310, e.g., due to RLF. Similarly, keeping the UE connected to network at this stage is important and the faster it happens the lower the interruption duration and the throughput loss will be. We believe that this interruption reduction from UE perspective is of importance. If the FBS has been already enabled for the UE during RLF, at the time of re-establishment either UE has more measured frequency layers or it can shorten the re-establishment delay quicker. Thus, for a UE supporting Rel-19 BSF, faster beam sweeping shall always be applied also during RRC re-establishment.

For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Measurement delay requirements

The discussion about measurement requirements (Scenario 1-4) is then a bit more complicated, as measurements not related to handover or other mobility procedures are more frequent. Some companies are worried that using BSF reduction for the measurements all the time may therefore lead to UE power consumption issues. On the other hand, the UE already has the freedom to decide on implementing faster beam sweeping, although the minimum requirements are written with the N=8 assumption. In the following we analyse the three options for defining when FBS should be applied for L3 measurement delay requirements.

(1) Event triggered BSF reduction: Event triggered BSF reduction does not necessarily require any additional signalling between the UE and the network. However, depending on what kind of conditions/events are used as BSF triggers, it may not be visible to the network when such event condition is met. Hence, if this option is used, the events that trigger BSF reduction would need to be selected so that the network has visibility to the UE conditions. As we have suggested in our previous contributions, the existing mobility events for L3 event triggered reporting can be used to determine when the UE is required to perform faster measurements and therefore should be using reduced BSF. The network will know from the received report that a mobility event was met at the UE side, and hence knows that the UE is using faster beam sweeping. Depending on which events trigger BSF reduction, additional network signalling may be needed.

In another option, RLF related counters could also be used to trigger FBS. For example the number of Qout could be used as an indication that the UE is at cell edge. Thus, monitoring the radio links close to a mobility event in a faster manner, reduces the probability of a link failure by having more measurement results at hand. However, the Qout events are only known to the UE, and this approach would not help the network to take decisions based on whether FBS is activated or not.

Depending on how events/conditions that trigger BSF reduction are defined, network may or may not have visibility to whether UE is using BSF reduction or not.

Using the existing L3 event triggered reporting conditions and conditioning BSF reporting to event triggered reporting would allow network visibility to UE beam sweeping behaviour based on the event triggered report reception. Additional network signalling may or may not be required.

(2) UE preference based BSF reduction: In this option the approach is to use UE preference indication as the way to enable/disable BSF reduction. The UE determines when to enable/disable FBS by reporting/requesting to network its preference, for example through UAI as in Rel-18. This way the UE can inform the network on deactivating FBS and benefit from power savings when necessary. Otherwise, FBS would be expected to be on. However, as discussed in Rel-18, by allowing the UE to independently deactivate FBS, it could lead to a situation where the network is unable to fully leverage the advantages of FBS, particularly in handover scenarios. In Rel-18 it was agreed that the network can de-configure group-based beam reporting based on the UE single-panel preference, but such deconfiguration for Rel-19 BSF is currently not possible. Hence, the network has no means to react to the UAI indication in Rel-19, unless additional RAN2 signalling is introduced.

While UE preference indication enables the UE to get the benefits of power saving through not using BSF reduction, network has no means to react to such preference indication without additional RAN2 signalling.

(3) Network indication based BSF reduction: In this option the network configures FBS enabling based on the reported UE measurements and UE multi-Rx preference indication. This way the network can better determine the UE's scheduling which could result in better handling of interruptions. Additionally, the UE can proactively perform faster measurements based on network indication. This option enables the UE for faster reaction towards a probable change in the channel condition and performing measurements without further delay due to exchanging FBS enabling with the network. However, the network may not always have sufficient information to define whether BSF reduction should be configured or not.

Network indication to enable/disable BSF reduction is a straightforward solution, but requires new signalling.

Taking observations 1-4 into account, each solution has their pros and cons. UE power saving aspects can be considered to be achieved either through UE as default not being required to use FBS all the time, or through UE/network signalling to turn FBS on and off.

As the UEs can already do faster beam sweeping based on UE implementation under the existing minimum requirements, we think to get more benefits from the Rel-19 capability, RAN4 should define scenarios under which FSB always applies. This would mean that a version of option (1) is to be used. As we have proposed in our previous papers, we think BSF reduction should be enabled at least when:

- L3 based event has been triggered (potentially with network configuration option)

- Conditional handover is configured

Using BSF reduction in these scenarios ensures that the UE can react more quickly to changes in channel conditions close to handover and during handover scenarios, thus maintaining optimal performance.

For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies when L3 mobility event is met. FFS whether additional signalling is needed.

For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies when conditional handover is configured.

Capability

In relation ot the UE capability, the following issue was listed for discussion in the last WF [1]:

Issue 1-5: feature capability of L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Option 1(Apple, vivo, Nokia, Samsung): - RAN4 to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19. - Option 1a(Apple): The detailed definition can be postponed to the end of the core part discussion. - Option 2(Eircsson): - To support FBS, RAN4 to check if those capabilities for multi-Rx in Rel-18, e.g., faster RX beam sweeping, enhanced scheduling and measurement restrictions and multi-Rx preference indication, can be used directly. If the NW indicates FBS and multi-RX in Rel-18 is not enabled parallelly, we can observe the possibility of reusing at least part of such capabilities.

As we have proposed in our previous contributions, we think Rel-19 BSF reduction should be an independent capability that does not need to depend on UE support of Rel-18 multi-Rx capabilities.

RAN4 to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

SSB processing delay

The following issue is open for discussion regarding SSB processing delay [1]:

Issue 1-2-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC [Way forward]: FFS: - Option 1 (CATT, vivo): No need to add additional processing time due to multiple SSBs within one SMTC. - Option 2 (Apple): allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing - Option 3 (Ericsson): Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement - Option 4 (Intel): - Consider UE baseband processing capabilities when specifying the L3 delay reduction for simultaneous receptions on multiple FR2 SSB-s. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Option 5 (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms

It has been brought up by some companies that a UE receiving multiple beams in a SMTC window will, due to UE hardware design, need additional time for processing the received SSBs. The SSB processing time is assumed to be 2 ms as in multiple legacy requirements. If UE receives 2 different beams in a SMTC, then it can process one SSB during/after the SMTC window (while receiving), and another SSB can be post-processed in the subsequent time period. Hence, when looking at the total cell identification delay, the additional SSB processing time would only need to be added once to the total delay to cover the processing of the last measurement.

If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as

  - Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms

  - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms

  - where TSSB_processing = 2 ms

DRX

DRX was included for discussion in the last meeting WF with the following issue [1]:

Issue 1-2-9: FFS: DRX is configured or not [Way forward]: FFS: - Option 1 (Xiaomi): SSB based L3 measurement delay reduction with DRX shall be deprioritized - Option 2 (Ericsson): FBS may cover DRX cases, no need to deprioritize DRX case.

We do not see a need to distinguish between DRX and non-DRX for BSF reduction purposes. The reduction may equally well apply for both DRX and non-DRX.

Rel-19 BSF reduction is enabled independent of DRX status.

Conclusion

In this paper the following observations and proposals were made:

Proposal 1: RAN4 to cover the impact of Rel-19 BSF reduction through an additional scaling factor across all supported requirements. This factor may depend on UE indicated N if different values for N are supported.

Proposal 2: At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported.

Proposal 3: For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during handover (Tsearch).

Proposal 4: For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies during RRC re-establishment.

Observation 1: Depending on how events/conditions that trigger BSF reduction are defined, network may or may not have visibility to whether UE is using BSF reduction or not.

Observation 2: Using the existing L3 event triggered reporting conditions and conditioning BSF reporting to event triggered reporting would allow network visibility to UE beam sweeping behaviour based on the event triggered report reception. Additional network signalling may or may not be required.

Observation 3: While UE preference indication enables the UE to get the benefits of power saving through not using BSF reduction, network has no means to react to such preference indication without additional RAN2 signalling.

Observation 4: Network indication to enable/disable BSF reduction is a straightforward solution, but requires new signalling.

Proposal 5: For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies when L3 mobility event is met. FFS whether additional signalling is needed.

Proposal 6: For UE supporting Rel-19 BSF reduction capability, faster beam sweeping always applies when conditional handover is configured.

Proposal 7: RAN4 to introduce a new individual optional capability for L3 BSF reduction due to multi-Rx operation in R19.

Proposal 8: If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as

a.	Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms

b.	Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms

c.	where TSSB_processing = 2 ms

Proposal 9: Rel-19 BSF reduction is enabled independent of DRX status.

- References

[1] R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, 3GPP RAN4#112 meeting, Maastricht, Netherlands, 08/2024.