3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416019

# Hefei, Anhui, China, 14th – 18rd October, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES

Agenda item:	6.22.2.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of on-demand SSB Scell operation for Rel-19 NES.

Discussion

On-demand SSB in SCell

RRC based signaling

In the last meeting, RAN4 agreed to focus on at least for MAC-CE based single SCell activation case as staring point. Meanwhile, in RAN1#118, RAN1 confirms supporting both RRC based signaling and MAC-CE signaling as follows:

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Generally, for RRC based signlaing to indicate on-demand SSB transmission, the NW can provide SCell configuration and on-demand SSB configuration before SCell activation command. When the UE receives SCell activation command, UE can measure OD-SSB after decoding MAC-CE.

In our understanding, the difference between RRC based signlaing and MAC-CE based signaling is applicapable scenario and processing delay for receiving on-demand SSB indication. For applicapable scenario, we think RRC based signaling can only be applied for Scenario #2A. For the processing delay, we think additional RRC processing delay can be expected. Since it was agreed to introduce RRC based signlaing, we think RAN4 needs to specify RRC based signlaing procedure requirement in addition to MAC-CE signaling procedure requirement.

Proposal 1: Specify RRC based signlaing procedure in addition to MAC-CE signaling procedure

SCell RRM measurement in Case1

In Case1, always-on SSB (AO-SSB) is not expected in SCell. A UE can be indicated multiple OD-SSB indication/deactivation as illustrated in Figure 1. In Figure 1, the UE receives OD-SSB indication1 after activation time, which is deactivated after 4 SSB bursts. After some time after OD-SSB deactivation, the UE receives another OD-SSB indication2. In the legacy SSB measurement, the time between each SSB bursts are always same (e.g. period). However, in OD-SSB operation, it is not guaranteed the time between multiple OD-SSB indication/deactivation is always same (e.g. time between OD-SSB deactivation and OD-SSB indication2), which may lead additional efforts to UE. For example, if the time is long enough to lose sync, UE may need to pay additional effort to detect PSS/SSS or PBCH detection. Thus, performing the measurement is not clear when multiple OD-SSB indication/deactivation is received.

Figure 1 OD-SSB transmission in Case1

Proposal 2: RAN4 to specify RRM requirements when multiple OD-SSB indication/deactivation is received in Case1

SCell RRM measurement in Case2

In Case2, there are AO-SSB from SCell, and OD-SSB can be indicated to perform dense measurement. Regarding OD-SSB following agreement was achieved in the last RAN1 meeting.

Agreement For a cell supporting on-demand SSB SCell operation, at least the following is supported. - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB. FFS: Additional support of OD-SSB for CD-SSB located on sync-raster.

According to agreement, OD-SSB is not allowed to locate on synchronization raster. On the other hand, it seems that AO-SSB is located on synchronization raster. It means that RF retuning may be required to measure AO-SSB and OD-SSB, and additional requirements may be needed. Even if AO-SSB and OD-SSB are configured in the same center frequency, when they are overlapped or time gap between AO-SSB and OD-SSB is small, how to measure them is not clear. Since RAN4 has not discussed the relationship between AO-SSB and OD-SSB, it needs to be specified.

Proposal 3: RAN4 to specify RRM requirements in terms of relationship between AO-SSB and OD-SSB in Case2

Unknown SCell activation

In previous meeting, RAN4 discussed whether to specify known and unkwon requirements for Case#1/ScenarioA and Case#1/Senario2A. In the end, wayforward for Case#1/Scenario#2A is captured [2] as follows:

Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

As speficied in 38.133 - 8.3.2, Scell in FR1 is considered as ‘known’ when the UE has sent a valid measurement report for the SCell being activated, and the SSB measured remains detectable during the period max(5*measCycleSCell, 5*DRX cycles) before the repection of the SCell activation command. For SCell in FR2 is considered as ‘known’ when the UE has sent a valid L3-RSRP measurement report with SSB index, and SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation during the period equal to 4s or 3s before UE receives the last activation command, which depends on the UE power class. Otherwise, the SCell is considered to be unknown.

If we apply the legacy known and unknown condition, sending ‘valid measurement report or L3-RSRP measurement report with SSB index depends on the OD-SSB indication timeline. If the UE is not provided enough time to make valid measurement report after OD-SSB indication, SCell can be remained unkown. Thus, known and unknown requirements needs to be specificed in Case#1/Scenario2. Furthermore, applying legacy known and unknown condition may waste OD-SSB, if known condition is not satisfied. Unlike Case#1/Scenario2, there are no SSB to be measured before SCell activation command in Case#1/Sencario2A. However, we are not sure that considering only single OD-SSB indication is reasonable in the scenario. In general, there can be multiple OD-SSB indication/deactivation before or after SCell activation command. Therefore, it is premature to close the door for known requirement at this stage.

Proposal 4: RAN4 to specify both known and unknwown requirements for Case#1/Scenario2/2A

LS replay

In last RAN1 meeting, RAN1 agreed to send LS[3] on timeline for on-demand SSB operation on SCell. In LS, RAN1 asks RAN4 to confirm the timeline between on-demand SCell indication via MAC-CE and on-demand SSB burst(s) is not less than existing timeline required for UE’s MAC-CE processing for SCell activation such as T_min=. In our understanding it is reasonable that the UE receives on-demand SSB burst(s) after transmitting UL transmission with HARQ-ACK and MAC-CE processing timeline. Since MAC-CE processing timeline is already applied in current requirements, reusing the same processing timeline seems reasonable.

Proposal 5: RAN4 to reply LS confirming working assumption of T_min

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: Specify RRC based signlaing procedure in addition to MAC-CE signaling procedure

Proposal 2: RAN4 to specify RRM requirements when multiple OD-SSB indication/deactivation is received in Case1

Proposal 3: RAN4 to specify RRM requirements in terms of relationship between AO-SSB and OD-SSB in Case2

Proposal 4: RAN4 to specify both known and unknwown requirements for Case#1/Scenario2/2A

Proposal 5: RAN4 to reply LS confirming working assumption of T_min

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2413885, “WF on Netw_Energy_NR_enh,” Ericsson

[3]R1-2407565,” LS on timeline for On-demand SSB operation on SCell,” Apple