# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416021

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.23.4.1

Source: 	Ericsson

Title:	Simulation on LP-WUS RRM measurement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement for simulation assumption[1]. In this contribution, we continue to share our views on simulation assumption on LP-SS RRM measurement.

Issue 2-1-1: SINR setting Agreement: - As starting point for RAN4 RRM simulation purpose only, -3dB, 0.5dB, 2dB dB Ês/Iot value is used for serving cell for both OOK based and OFDM based LP-WUR. - The SINR value can be updated if any update on the noise figure value in RF session. - FFS the side condition for RAN4 requirement. Issue 2-1-1-1: SNR setting for serving and interference cell derivation from SINR setting Agreement: - Consider two cells for RAN4 simulation: one serving cell and one interference cell. Agreement: - To derive SNR for serving cell and interference cell from serving cell Ês/Iot, a relationship for the SNR or transmission power between serving cell and interference cell need be pre-defined. - Consider the SNR/transmission power of the interference cell is 9 dB or 6 dB lower compared with that of the serving cell. Other value can be considered. Issue 2-1-3: Time/frequency error Agreement: Residual frequency error: OFDM based receiver [5] ppm OOK based receiver [10 20] ppm Issue 2-1-5: Measurement interval Agreement: Use 320ms for SSB based LP-WUR firstly.

- Simulation assumption

In RAN1 #118 meeting, the new agreement about frequency error is as follow. RAN4 needs to update the simulation assumption for frequency error as 5-10ppm for OOK-based LP-WUR.

RAN1 #118 Agreement For OOK-based LP-WUR, the LP-WUS design assumes the following: - As starting point, the time error is based on the 5-10ppm residual frequency error after frequency error correction without considering impact of drift. - The frequency error: up to company report. - Note: 5-10ppm assumes frequency error correction is performed, e.g., RTC calibration and/or MR assistance - Note: Companies can assume other values outside of 5-10ppm as long as the values are justified

Proposal 1: RAN4 to update the Frequency Offset relative to UE frequency reference as 5, 10ppm.

In last meeting, some simulation parameters are still uncertain in RAN4 simulation. To simplify the simulation workload, we proposed to choose a subset of agreed RAN1 simulation. The detail parameters are defined as follow,

Proposal 2: RAN4 to agree data and control channel subcarrier spacing as follow.

- OFDM based: The same as SSB subcarrier spacing

- OOK based: the same as one of the SCS(s) used for other NR transmission

Proposal 3: RAN4 to agree the 1 Tx and 1 Rx as the simulation assumption.

Proposal 4: RAN4 to agree only using SCS = 30KHz in the simulation.

Proposal 5: RAN4 to agree the LP-SS binary sequence pattern for M=4 as follow.

Cell 1: [1010] Cell 2: [0101]

- Summary

In this contribution, we have initially discussed the LP-WUS RRM measurement simulation assumption. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to update the Frequency Offset relative to UE frequency reference as 5, 10ppm.

Proposal 2: RAN4 to agree data and control channel subcarrier spacing as follow.

- OFDM based: The same as SSB subcarrier spacing

- OOK based: the same as one of the SCS(s) used for other NR transmission

Proposal 3: RAN4 to agree the 1 Tx and 1 Rx as the simulation assumption.

Proposal 4: RAN4 to agree only using SCS = 30KHz in the simulation.

Proposal 5: RAN4 to agree the LP-SS binary sequence pattern for M=4 as follow.

Cell 1: [1010] Cell 2: [0101]

- Reference

[1]. R4-2413899, “WF for RRM requirements for LP-WUS/WUR”, vivo