# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416023

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.1

Source: 	Ericsson

Title:	Discussion on OD-SSB in Rel-19 NES

Document for:	Discussion

# Introduction

In RAN Penlary #102, enhancements of network energy savings for NR[1] are introduced. In last RP meeting, the WID (mainly in Obj. 2) is updated with the following objectives.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we continue to discuss on the topic on OD-SSB operation of Rel-19 NES WI from RAN4 perspective.

- On-demand SSB(OD-SSB)

  - General

In last meeting, RAN4 made the agreements to define the prioritized scenarios for OD-SSB based SCell activation.

Issue 1-1-1: OD-SSB work plan - Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed. Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement.

To convenient the discussion, the definition of each RAN1 terminology is copied below.

OD-SSB case and scenarios definition - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #2A refers to “When UE receives SCell activation command (e.g., as defined in TS 38.321)”

Based on the above RAN1 agreement, we noticed that RAN4 needs to differentiate Case #1 and Case #2 which is agreed in RAN1. However, whether the OD-SSB indication is transmitted before or together with SCell activation command (scenario #2 and #2A) has not too much impact on RAN4 SCell activation requirement. In our understanding, the main difference to be captured in RAN4 spec. is about known and unknown requirement.

Observation 1: The Scenario #2 and #2a scenarios are mainly introduced and used in RAN1.

From a SCell addition configuration to the SCell activation, a typical OD-SSB based SCell activation procedure includes five possible phases (some phases may be repeated) which is shown as follow.

Figure 1. OD-SSB based known SCell activation(Case 1)

NW can stop transmitting the OD-SSB and achieve the power saving gain during the phase 1. During phase 2, NW indicates the OD-SSB activation for deactivated SCell measurement (Scenario #2). After NW receiving the L3 MR, NW can decide whether to stop the OD-SSB transmission in phase 3. When NW indicates both OD-SSB transmission and SCell activation (both Scenario #2 and #2A are possible), UE performs SCell activation procedure in phase 4. After UE finishing the SCell activation and transmitting the valid CQI report, UE will stay in an active serving cell as phase 5.

Observation 2: For both known and unknown Scell activation, OD-SSB indication can be before/together with Scell activation command.

Proposal 1: RAN4 shall focus on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2a scenarios.

  - OD-SSB based deactivated SCell measurement

General

In last meeting, there is an open issue about whether UE needs to always perform cell identification to obtain coarse timing before deactivated Scell activation. The motivation to always require additional cell indetification is due to OD-SSB is not always transmitted.

Issue 2-1-6: OD-SSB based deactivated SCell measurement components - Proposals - Option 1: vivo - RAN4 to clarify the deactivated SCell measurement components includes both cell identification and measurement.

However, we think at least for the following cases UE doesn’t need to always perform cell identification. Firstly, for always-on SSB transmitted in case 2 means UE has performed the cell identification based on always-on SSB and doesn’t need to perform cell identification when OD-SSB is indicated. Secondly, in Case 1, if the OD-SSB SCell being measured is contiguous to an active serving cell in the same band, UE can refer on the active serving cell’s timing as legacy. No cell identification is needed. Thirdly, if UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is less than 5s and then the OD-SSB is transmitted again, cell identification is also unnecessary.

Proposal 2: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [5s] and then the OD-SSB is transmitted again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Another open issue is whether to consider neighbour cell measurement with OD-SSB. In the WID, it clearly specifies the procedures and signaling method(s) to support on-demand SSB SCell operation for UEs. In our understanding, the main purpose of such OD-SSB is for SCell’s power saving. It’s unnecessary to indicate UE whether OD-SSB is transmitted or not from neighbour cells. Furthermore, UE also doesn’t need to understand whether there is OD-SSB based neighbour cells. Thus, we also propose to focus on OD-SSB based serving cell measurement.

Issue 2-1-4: OD-SSB based L3 neighbour cell measurement - Proposals - Option 1: Huawei - RAN4 to deprioritize OD-SSB based L3 neighbour cell measurement, as feasibility and reliability are questionable and gain is not clear. - Option 2: Samsung - RAN4 needs to clarify if L3 measurements based on on-demand SSB is needed or not.

Proposal 3: RAN4 to deprioritize OD-SSB based L3 neighbour cell measurement.

Case 1: No always-on SSB on the cell

In last meeting, RAN4 made the following agreements about OD-SSB based deactivated SCell measurement in Case 1.

Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion.

On the one hand, to maximize the NES gain, NW is expected to extend the muting duration before SCell activation as much as possible. On the other hand, to guarantee the SCell activation as a known cell, the OD-SSB transmission period should be longer than the required deactivated SCell measurement period. It means the OD-SSB transmission period will be much longer once UE performs the measurement in a very slow manner as legacy.

Observation 3: To guarantee the known condition, OD-SSB transmission time duration should be longer than deactivated Scell measurement period.

In last meeting, some companies mentioned UE follows the measCycleSCell for UE power saving in legacy SCell activation. As shown in the agreement above, UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered which has already relieved the UE’s power burden for deactivated SCell measurement. Considering the WI aims to achieve the network’s power saving, we propose that UE is requested to perform deactivated SCell measurement based on indicated OD-SSB periodicity when NW indicates the OD-SSB transmission for deactivated SCell measurement.

Observation 4: To maximize the NES gain, OD-SSB based deactivated SCell measurement shall be limited to a shorter duration.

Proposal 4: In Case 1, UE is assumed to perform deactivated SCell measurement based on OD-SSB when NW indicates the OD-SSB transmission during the deactivated SCell stage.

Case 2: With always-on SSB on the cell

RAN1 has made the following agreements about the type of OD-SSB and always-on SSB. Based on the following agreements, it can be observed that OD-SSB and always-on SSB may be in the same or different frequencies.

(RAN1 #116b agreement) - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB (RAN1 #118 agreement) - For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB - FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

Thus, the following OD-SSB and always-on SSB time-frequency domain relations are possible.

- Scenario 1: same frequency, same time offset, different periodicities

In our understanding, the typical scenario in Case 2 is NW configures both always-on SSB and OD-SSB within the same frequency. NW can use a sparse periodicity to transmit the always-on SSB and sometimes transmit the OD-SSB as a complement, such as to speed up the SCell activation process.

Figure 2. OD-SSB and always-on SSB(same frequency, same time offset, different periodicities)

- Scenario 2: same frequency, different time offset, different periodicities

The difference between scenario 2 and scenario 1 is only the time offset. In our understanding, it’s unnecessary to configure different SSB offset between OD-SSB and always-on SSB. In legacy release, when we introduced the NCD-SSB, it can have different offset with CD-SSB. The reason is the power limitation of SSB transmission. However, we don’t think there is a power limitation issue here since OD-SSB and always-on SSB are in the same frequency. They can assumed to be the same SSB but with different periodicities. But we’re open whether this is a valid case.

Figure 3. OD-SSB and always-on SSB(same frequency, different time offset, different periodicities)

- Scenario 3: different frequencies

The third scenario is typical as legacy CD-SSB and NCD-SSB. Always-on SSB can be a CD-SSB in the sync-raster for the coverage, and the OD-SSB is an NCD-SSB in another frequency for some specific UEs. However, we have a little bit concern on this scenario. To achieve the NES gain, the periodicity of the always-on SSB should be a relatively large periodicity. It will be a contradiction to configure always-on SSB periodicity as CD-SSB with 20ms periodicity. In other words, it’s unnecessary or no significant gain to introduce another OD-SSB. (20ms is already so fast and no power saving gain from network side). But we’re open on this scenario. For example, RAN1 may introduce some mechanisms to allow a sparse CD-SSB periodicity in NES.

Figure 4. OD-SSB and always-on SSB(different frequencies)

Based on the analysis above, we believe the scenario 1 should be the typical case. RAN4 shall prioritize to define the requirement for OD-SSB and always-on SSB within the same frequency, same time offset but with different periodicities.

Proposal 5: In Case 2, RAN4 to prioritize the scenario in which OD-SSB and always-on SSB are within the same frequency, same time offset but with different periodicities.

In Case 2, NW transmits the legacy always-on SSB together with the OD-SSB as follows: (Green SSB is legacy always-on SSB, Orange SSB is the OD-SSB). Before NW indicating the OD-SSB transmission, UE is assumed to use legacy always-on SSB to perform deactivated SCell measurement. UE can combine both OD-SSB and always-on SSB to perform deactivated SCell measurement once UE receiving the OD-SSB transmission indication.

Figure 5. OD-SSB based deactivated SCell measurement and reporting(Case 2)

Observation 5: UE can combine the measurement results if the difference between always-on SSB and OD-SSB is only the periodicity.

Proposal 6: In Case 2, the UE is assumed to perform deactivated SCell measurement based on legacy SSB before UE receiving OD-SSB transmission indication and after the OD-SSB transmission deactivation.

Proposal 7: In Case 2, the UE is assumed to perform measurement by combining the legacy SSB and OD-SSB when OD-SSB transmission is indicated.

  - OD-SSB based SCell activation

Based on current known condition in SCell activation, the UE needs to send a valid L3 measurement report (MR) for the SCell being activated. Thus, during the time period where the Scell is deactivated, UE needs to perform deactivated SCell measurement. After NW receiving the L3 MR, the SCell can be believed as a known SCell. When NW indicates the SCell activation command, UE is assumed to perform SCell activation based on known SCell requirement. In other words, the SCell is an unknown SCell if no L3 MR is transmitted to NW.

In last meeting, there was a discussion about whether known/unknown scenario for Scenario 2A needs to be revisted. As agreed in RAN1, OD-SSB transmission can be before (Scenario 2) or together (Scenario 2A) with SCell activation command. We understand that both known and unknown cases are possible based on the combination of scenario #2 and #2A. From RAN4’s perspective, the most important thing is whether the known condition is met. If the known condition is met, UE will follow the known SCell activation requirements. Otherwise, UE will follow unknown SCell activation requiremenst. Thus, we propose RAN4 to focus on OD-SSB based Scell activation procedure (known and unknown case) and not to specifically differentiate Scenario 2/2a scenarios.

Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

Observation 6: For unknown Scell activation, OD-SSB indication can be before/together Scell activation command.

Proposal 8: RAN4 to focus on OD-SSB based Scell activation procedure (known and unknown case) and not to specifically differentiate Scenario 2/2a scenarios.

Furthermore, we think the principles of legacy known and unknown SCell activation requirement can be reused except considering the following possible impact.

Proposal 9: When OD-SSB transmission is before/together with the SCell activation command, UE follows the principles of legacy known/unknown SCell activation requirement once the known condition is met.

Aspect 1: which SSB will be used in SCell activation?

From our understanding, UE is required to use OD-SSB to perform SCell activation in both Case 1 and Case 2. Especially, in Case 2, UE can receive both always-on SSB and OD-SSB, but UE is still required to switch to OD-SSB for SCell activation after receiving the SCell activation command.

Proposal 10: After receiving the SCell activation and OD-SSB indication command, UE is required to always use OD-SSB for Scell activation.

Aspect 2: OD-SSB uncertainty

Currently, in legacy requirement, TFirstSSB, TFirstSSB_MAX are introduced to define the uncertainty to receive the first complete SSB burst after HARQ processing and MAC-CE parsing time. We think the similar uncertainty shall also be introduced in OD-SSB based SCell activation. UE needs to wait for the first complete OD-SSB burst after HARQ processing and MAC-CE parsing time.

Proposal 11: RAN4 to introduce the OD-SSB transmission uncertainty duration for UE receiving first complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX.

Aspect 3: whether OD-SSB based L1-RSRP measurement is needed in Case 2?

In legacy SCell activation procedure, if UE performs L3 measuremernt and reporting the quality within a valid time(such as 5s), the SCell is believed as a known cell. UE can perform SCell activation without L1-RSRP measurement. However, when UE performs the deactivated SCell measurement by always-on SSB or OD-SSB and NW activates the SCell by another OD-SSB. UE may not know the TCI information of such OD-SSB if OD-SSBs and/or always-on SSB use different spatial filters. The issue is still discussing in RAN1. RAN4 can wait RAN1’s further progress to clarify the spatial relation within OD-SSBs and/or with always-on SSB.

Proposal 12: RAN4 to wait RAN1’s further progress on spatial relation between OD-SSBs and always-on SSB.

Aspect 4: SSB-less SCell scenario

RAN4 also introduced the SSB-less SCell activation requirement for intra-band and inter-band collocated scenario . UE follows the SCell activation command to activate the SCell without SSB. In our understanding, UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation phase. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 13: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Aspect 5: known cell condition

Currently, the known cell condition is as follow.

FR1 SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. FR2 For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

First of all, it should consider different time period to define the known condition in FR1 once RAN4 agrees to use OD-SSB periodicity for deactivated SCell measurement. Obviously, it should be related to OD-SSB measurement other than measCycleSCell and DRX. We prefer to use a solid value, such as 5s similar other FR1 known cell condition. Secondly, RAN4 shall also wait RAN1’s progress on TCI and spatial relation and further check whether known cell condition needs to be updated.

Proposal 14: RAN4 to discuss the known cell condition related to the OD-SSB based SCell activation.

- Summary

In this contribution, we have discussed the MUSIM gaps requirements. Based on the discussions, we have made following proposals and observations:

Observation 1: The Scenario #2 and #2a scenarios are mainly introduced and used in RAN1.

Observation 2: For both known and unknown Scell activation, OD-SSB indication can be before/together with Scell activation command.

Observation 3: To guarantee the known condition, OD-SSB transmission time duration should be longer than deactivated Scell measurement period.

Observation 4: To maximize the NES gain, OD-SSB based deactivated SCell measurement shall be limited to a shorter duration.

Observation 5: UE can combine the measurement results if the difference between always-on SSB and OD-SSB is only the periodicity.

Proposal 1: RAN4 shall focus on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2a scenarios.

Proposal 2: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [5s] and then the OD-SSB is transmitted again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Proposal 4: In Case 1, UE is assumed to perform deactivated SCell measurement based on OD-SSB when NW indicates the OD-SSB transmission during the deactivated SCell stage.

Proposal 5: In Case 2, RAN4 to prioritize the scenario in which OD-SSB and always-on SSB are within the same frequency, same time offset but with different periodicities.

Proposal 6: In Case 2, the UE is assumed to perform deactivated SCell measurement based on legacy SSB before UE receiving OD-SSB transmission indication and after the OD-SSB transmission deactivation.

Proposal 7: In Case 2, the UE is assumed to perform measurement by combining the legacy SSB and OD-SSB when OD-SSB transmission is indicated.

Proposal 8: RAN4 to focus on OD-SSB based Scell activation procedure (known and unknown case) and not to specifically differentiate Scenario 2/2a scenarios.

Proposal 9: When OD-SSB transmission is before/together with the SCell activation command, UE follows the principles of legacy known/unknown SCell activation requirement once the known condition is met.

Proposal 10: After receiving the SCell activation and OD-SSB indication command, UE is required to always use OD-SSB for Scell activation.

Proposal 11: RAN4 to introduce the OD-SSB transmission uncertainty duration for UE receiving first complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX.

Proposal 12: RAN4 to wait RAN1’s further progress on spatial relation between OD-SSBs and always-on SSB.

Proposal 13: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 14: RAN4 to discuss the known cell condition related to the OD-SSB based SCell activation.

- References

- RP-234065, “Rel-19 WID: Enhancements of network energy savings for NR”, Ericsson, Apple

- RP-242354, “Revised WID - Enhancements of network energy savings for NR”, Ericsson

- 3GPP TR 38.864 V18.1.0, “Study on network energy savings for NR”

- Backup

The summary of SCell activation requirement impact due to the OD-SSB.

FR1

| # Case | # To-be-activated target SCell | # Rules | # Legacy Requirement | # OD-SSB impact |
| --- | --- | --- | --- | --- |
| # #1 | # FR1 known Scell with meas. period <= 2400ms | # Known, measurement period | # TFirstSSB | # Aspect 2 |
| # #2 | # FR1 known Scell with meas. period > 2400ms |  | # TFirstSSB_MAX+ Trs |  |
| # #3 | # FR1 unknown Scell with active serving cell(s) on the same band | # Unknown, with ssb-PositionInBurst, TCI indication | # TFirstSSB_MAX + TSMTC_MAX+ Trs | # Aspect 2 |
| # #4 | # FR1 unknown Scell without active serving cell(s) on the same band |  | # TFirstSSB_MAX + TSMTC_MAX+ 2Trs |  |
| # #5 | # FR1 unknown Scell without TCI indication in MAC-CE, and with SP CSI-RS for CSI reporting | # Purely Unknown | # TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2, 3 |
| # #6 | # FR1 unknown Scell without TCI indication in MAC-CE, and with P CSI-RS for CSI reporting |  | # 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay) |  |
| # #7 | # FR1 with active serving cell(s) on the same band | # SSB-less | # 3ms | # Aspect 4 |

FR2

| # Case | # To-be-activated target SCell | # Rules | # Legacy Requirement | # OD-SSB impact |
| --- | --- | --- | --- | --- |
| # #8 | # FR2 with active serving cell(s) on the same band with configured SMTC | # Active serving cell in the same band(SSB-less) | # TFirstSSB+ 5ms | # Aspect 2 |
| # #9 | # FR2 with active serving cell(s) on the same band but without configured SMTC, and the RS(s) of Scell being activated is QCL-TypeD with RS of one active serving cell on that FR2 band |  | # 3ms | # Aspect 4 |
| # #10 | # FR2 known Scell without active serving cell on the same band, and with SP-CSI-RS for CSI reporting | # First active FR2 Scell # P1: Known and unknown # P2: SP-CSI-RS, P-CSI-RS | # 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2 |
| # #11 | # FR2 known Scell without active serving cell on the same band, and with P-CSI-RS for CSI reporting |  | # max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) |  |
| # #12 | # FR2 unknown Scell without active serving cell on the same band, and with SP-CSI-RS for CSI reporting |  | # 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2, 3 |
| # #13 | # FR2 unknown Scell without active serving cell on the same band, and with P-CSI-RS for CSI reporting |  | # 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |  |