# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416024

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.22.2.2

Source: 	Ericsson

Title:	Discussion on RS and channel adaptation in Rel-19 NES

Document for:	Discussion

# Introduction

In RP #102, enhancements of network energy savings for NR[1] are introduced. In last RP meeting, the WID (mainly in Obj. 2) is updated with the following objectives[2].

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

In this contribution, we continue to discuss on the Rel-19 NES operation from RAN4 perspective.

# Adaptation of common signal/channel transmissions (CS/CC adaptation)

In last meeting, RAN4 made the following agreements[3]. RAN4 will deprioritize the paging and PRACH adaptation and only focus on SSB adaptaion. In SSB adaptation, RAN4 will prioritize the SSB adaptation in CONNECTED mode.

### Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress. ### Sub-topic 3-2: Impact on Paging Adaptation requirements Issue 3-2-1: Paging adaptation Agreement: - Deprioritize Paging adaptation until the RAN1 concrete conclusion is available. ### Sub-topic 3-3: Impact on PRACH Adaptation requirements Issue 3-3-1: PRACH adaptation Agreement: - Deprioritize PRACH adaptation until the RAN1 concrete conclusion is available.

SSB adaptation

In RAN1 #116 meeting, RAN1 agreed to further study the SSB adaptation in time domain to both PCell and SCell. In last RAN1 meeting, RAN1 agreed that the SSB adaptation is at least the SSB not in sync raster in SCell, but whether the SSB adaptation can be applied to PCell is still FFS. Thus, at least RAN4 will define the SSB adaptation on SCell for NCD-SSB.

RAN1 #116 Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s) RAN1 #118 Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Proposal 1: RAN4 to define the NCD-SSB adaptation requirement for SCell first and wait RAN1’s conclusion on whether SSB adaptation can be applied to PCell.

In RAN1 #116bis meeting, NW controlled SSB adaptation is agreed. The UE triggered SSB adaptation is FFS. Thus, we propose RAN4 to focus on NW controlled SSB adaptation firstly.

RAN1 #116 Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration RAN1 #116bis Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger

Proposal 2: RAN4 to focus on NW controlled SSB adaptation firstly.

To achieve the power saving, NW may adapt the SSB periodicities. The case is similar as OD-SSB Case 2. During the transition measurement period, UE can combine the SSB before and after adaptation since the only difference is the periodicities. Subsequent to this measurement period, the measurement period requirements are corresponding to the updated SSB periodicity after transition. Thus, we peopose UE shall combine the SSB measurements before and after SSB adaptation with different periodicities for L1 measurement. However, for L3 measurement, RAN4 needs to wait other WGs’ input on whether SMTC adaptation will be introduced. If new SMTC will be introduced, UE shall follow the SMTC adaptation. Otherwise, UE may follow the same SMTC although the SSB periodicity changes.

Proposal 3: In L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. Subsequent to the transition measurement period, the cell identification and measurement period requirements will follow the SSB periodicity after adaptation.

Proposal 4: In L3 measurment, RAN4 to wait other WGs’ input on whether new SMTC after SSB adaptation will be applied.

- Summary

In this contribution, we have discussed the RS adaptation requirements. Based on the discussions, we have made following proposals and observations:

Proposal 1: RAN4 to define the NCD-SSB adaptation requirement for SCell first and wait RAN1’s conclusion on whether SSB adaptation can be applied to PCell.

Proposal 2: RAN4 to focus on NW controlled SSB adaptation firstly.

Proposal 3: In L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. Subsequent to the transition measurement period, the cell identification and measurement period requirements will follow the SSB periodicity after adaptation.

Proposal 4: In L3 measurment, RAN4 to wait other WGs’ input on whether new SMTC after SSB adaptation will be applied.

- References

- RP-234065, “Rel-19 WID: Enhancements of network energy savings for NR”, Ericsson, Apple

- RP-242354, “Revised WID - Enhancements of network energy savings for NR”, Ericsson

- R4-2413885, “WF on Netw_Energy_NR_enh”, Ericsson

- 3GPP TR 38.864 V18.1.0, “Study on network energy savings for NR”