# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416030

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C

Agenda Item:	5.3.3

Document for:	Approval

Introduction

In the RAN4#110bis meeting, one issue was raised in the contribution [1] to discuss whether MSD test configuration with intra-band UL CA should be reconsidered. And the way forward [2] was approved with the following issues to be further discussed.

Proposal 1: Companies are encouraged to consider the following options for handling the MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations: Option 1: No change from TR 38.862 guidelines Option 2: Do not consider all MSD requirements resulting from intra-band contiguous UL CA configured with non-contiguous allocations. Option 3: Do not consider only the MSD requirements resulting from intra-band contiguous UL CA configured with 1RB+1RB allocations. Proposal 2: If Option 2 or Option 3 in Proposal 1 would be considered, which release to start taking effect? Option 1: From earliest release where such MSD requirements have been specified. Option 2: Rel-18 Option 3: Rel-19 Proposal 3: Is there a need to introduce cross-band MSD requirements resulting from intra-band contiguous UL CA configured with fully allocated maximum aggregated BW? Option 1: Yes Option 2: No

In last meeting, RAN4 also discuss the MSD for CA_n41C-n79A with UL intra-band CA_n41C. There is no conclusion on this band combination. Based on the operator’s demand, they would like to check the MSD with full RB allocation as the full RB allocation is the main scenario for the deployment. In this paper, we’d like to share our views on MSD with the fully allocated maximum aggregated BW.

Discussion

fully allocated maximum aggregated BW

For CA_n41C-n79A with UL intra-band CA_n41C, if RAN4 choose the fully allocated maximum aggregated BW, the DL band n79 will not be affected by the 1st to 5th adjacent channel leakage of UL intra-band CA_n41C. Since the PA non-linearity noise (-130dBm/Hz) is lower enough and band n41 Tx filter performance at DL frequency range of band n79 can provide good rejection 40 dB above with antenna isolation. Thus, there is no need to specify MSD with fully allocated maximum aggregated BW for CA_n41C-n79A with UL intra-band CA_n41C.

Generally, separate antenna RF architecture is used for DL_n41C-n79A_UL_n41C as shown below.

Figure 1 separate antenna RF architecture for DL_n41C-n79A_UL_n41C

And the MSD calculation for CA_n41C-n79A cross band isolation is shown below. The PA noise PSD at PA output port is assumed as -130dBm/Hz. Band n79 filter rejection at 2515~2675MHz is assumed as 40dB. And n41 Tx filter rejection at 4400~5000MHz is assumed as 29 dB. 10dB antenna isolation is assumed. It can be found that there is no MSD for full RB allocation of UL n41C.

|  | parameters | n79 main path | n79 diversity path |
| --- | --- | --- | --- |
| transmit power for n41C, dBm |  | 23 | 23 |
| Tx band n41C BW(MHz, Lcrb) | 190 |  |  |
| RFFE loss, dB | 4 |  |  |
| Diplexer isolation at n41C uplink freq，dB | 10 |  |  |
| antenna isolation, dB | 10 |  |  |
| n79 receival singal at ANT port, dBm |  | 13 | 13 |
| n79 filter rejection at 2515~2675MHz, dB | 40 |  |  |
| signal After n79 filter, dBm |  | -31 | -31 |
| Typical receiver IIP2, dB | 50 |  |  |
| TX IM2 noise level refer to RX LNA input, dBm |  | -112 | -112 |
| front-end loss | 4 |  |  |
| TX IM2 noise level at ANT port, dBm |  | -108 | -108 |
| noise figure dB | 9 |  |  |
| Thermal noise at RX ant port(dBm/Hz) | -165 |  |  |
| Rx band n79 BW(MHz, NRB) | 38.16 |  |  |
| Thermal noise, dBm |  | -89.18 | -89.18 |
| n41 PA noise PSD at PA output port, dBm/Hz | -130 |  |  |
| n41 PA noise PSD at PA output port, dBm/RxBW |  | -54.18 |  |
| n41 Tx filter rejection at 4400~5000MHz, dB | 29 |  |  |
| n41 PA noise power at Rx n79 ant port at 4400~5000MHz, dBm |  | -93.18 | -93.18 |
| Total noise level at ANT port |  | -87.69 | -87.69 |
| SNR requirement for QPSK | -1 |  |  |
| REFSENSE (referred to antenna)(40MHz BW) |  | -88.69 | -88.69 |
| Implementation Margin, dB | 2 |  |  |
| combined REFSENS(40MHz BW), dBm | -89.70 |  |  |
| MSD | -0.05 |  |  |

Proposal 1: There is no MSD issue with fully allocated maximum aggregated BW for CA_n41C-n79A with UL intra-band CA_n41C.

However, in order to verify this important scenario, if necessary, RAN4 can specify the MSD test case for DL_ n41C-n79A_UL_n41C with fully allocated maximum aggregated BW.

Proposal 2: if necessary, RAN4 can specify the MSD test case for DL_ n41C-n79A_UL_n41C with fully allocated maximum aggregated BW to verify the scenario.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n79 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 4420 | 40 | 0.5 | >ACLR2 |

Summary

Proposal 1: There is no MSD with fully allocated maximum aggregated BW for CA_n41C-n79A with UL intra-band CA_n41C.

Proposal 2: if necessary, RAN4 can specify the MSD test case for DL_ n41C-n79A_UL_n41C with fully allocated maximum aggregated BW to verify the scenario.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n79 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 4420 | 40 | 0.5 | >ACLR2 |

- References

[1] R4-2404180, Reconsideration on MSD requirements with intra-band contiguous UL CA, Apple

[2] R4-2406676, WF on MSD requirements with intra-band contiguous UL CA, Apple