3GPP TSG-RAN WG4 Meeting #112bis                             R4-2416038

Hefei, China, October 14 – 18, 2024

Title:	Draft LS on enabling RRC configuration UL CA_n5-n8 with DL CA_n5-n8

Response to:

Release:	Release 19

Work Item:	NR_CADC_SUL_R19-Core

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:

Contact person:	Peng, Zhang

zhangpeng169@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

- 1	Overall description

During the Rel-18, RAN4 studied the UL/DL CA_n5A-n8A and all the outcomes of CA_n5A-n8A in Rel-18 have been captured into the clause 5.1 of TR 38.872 [1]. Finally, RAN4 only specified the DL CA_n5A-n8A with only UL in band n5 (i.e. Option 1) in Rel-18 due to the limited timeline.

In order to support UL CA_n5A-n8A feature, RAN4 is discussing the following two options in Rel-19.

-	Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL (UE is configured by RRC with two UL and two DL, i.e. not two UL and one DL referring to the LS [2] R4-2314656.)

-	Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter with partial frequency range to allow simultaneous n5 DL and n8 UL

RAN4 would like kindly request RAN2 to design the UE capability for option 2 and option 3 to explicitly indicate the different UE implementations and refer to the following characteristics.

1) For Option 2, it’s a FDD-FDD non-simultaneous Rx/Tx capability due to the frequency overlapping between DL frequency range of band n5 and UL frequency range of band n8.

2) For Option 3, the partial frequency range in UL band n8 is 904~915MHz, and the partial frequency range in DL band n5 is 869~880MHz.

3) Considering the forward compatibility, the UE capabilities can support the indication of band pair (n5, n8) in a higher order band combination, e.g. DL_n5A-n8A-n78A_UL_n5A-n8A and DL_n5A-n8A-n78A_UL_n8A-n78A.

4) If UE indicate the band n8 in the UL band list of band combination and the band n5 in the DL band list of band combination, e.g. (UL: n8, DL: n5+n8), (UL: n5+n8, DL: n5+n8), (UL: n5+n8, DL: n5+n8+n78) and (UL: n8+n78, DL: n5+n8+n78), UE has to indicate one kind of implementation from option 2 and option 3.

- 2	Actions

To TSG RAN WG2

ACTION: 	RAN4 would like respectfully request RAN2 to take the information above into account in future meetings.

- 3	Dates of next TSG-RAN WG4 meetings

TSG-RAN4 Meeting#113 	     	   18 – 22 Nov 2024	Orlando, US

TSG-RAN4 Meeting#114 	      	17 – 21 Feb 2025 	Athens, GR

- 4	References

[1] TR 38.872, Study on enhancement for 700/800/900MHz band combinations, v18.4.0

[2] R4-2314656, Reply LS on non-simultaneous UL and DL from different two bands during UL CA, RAN4