# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416039

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on UL CA_n5-n8

Agenda Item:	5.3.3

Document for:	Approval

Introduction

In the RAN#105 meeting, the basket WI [1] for NR CA/DC/SUL was approved with the DL_n5A-n8A_UL_n5A-n8A.

| Band combination configuration | UpLink configuration | BCS | Subclass | Contact Name | Contact Company | Contact Email | Other supporting companies (min.3) | RAN Plenary in which the conbination was requested |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5A-n8A | CA_n5A-n8A | 0 | Only FR1 | Bo Liu | China Telecom | liubo1@chinatelecom.cn | CATT, ZTE, sanechips | RAN#92 |

In the RAN4#112 meeting, one discussion paper [2] was submitted to continue the discussion on UL CA_n5A-n8A. Although there are some impacts on other WG(s) depending on solution(s), the impacts is very limited. That’s why RAN plenary approved the basket WI for NR CA/DC/SUL to include UL CA_n5A-n8A, and merged Rel-19 low-low combos WI [3] into the basket WI [4]. The RAN plenary minutes in the RAN#104 meeting can be found below.

RP-241337 New WID on Rel-19 NR inter-band Carrier Aggregation for Low-Low CATT, China Telecom, Spark bands Replaces New WID proposal for low-low band combinations merged into RP-241637 The document was merged.

In fact, the above situation exactly aligns with the outcome captured in TR 38.872 [5] which says “if operator requests this 2UL/2DL CA_n5-n8 in future release, it should be further decided whether option 2, option 3 or both are specified. If UE capabilities need to be updated or defined to distinguish different implementations, it’s encouraged to send LS to RAN2 for UE capabilities as soon as RAN4 has decided which options to specify”. Thus, it’s RAN plenary’s common understanding to continue the discussion of UL CA_n5A-n8A in Rel-19 basket WI even if it may have some impacts on other WG(s).

In this paper, we’d like to share some outcomes of Rel-18 discussion and our views on how to complete UL CA_n5A-n8A in Rel-19.

Rel-18 outcomes of UL CA_n5A-n8A

1) Three UE implementations for CA_n5-n8:

In Rel-18, RAN4 studied the UL/DL CA_n5A-n8A and finally RAN4 only specified the DL CA_n5A-n8A with only UL in band n5 due to the limited Rel-18 timeline. Referring to TR 38.872 [5], all the outcomes of CA_n5A-n8A in Rel-18 have been captured. During the initial discussion in Rel-18, three options were proposed below. Finally, only option 1 was specified in Rel-18 spec without UL CA_n5A-n8A feature according to the clause 5.1.5 of TR 38.872 [5].

1) Full band n5 and n8 RF filters implementation with option 1 and option2: - Option 1: Only support 1UL/2DL CA. Single UL in n5 - Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL 2) Dedicated RF filters implementation with partial frequency range - Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL According to the updated WID for 700/800/900MHz band combinations in RAN#101, uplink is only in band n5 for CA_n5-n8. Thus, only Option 1 is specified in Rel-18 specification. However, if operator requests this 2UL/2DL CA_n5-n8 in futurte release, it should be further decided whether option 2, option 3 or both are specified. If UE capabilities need to be updated or defined to distinguish different implementations, it’s encouraged to send LS to RAN2 for UE capabilities as soon as RAN4 has decided which options to specify.

2) CA_n5-n8 MSD values:

If option 3 is specified (Dedicated filter to allow simultaneous n5 DL and n8 UL), The following test configuration and MSD from n8 UL to n5 DL are agreed in Table 5.1.3.4-2 and the requirements apply.

UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n8; n5; 909; 10; 15; 25 (RBstart=0); 877.5; 5; 12.3; >ACLR2.

3) Clarification on the scenario:

For RAN2’s questions in the LS [6], the answers can be found in the replied LS [7], which are quoted below.

Clarification on the scenario: The RAN4 discussion for the 2UL scenario for CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL is based on the assumption that UE is configured by RRC with two UL and two DL, i.e. not two UL and one DL. Based on that assumption, RAN4 has the following answers for the questions in the LS R2-2306862. Answer to Question 1: - Cross carrier scheduling is transparent from RAN4’s perspective. RAN4 didn’t see any impact to RAN4 specifications. Answer to Question 2: - RAN4 expects measurement of the SCell (n5) in this scenario is needed. - In RAN4 understanding, network would avoid the collision with the DL transmission of SCell (n5) when scheduling the UL in n8. - No impact to existing RRM specifications is seen from RAN4 perspective.

It’s clear enough that UE is configured by RRC with two UL and two DL, i.e. not two UL and one DL when we discussed CA_n5-n8 with the scheduling restriction of non-concurrent n5 DL and n8 UL.

Observation 1: All the outcomes of CA_n5A-n8A in Rel-18 have been captured into the clause 5.1 of TR 38.872. In order to support UL CA_n5A-n8A feature, the following two options can be discussed in Rel-19.

1)	Full band n5 and n8 RF filters implementation with option 1 and option2:

-	Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL (UE is configured by RRC with two UL and two DL, i.e. not two UL and one DL referring to the LS R4-2314656.)

2)	Dedicated RF filters implementation with partial frequency range

-	Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

How to complete UL CA_n5A-n8A in Rel-19

Given RAN4 didn’t reach a consensus on which option would be specified, option 2 or option 3 since Rel-18, RAN4 can consider to introduce both option 2 and option 3 into the specification as a compromise to leave more flexibility for UE implementation. Referring to the clause 5.1.5 of TR 38.872, four situations are listed and the corresponding solutions were proposed as well.

However, no matter whether RAN4 choose option 2, option 3 or both in the end, it’s better to introduce the corresponding UE capability for each option to explicitly indicate the UE implementation in order to avoid the interoperation issue between the BS and UE.

There are some following reasons to introduce the explicit UE capabilities for these two kinds of implementations.

1) For option 2, the wide band filter to cover both DL frequency range of band n5 and UL frequency range of band n8 was provided in Figure 5.1.1-1 of TR 38.872. In addition, a switch is needed to implement the switching between DL band n5 and UL band n8. In total, the design of RF front end is complex and there are some restrictions for network scheduling as DL band n5 and UL band n8 can’t work simultaneously. Thus, it’s necessary to clearly indicate this kind of UE implementation to network for scheduling optimization.

2) For option 3, the dedicated filter is needed to enable the simultaneous two UL CA_n5A-n8A, but the frequency range for UL band n8 and DL band n5 should be restricted. It will lead to the restriction of network scheduling due to the partial frequency range implementation. Thus, it’s necessary to clearly indicate this kind of UE implementation to network considering the partial frequency range implementation.

3) Some companies said that CA_n5-n8 is only a band combination to require special solution. Thus, the solution that only some wordings/NOTEs are introduced into RAN4 spec to clarify the restriction is enough. However, Operators may introduce new band combinations, e.g. DL_n5-n8-n78_UL_n8-n78, in the future. It’s very hard, complex and uncertain for Network to schedule UE only based on RAN4 spec’s clarification. However, UE capability can help solve the interoperation issues.

Proposal 1: no matter whether RAN4 choose option 2, option 3 or both in the end, it’s proposed to introduce the corresponding UE capability for each option to explicitly indicate the UE implementation in order to avoid the interoperation issue between the BS and UE.

Since RAN2 can discuss the capability design in parallel without waiting for RAN4’s conclusion on the option(s), in order to complete this topic as quickly as possible, it’s proposed to send a LS to RAN2 and trigger the discussion of capability design in parallel.

Proposal 2: To send a LS to RAN2 and trigger the discussion of capability design in parallel.

Summary

Observation 1: All the outcomes of CA_n5A-n8A in Rel-18 have been captured into the clause 5.1 of TR 38.872. In order to support UL CA_n5A-n8A feature, the following two options can be discussed in Rel-19.

1)	Full band n5 and n8 RF filters implementation with option 1 and option2:

-	Option 2: Support both 1UL/2DL and 2UL/2DL CA. Non-concurrent n5 DL and n8 UL (UE is configured by RRC with two UL and two DL, i.e. not two UL and one DL referring to the LS R4-2314656.)

2)	Dedicated RF filters implementation with partial frequency range

-	Option 3: Support both 1UL/2DL and 2UL/2DL CA. Dedicated filter to allow simultaneous n5 DL and n8 UL

Proposal 1: no matter whether RAN4 choose option 2, option 3 or both in the end, it’s proposed to introduce the corresponding UE capability for each option to explicitly indicate the UE implementation in order to avoid the interoperation issue between the BS and UE.

Proposal 2: To send a LS to RAN2 and trigger the discussion of capability design in parallel.

- References

[1] RP-241833, Revised WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single SUL or two SUL cells, Ericsson, ZTE, Huawei, HiSilicon

[2] R4-2411077, Discussion on RF requirements for 2U/2D CA_n5-n8, CATT

[3] RP-241337, New WID on Rel-19 NR inter-band Carrier Aggregation for Low-Low bands, CATT, China Telecom, Spark

[4] RP-241637, New WID: Rel-19 NR Carrier Aggregation (CA)/Dual Connectivity (DC) for x bands DL with y bands UL (x<7, y<3) and Supplementary Uplink (SUL) band combinations/CA band combinations with a single	SUL or two SUL cells, RAN4 chair (Huawei)

[5] TR 38.872, Study on enhancement for 700/800/900MHz band combinations, v18.4.0

[6] R2-2306862, Reply LS on non-simultaneous UL and DL from different two bands during UL CA, TSG RAN WG2

[7] R4-2314656, Reply LS on non-simultaneous UL and DL from different two bands during UL CA, RAN4