# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416045

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the potential specification updates due to simultaneous operation with GNSS

Agenda Item:	6.26.2.1

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 approved the following Way Forward [1] for this WI.

Agreement: Further study NR NTN UE simultaneous operation with GNSS considering e.g. - Operating bands with UL close to GNSS frequencies (n254, n255) - Emissions on GNSS frequencies - Whether transmission gaps enabling successful GNSS reception are present - Potential impact on field performance - Potential need for specification updates, e.g. frequency error

Since the simultaneous operation with GNSS is the pre-condition of UE doppler shift pre-compensation, it’s necessary to clarify what the frequency error requirements are if some scenarios can’t support simultaneous operation with GNSS.

In this paper, we’d like to discuss the potential clarification in the specification for frequency error requirements when NR NTN UE simultaneous operation with GNSS can’t be achieved.

Discussion

In the RAN4#98bis-e meeting, RAN4 concluded the frequency error requirements by sending LS [2] to RAN1 based on the companies’ contributions [3] [4] [5].

Before RAN4 concluded such requirements, there is an assumption or condition, which was captured into the status report in RAN#90e [6].

RAN1 Agreements on “UL time and frequency synchronization” - An NR NTN UE in RRC_IDLE and RRC_INACTIVE states shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link. - An NR NTN UE in RRC_CONNECTED states shall be capable of at least using its acquired GNSS position and satellite ephemeris to perform frequency pre-compensation to counter shift the Doppler experienced on the service link.

It’s highlighted that An NR NTN UE shall be capable of at least using its acquired GNSS position and satellite ephemeris to calculate frequency pre-compensation to counter shift the Doppler experienced on the service link.

Based on this consensus or assumption, RAN4 specified the frequency error requirements in the approved TP [7] and highlighted that the location of the UE is explicitly provided to the UE from the test equipment.

Observation 1: RAN4 concluded the frequency error requirements including doppler pre-compensation as 0.1ppm based on the assumption that an NR NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

However, it’s identified that some of GNSS services operating in the frequency range 1559~1610MHz can’t be simultaneously supported together with NR NTN operation in some of NTN bands. Thus, it will have impacts on whether UE can achieve the 0.1ppm frequency error requirements without acquiring the GNSS location and satellite ephemeris.

From the real system and UE implementation perspective, it’s very hard to assume that UE shall be always capable of acquiring the exact GNSS location and satellite ephemeris. After going through the Rel-17 discussion in NTN topic, RAN4 never discuss the frequency error requirements for NTN UE when UE aren’t capable of acquiring the exact GNSS location and satellite ephemeris. However, in the RAN4#109 meeting, the CR [8] for Rel-18 NGSO testing work removed this clarification below as it’s just for test case.

- 6.4.1 Frequency error The NTN satellite UE basic measurement interval of modulated carrier frequency is 1 UL slot. The NTN satellite UE pre-compensates the uplink modulated carrier frequency by the estimated Doppler shift according to 3GPP TS 38.300 [9] clause 16.14.2. The mean value of basic measurements of NTN UE modulated carrier frequency shall be accurate to within ± 0.1 PPM observed over a period of 1 ms of cumulated measurement intervals compared to ideally pre-compensated reference uplink carrier frequency. [NOTE: The ideally pre-compensated reference uplink carrier frequency consists of the UL carrier frequency signalled to the UE by SAN and UL pre-compensated Doppler frequency shift.]

Observation 2: RAN4 never discuss the frequency error requirements for NTN UE when UE can’t acquire the exact GNSS location and satellite ephemeris.

However, in case reader misunderstand that current 0.1ppm frequency error requirements for NTN UE are applicable any scenarios without pre-condition, it’s necessary to clarify that current 0.1ppm frequency error requirements for NTN UE including NTN RedCap UE are only applicable when NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

Proposal 1: To clarify that current 0.1ppm frequency error requirements for NTN UE including NTN RedCap UE are only applicable when NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

Summary

Observation 1: RAN4 concluded the frequency error requirements including doppler pre-compensation as 0.1ppm based on the assumption that an NR NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

Observation 2: RAN4 never discuss the frequency error requirements for NTN UE when UE can’t acquire the exact GNSS location and satellite ephemeris.

Proposal 1: To clarify that current 0.1ppm frequency error requirements for NTN UE including NTN RedCap UE are only applicable when NTN UE is capable of acquiring the exact GNSS location and satellite ephemeris.

- References

[1] R4-2413527, Way Forward for [112][311] NR_NTN_Ph3_UE_RF, Qualcomm Inc.

[2] R4-2106174, LS reply on NTN UL time and frequency synchronization requirements, RAN4

[3] R4-2104762, Discussion on RF requirements for NTN UE, CATT

[4] R4-2106900, Reply LS to RAN4 on NTN UL time and frequency synchronization requirements (frequency), Ericsson

[5] R4-2106361, Discussion on UE UL frequency synchronization requirements in NTN, MediaTek Inc.

[6]	RP-20257, NR NTN Status Report to RP#90e, December 2020

[7] R4-2211220, TP for 38.101-5 on frequency error, Qualcomm

[8] R4-2321032, CR to 38.101-5 on clarification for NR NTN UE RF and Demod requirements test conditions, Apple, Ericsson, Samsung, MediaTek Inc