3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416046

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with DL intra-band contiguous CA

Agenda Item:	6.10.2.1

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic.

In the RAN4#111 meeting, we provided one more contribution [4]. RAN4 has agreed the following agreements in the way forward [3] for intra-band DL CA.

Issue 2-1-1: BCS for DL CA_n79C Agreement: - Reuse the existing BCS table for ATG - Clarify that uplink CA is not supported for ATG UE elsewhere for Rel-19 - FFS on whether BCS#4 and #5 are not supported for ATG UE for Rel-19 Issue 2-1-2: whether/how to reflect operation band information in spec Agreement: - The information that only DL CA is supported in Rel-19 for ATG can be reflected in the clause of operating band for ATG Issue 2-2: ATG Rx requirements applicability Agreement: - Following requirements needs to be updated for ATG CA compared with TN CA. - Max input level - ACS testing case 2 - Following requirements can reuse legacy TN CA requirements - Blocking including in-band and out of band blocking, narrow-band blocking - REFSENSE - Spurious response - Intermodulation - Following requirement is not applicable for ATG CA - No CA Rx Spurious emission - No CA narrow-band blocking - Following single carrier requirement of ATG is enough and no need to define ATG CA specific requirements - diversity characteristics requirement Issue 2-3: max input level Agreement: Table 3: Maximum input level for ATG with CA_n79C Rx Parameter; Units; ATG UE Types. Omni-directional antenna; Antenna array. Power in largest transmission bandwidth configuration CC, Plargest BW; dBm; -422; -302. -443; -323. Power in each other CC; dBm; Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)}. The applicable NR CA Bandwidth Class; MHz; C. NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 and 7.3F.2-3 for shared spectrum channel access operation with PCMAX_L,f,c as defined in clause 6.2.4. NOTE 2: Reference measurement channel is A.3.2.3 or A.3.3.3 for 64 QAM. NOTE 3: Reference measurement channel is A.3.2.4 or A.3.3.4 for 256 QAM.. Issue 2-4: ACS testing parameters for case 2 Agreement: - the principle of TN contiguous CA could be reused

In the RAN4#112 meeting, RAN4 reached the consensus on BCS and ACS test case 2 for ATG DL intra-band CA based on the WF [5].

Issue 2-1-2: whether/how to reflect operation band information in spec Agreement: - BCS 4 and 5 also needs to be specified besides BCS0. Table 1: BCS for CA_n79C NR CA configuration / Bandwidth combination set. NR CA configuration; Uplink CA configurations or single uplink carrier5; Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Maximum aggregated bandwidth (MHz); Bandwidth combination set. CA_n79C; CA_n79C; 50; 60, 80, 100; 200; 0. 60; 60, 80, 100. 80; 80, 100. 100; 100. See n79 channel bandwidths in Table 5.3.5-1 for each carrier2; 200; 4 and 5. Issue 2-2: ACS testing parameters for case 2 Agreement: - Further check the following table. - Table 3: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2 Rx Parameter; Units; NR CA bandwidth class. C. Pw in Transmission Bandwidth Configuration, per CC; dBm; -73.54 -61.55. PInterferer; dBm; -424 -305. BWInterferer; MHz; BWchannel CA. FInterferer (offset); MHz; BWchannel CA / -BWchannel CA. NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array..

It seems that most of the open issues for this objective have been addressed. Thus, we’d like to discuss how to capture the corresponding RF requirements about intra-band DL CA for ATG UE in this paper.

Discussion

In last meeting, it has been decided that BCS#0, BCS#4 and #5 are supported for ATG UE CA_n79C for Rel-19. Currently, the configurations for CA_n79C are specified below.

Table 5.5A.1-1: NR CA configurations and bandwidth combination sets defined for intra-band contiguous CA NR CA configuration / Bandwidth combination set. NR CA configuration; Uplink CA configurations or single uplink carrier5; Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Channel bandwidths for carrier (MHz); Maximum aggregated bandwidth (MHz); Bandwidth combination set. CA_n79C; CA_n79C3; 50; 60, 80, 100; 200; 0. 60; 60, 80, 100. 80; 80, 100. 100; 100. See n79 channel bandwidths in Table 5.3.5-1 for each carrier2; 200; 4 and 5. CA_n79D; -; 100; 100; 100; 300; 0. See n79 channel bandwidths in Table 5.3.5-1 for each carrier2; 300; 4 and 5. NOTE 1: 5 MHz is not applicable for 30/60 kHz SCS. NOTE 2: The aggregated bandwidth must be greater than or equal to the minimum for the bandwidth class defined in Table 5.3A.5-1, and smaller than or equal to the maximum aggregated bandwidth. NOTE 3: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 4: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 5: Only single uplink carriers with power class other than PC3 are listed..

Since RAN4 has clearly specified the CA_n79C_BCS0, CA_n79C_BCS4&5 in current specification, there are some risks to lead to some ambiguities if RAN4 specify a separate configuration table for ATG UE.

Thus, in order to keep the consistency as much as possible in the specification, it is no encouraged to create a separate configuration table for ATG UE.

Proposal 1: To reuse the existing configuration table for ATG UE supporting DL CA_n79C, i.e. no separate configuration table for ATG UE CA feature.

As most of the open issues for this objective have been addressed, it’s encouraged for companies to check the changes for this feature as proposed in Annex.

Summary

Proposal 1: To reuse the existing configuration table for ATG UE supporting DL CA_n79C, i.e. no separate configuration table for ATG UE CA feature.

- References

[1] R4-2406594, WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344, General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2410687, WF on Rel-19 ATG UE requirements, CMCC

[4] R4-2409333, Discussion on Rel-19 ATG UE RF with DL intra-band contiguous CA, Huawei, HiSilicon

[5] R4-2414322, WF on Rel-19 ATG UE RF requirements, CMCC

- Annex

- <<Start of Change>>

- 5.2J	Operating band for ATG

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NR DL intra-band carrier aggregation is designed to operate for ATG in the operating bands defined in Table 5.2J-1, where all operating bands are within FR1.

Table 5.2J-1: Intra-band contiguous CA operating bands for ATG in FR1

| NR CA DL Band | NR CA UL Band |
| --- | --- |
| CA_n79 | - |
| NOTE 1: “-” means that only single carrier is supported for ATG UL deployment. |  |

- <<Next of Change>>

- 7.3J	Reference sensitivity for ATG

- 7.3J.1	General

For ATG UE with multiple omni-directional antennas not indicating the capability antennaArrayType-r18, the reference sensitivity power level REFSENS is the minimum mean power per polarization at antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

For ATG UE with antenna array indicating the capability antennaArrayType-r18,the reference sensitivity power level REFSENS is the minimum mean power per polarization at TAB antenna connector, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

- 7.3J.2	Reference sensitivity power level

For a ATG UE(s) equipped with 2 Rx antenna ports, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in Annex A.5.1.1) with parameters specified in Table 7.3.2-1a and Table 7.3.2-1b for the applicable operating bands.

For ATG UE(s) equipped with 4 Rx antenna ports, reference sensitivity for 2Rx antenna ports shall be modified by the amount given in ΔRIB,4R in Table 7.3.2-2 for the applicable operating bands.

The reference sensitivity (REFSENS) requirement for a ATG UE shall be met with uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3.

For ATG UE supporting intra-band contiguous carrier aggregation in TDD band, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1) with parameters specified in Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, and Table 7.3.2-3.

- <<Next of Change>>

- 7.4J	Maximum input level for ATG

Maximum input level is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel. The throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4J-1.

Table 7.4J-1: Maximum input level for ATG

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Omni-directional antenna: receiver characteristics specified at the antenna connector(s) | Antenna array: receiver characteristics specified at transceiver array boundary (TAB) connectors |  |
| Power in Transmission Bandwidth Configuration | dBm | -42 | -30 | A.3.2.3 or A.3.3.3 for 64 QAM |
|  |  | -44 | -32 | A.3.2.4 or A.3.3.4 for 256 QAM |
| The applicable channel bandwidths | MHz | 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2J.2. |  |  |  |  |

For ATG UE supporting intra-band contiguous carrier aggregation in TDD band, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexs A.3.3 (with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1) with parameters and Maximum input level specified in Table 7.4J-2.

Table 7.4J-2: Maximum input level for ATG UE supporting intra-band contiguous CA

| Rx Parameter | Units | ATG UE Types |  | Reference measurement channel |
| --- | --- | --- | --- | --- |
|  |  | Omni-directional antenna | Antenna array |  |
| Power in largest transmission bandwidth configuration CC, Plargest BW | dBm | -422 | -302 | A.3.3.3 for 64 QAM |
|  |  | -443 | -323 | A.3.3.4 for 256 QAM |
| Power in each other CC | dBm | Plargest BW +10*log{(NRB,c*SCSc)/(NRB,largest BW*SCSlargest BW)} |  |  |
| The applicable NR CA Bandwidth Class | MHz | C |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2J.2. |  |  |  |  |

- <<Next of Change>>

- 7.5J	Adjacent channel selectivity for ATG

Adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an NR signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

The UE shall fulfil the minimum requirements specified in Table 7.5J-1 for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz and the minimum requirements specified in Table 7.5J-2 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. These requirements apply for all values of an adjacent channel interferer up to -42 dBm with omni-directional antenna and -30dBm with antenna array for any SCS specified for the channel bandwidth of the wanted signal. However, it is not possible to directly measure the ACS; instead the lower and upper range of test parameters are chosen as in Table 7.5J-3 and Table 7.5J-4 for verification of the requirements specified in Table 7.5J-1, and as in Table 7.5J-5 and Table 7.5J-6 for verification of the requirements specified in Table 7.5J-2. For these test parameters, the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1).

Table 7.5J-1: ACS for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 | 30 | 27 – 10log10(BWChannel /20) |
| NOTE1: ACS value is rounded to the next higher 0.5dB value |  |  |  |  |

Table 7.5J-2: ACS for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| ACS | dB | 33 |

Table 7.5J-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm |  | REFSENS + 45.5 dB | REFSENS + 42.5 dB | REFSENS + 39.5 – 10log10(BWChannel /20) |
| BWinterferer | MHz |  | 5 |  |  |
| Finterferer (offset) | MHz |  | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5J-4: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration4 | dBm | -73.55 -61.56 | -70.55 -58.56 | -67.5 + 10log10(BWChannel /20)5 -55.5 + 10log10(BWChannel /20)6 |
| Pinterferer | dBm | -425 -306 |  |  |
| BWinterferer | MHz | 5 |  |  |
| Finterferer (offset) | MHz | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 NOTE 4: 10log10(x) is rounded to the next higher 0.5dB value. NOTE 5: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 6: Pinterferer shall be set to -30dBm for antenna array. |  |  |  |  |

Table 7.5J-5: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |
| Pinterferer | dBm | REFSENS + 45.5 dB |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. |  |  |

Table 7.5J-6: Test parameters for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |
| --- | --- | --- |
|  |  | 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| Power in transmission bandwidth configuration | dBm | -73.54 -61.55 |
| Pinterferer | dBm | -424 -305 |
| BWinterferer | MHz | BWChannel |
| Finterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2 NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with an SCS equal to that of the wanted signal. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.2.2 and A.3.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

For ATG UE supporting intra-band contiguous carrier aggregation in TDD band, The UE shall fulfil the minimum requirements specified in Table 7.5J-7 for NR bands with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz. These requirements apply for all values of an adjacent channel interferer up to -42 dBm with omni-directional antenna and -30dBm with antenna array for any SCS specified for the channel bandwidth of the wanted signal. The lower and upper range of test parameters are chosen as in Table 7.5J-8 and Table 7.5J-9 for verification of the requirements specified in Table 7.5J-7

Table 7.5J-7: ACS for ATG UE supporting intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz

|  |  | NR CA bandwidth class |
| --- | --- | --- |
| Rx Parameter | Units | C |
| ACS | dB | 33.0 |

Table 7.5J-8: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 1

| Rx Parameter | Units | NR CA bandwidth class |  |
| --- | --- | --- | --- |
|  |  | C |  |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + 14 dB |  |
| PInterferer | dBm | Aggregated power + 31.5 dB |  |
| BWInterferer | MHz | BWchannel CA |  |
| FInterferer (offset) | MHz | BWchannel CA/2 + Foffset / -BWchannel CA/2 + Foffset |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1. |  |  |

Table 7.5J-9: Test parameters for intra-band contiguous CA with FDL_low ≥ 3300 MHz and FUL_low ≥ 3300 MHz, case 2

| Rx Parameter | Units | NR CA bandwidth class |
| --- | --- | --- |
|  |  | C |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -73.54 -61.55 |
| PInterferer | dBm | -424 -305 |
| BWInterferer | MHz | BWchannel CA |
| FInterferer (offset) | MHz | BWchannel CA/2 + Foffset / -BWchannel CA/2 + Foffset |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2J.2. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the carrier closest to the interferer in MHz. The interferer is an NR signal with an SCS equal to that of the closest carrier. NOTE 3: The interferer consists of the RMC specified in Annexes A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD for the DL-signal as described in Annex A.5.2.1. NOTE 4: Pinterferer shall be set to -42dBm for omni-directional antenna. NOTE 5: Pinterferer shall be set to -30dBm for antenna array. |  |  |

- <<Next of Change>>

- 7.6J	Blocking characteristics for ATG

- 7.6J.1	General

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occurs.

- 7.6J.2	In-band blocking for ATG

For ATG UE, the in-band blocking requirement defined in clause 7.6.2 applies.

For ATG UE supporting intra-band contiguous carrier aggregation in TDD band, the in-band blocking requirement defined in clause 7.6A.2.1 applies.

- 7.6J.3	Out-of-band blocking for ATG

For ATG UE, the out-of-band blocking requirement defined in clause 7.6.3 applies.

NOTE:	In 3GPP, the ATG UE out-of-band blocking specification is defined to ensure the telecommunication link and there may be other sources of interference and regulatory issues that need to be considered when designing ATG UE, i.e. avionic equipment.

For ATG UE supporting intra-band contiguous carrier aggregation in TDD band, the out-of-band blocking requirement defined in clause 7.6A.3.1 applies.

- <<Next of Change>>

- 7.7J	Spurious response for ATG

For ATG UE, the spurious response defined in clause 7.7 applies.

For ATG UE supporting intra-band contiguous carrier aggregation in TDD band, the spurious response defined in clause 7.7A.1 applies.

- <<Next of Change>>

- 7.8J	Intermodulation characteristics for ATG

- 7.8J.1	General

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its

assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

- 7.8J.2	Wide band intermodulation for ATG

For ATG UE, the wide band intermodulation requirement defined in clause 7.8.2 applies.

For ATG UE supporting intra-band contiguous carrier aggregation in TDD band, the wide band intermodulation requirement defined in clause 7.8A.2.1 applies.

- <<Next of Change>>

- 7.9J	Spurious emissions for ATG

For ATG UE, the spurious emissions as specified in clause 7.9 applies.

- <<End of Change>>