# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416048

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with UL MIMO

Agenda Item:	6.10.2.3

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic.

In last meeting, one way forward [3] was approved with some open issues.

Issue 4-1-1: ULFPTx mode for UL-MIMO Agreement: - FFS on whether to apply ULFPTx mode for ATG UE Issue 4-1-2: whether preclude the single antenna port related requirement Agreement: - FFS on whether to preclude the single antenna port related requirement. Issue 4-2-1: how to modify the NR UL MIMO requirement with ATG capability antennaArrayType-r18 Agreement: - If the legacy requirement for UL MIMO is defined at each antenna connector, - For ATG UE with antenna array, the requirement should be updated [at each TAB connector]. - If the legacy requirement for UL MIMO is defined per layer, FFS - Option 1: for ATG UE with capability antennaArrayType-r18, the requirement should be updated as sum of all TAB connectors per layer. - Option 2: for ATG UE with capability antennaArrayType-r18, the requirement should be updated per layer without explicitly emphasize the TAB connector. Issue 4-2-2: whether to apply coherent UL MIMO requirement for ATG UE Agreement: - FFS on whether to apply coherent UL MIMO requirement for ATG UE Issue 4-3: receiver requirements Agreement: - Reuse ATG single carrier requirements for following Rx requirements (ZTE, Ericsson) - REFSENSE - Max input level - ACS - Blocking - Spurious emission - Intermodulation - FFS whether to test the Rx requirements under the UL MIMO scenario since Rx requirements is the same for R18 ATG UE and R19 ATG UE supporting UL-MIMO.

In this meeting, we’d like to share our views on the remaining open issues.

Discussion

Since ATG UE can report the rated maximum output power, it’s totally different from handheld UE (power class report). Currently, the supported range of MOP for ATG UE is about 23~40dBm. Thus, the typical scenarios for handheld ULFPTx mode are 2*23dBm PA supporting 23dBm full power Tx per layer with 23dBm power class 3 report and 2*26dBm PA supporting 26dBm full power Tx per layer with 26dBm power class 2 report. However, for these scenarios, ATG UE can report higher MOP without full power mode as shown below.

| PA implementation | Handheld ULFPTx mode | ATG UE MOP report |
| --- | --- | --- |
| 2*23dBm PA | 23dBm full power Tx per layer with 23dBm power class 3 | 26dBm without ULFPTx mode |
| 2*26dBm PA | 26dBm full power Tx per layer with 26dBm power class 2 report | 29dBm without ULFPTx mode |

Since Handheld ULFPTx mode only discussed the limited kinds of RF architectures, it can’t cover all the implementations of ATG UE. For potential ATG UE supporting ULFPTx, It’s not expected to introduce additional spec impacts. Thus, as a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not. It’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

Proposal 1: As a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not. It’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

Generally, the number of Tx port for handheld UE is assumed as 2 or 4. It’s unclear what’s assumption for ATG UE supporting UL MIMO. If only polarization diversity or multiplex is assumed, it seems that only two Tx ports can be assumed for ATG UE.

Proposal 2: there is no need to apply coherent UL MIMO for ATG UE since only polarization diversity or multiplex is assumed for ATG UL MIMO.

Since no matter whether UL MIMO is configured for ATG UE or not, the impacts on the Receiver are similar. Thus, as the Rx requirements have been defined or tested for general ATG UE, there is no need to repeat the test for same requirements under the UL MIMO scenario. Otherwise, it will become the test burden for ATG UE without meaning.

Proposal 3: No need to test the Rx requirements under the UL MIMO scenario for ATG UE as general Rx requirements have been tested.

For the Tx requirements when ATG UE supporting MIMO, the method of definition for general ATG Tx requirements referring to agreed CR R4-2414321 can be used.

For Transmit modulation quality for ATG, it can be clarified below.

- 6.4J.2 Transmit modulation quality for ATG The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM. For ATG UE, the requirements for Error Vector Magnitude, Carrier leakage and EVM equalizer spectrum flatness are defined per layer. The requirements for In-band emission are defined at each transmit antenna connector or each TAB connector.

Summary

Proposal 1: As a compromise, it’s necessary to mention whether the ULFPTx mode is supported or not. It’s up to ATG UE implementation whether to leverage the existing ULFPTx mode or not since current Handheld ULFPTx mode is not always applicable to all of the ATG UE implementation.

Proposal 2: there is no need to apply coherent UL MIMO for ATG UE since only polarization diversity or multiplex is assumed for ATG UL MIMO.

Proposal 3: No need to test the Rx requirements under the UL MIMO scenario for ATG UE as general Rx requirements have been tested.

Proposal 4: For the Tx requirements when ATG UE supporting MIMO, the method of definition for general ATG Tx requirements referring to agreed CR R4-2414321 can be used. For Transmit modulation quality for ATG, it can be clarified below.

- 6.4J.2 Transmit modulation quality for ATG The requirements for transmit modulation quality defined in clause 6.4.2 shall apply for ATG UE except for the phase continuity requirements for DMRS bundling in 6.4.2.5. And the requirements for 256QAM modulation are only applicable to ATG UE indicating support of 256QAM. For ATG UE, the requirements for Error Vector Magnitude, Carrier leakage and EVM equalizer spectrum flatness are defined per layer. The requirements for In-band emission are defined at each transmit antenna connector or each TAB connector.

- References

[1] R4-2406594	WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344	General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2414322	WF on Rel-19 ATG UE RF requirements, CMCC