# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416049

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the specific wordings for Rel-19 PC2 TDD RedCap UE

Agenda Item:	6.7.1

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 reached the following Way Forward [1] for this WI.

Issue 1-1-1: PC2 RedCap UE for TDD bands Agreement: Add one sentence in the clause of 6.2I.1 to state that PC2 also applies for RedCap/eRedCap UE in a band agnostic way for TDD bands

In the RAN#105 meeting, the revised WI [2] was approved to remove the exemplary TDD bands based on the band agnostic method.

In this paper, we’d like to discuss the specific wordings as RAN4 just reached the consensus on the general principle without the agreements on the specific wordings.

Discussion

For the general clarification in the clause 6.2I, it seems that only one company has concerns that the other clause in current spec have covered the clarification.

- 6.2I Transmitter power for RedCap For Redcap and eRedCap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

As this general clarification is similar to the Rx part in clause 7.1I, we didn’t see any technical issue for this clarification. Although there are some overlapping with the other clauses, the clarification can make the spec clear without any harm. One possible improvement is to fix the correct clause number.

For the power class clarification in the clause 6.2I.1, it seems that RAN4 has reached the consensus on it. Thus, we proposed to consider the following specific wordings for PC2 RedCap UE.

- 6.1I General for RedCap For Redcap and eRedCap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.1. ### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 specified in clause 6.2.1 apply. For RedCap and eRedCap UE, the requirements for power class 2 specified in clause 6.2.1 [for TDD bands] apply.

Summary

Proposal 1: To consider the following wordings for TDD PC2 RedCap.

- 6.1I General for RedCap For Redcap and eRedCap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.1. ### 6.2I.1 Maximum output power for RedCap For Redcap and eRedCap UE, the requirements for power class 3 specified in clause 6.2.1 apply. For RedCap and eRedCap UE, the requirements for power class 2 specified in clause 6.2.1 [for TDD bands] apply.

- References

[1] R4-2414286, WF on RedCap PC2 requirements, China Telecom

[2] RP-241997, Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1, China Telecom