# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416050

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on the full NTN L band definition

Agenda Item:	5.19.2

Document for:	Approval

Introduction

In the RAN#104 meeting, a new WI [1] for extended L-band and combined L-band was established. It’s expected that Two new NR NTN bands will be introduced with the following frequency range.

- Specify a new NR NTN FDD band with a UE transmitting at 1668 – 1675 MHz and SAN transmitting at 1518 – 1525 MHz;

  - Support 5MHz channel bandwidth with 15kHz SCS

- Specify new NR NTN FDD band(s) with SAN transmitting at 1518 – 1559 MHz and UE transmitting at 1626.5 – 1660.5 MHz or 1668 – 1675 MHz;

  - Support channel bandwidths and SCS as presented in Table 4.1-1 below:

Table 4.1-1: DL Channel bandwidth and SCS system parameters

| NTN satellite band | SCS kHz | DL Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| [n25x] | 30 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 | 35 | 40 |

In the RAN4#112 meeting, RAN4 reached the consensus on band plan for this WI captured in the WF [6].

Offline Agreement: - Proposed band numbering as follows: Table 1: NR-NTN satellite bands in FR1-NTN NTN satellite operating band; Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high; Duplex mode. n253; 1668MHz – 1675MHz; 1518MHz – 1525MHz; FDD. n251; 1626.5MHz – 1660.5MHz; 1518MHz – 1559MHz; FDD. n250; 1668MHz – 1675MHz; 1518MHz – 1559MHz; FDD. NOTE: NTN satellite bands are numbered in descending order from n256..

In this paper, we’d like to share our views on this WI.

Discussion on in-band and out-of-band blocking requirements

In Rel-18, IoT NTN band 253 was introduced into TS 36.102 referring to the CR [2]. During the Rel-18, RAN4 has fully discussed how NTN UE can comply with CEPT regulation according to ECC Report 263 [3] and ETSI EN 301 681 [4], especially for blocking characteristics.

In the end of Rel-18, RAN4 reached the following agreements in the WF [5].

Issue 1-2-1: In-Band Blocking Requirements for Cat M1 + Issue 1-2-2: In-Band Blocking Requirements for Cat NB1 NB2 Agreement: - Put the action on capturing the additional blocking requirements from EN 301 681 and ECC Report 263 on hold until further information from ETSI, considering the UE implementation difficulty and the ongoing discussion in ETSI. - Send LS to ETSI to show the discrepancy and applicability to device types - Add the note in the TS and TR that the requirements would be defined after the feedback of ETSI is received. Issue 1-2-2: Out-of-Band Blocking Requirements for Cat M1 + Issue 1-2-2: In-Band Blocking Requirements for Cat NB1 NB2 Agreement: - Do not define the additional out-of-band blocking requirements in Rel-18 - Further discuss it, if needed, in the future release in TEI or WI

Thus, for these two new bands, the similar method can be considered.

Proposal 1: For two new NR NTN L bands, referring to the conclusions in Rel-18 for band 253, do not define the additional in-band / out-of-band blocking requirements in Rel-19 until RAN4 gets clear information from ETSI.

Discussion on combined L band

The frequency range for the combined L-band was proposed below.

DL frequency range: 1518 – 1559 MHz

UL frequency range: 1626.5 – 1660.5 MHz or 1668 – 1675 MHz

NOTE: UL channel allocations spanning across the 1660.5-1668 MHz gap shall not be allowed

For DL filter, it is natural to implement a full band DL filter covering frequency range 1518 – 1559 MHz. However, it’s unclear how to implement UL Tx filter with two separate frequency segment, i.e. 1626.5 – 1660.5 MHz or 1668 – 1675 MHz. The following implementations could be considered in the future.

Implementation 1: Two separate duplexers with additional switching: one duplexer is (DL 1518 – 1559 MHz, UL 1626.5 – 1660.5 MHz) and another one is (DL 1518 – 1559 MHz, UL 1668 – 1675 MHz).

Implementation 2: one big duplexer to cover the whole frequency range, i.e. DL 1518 – 1559 MHz, UL 1626.5 – 1675 MHz.

From implementation perspective, it’s very efficient for NTN UE to implement the whole UL frequency range 1626.5 – 1675 MHz. Only one duplexer (DL: 1518 – 1559 MHz, UL: 1626.5 – 1675 MHz) can support all four L-bands, i.e. n255, n253, n251 and n250.

Observation 1: it’s very efficient for NTN UE to implement only one duplexer (DL: 1518 – 1559 MHz, UL: 1626.5 – 1675 MHz) so that all four L-bands, i.e. n255, n253, n251 and n250 can be supported.

Proposal 2: When RAN4 discuss the RF requirements for the new introduced NR NTN L-bands, the implementation should be considered with only one duplexer (DL: 1518 – 1559 MHz, UL: 1626.5 – 1675 MHz) to cover all four L-bands, i.e. n255, n253, n251 and n250.

Summary

Proposal 1: For two new NR NTN L bands, referring to the conclusions in Rel-18 for band 253, do not define the additional in-band / out-of-band blocking requirements in Rel-19 until RAN4 gets clear information from ETSI.

Observation 1: it’s very efficient for NTN UE to implement only one duplexer (DL: 1518 – 1559 MHz, UL: 1626.5 – 1675 MHz) so that all four L-bands, i.e. n255, n253, n251 and n250 can be supported.

Proposal 2: When RAN4 discuss the RF requirements for the new introduced NR NTN L-bands, the implementation should be considered with only one duplexer (DL: 1518 – 1559 MHz, UL: 1626.5 – 1675 MHz) to cover all four L-bands, i.e. n255, n253, n251 and n250.

- References

[1] RP-241666, New WID proposal: Introduction of new NR NTN bands to support the Extended L-band (UL 1668-1675MHz, DL 1518-1525MHz) and the combined MSS L-band and Extended L-band ranges (DL 1518-1559 MHz, UL 1626.5-1660.5 MHz and 1668-1675 MHz), Inmarsat, Viasat, Thuraya

[2] R4-2321812, CR to TS36.102 Introduction of the Extended L-band, ZTE Corporation

[3] ECC Report 263, Adjacent band compatibility studies between IMT operating in the frequency band 1492-1518 MHz and the MSS operating in the frequency band 1518-1525 MHz, Approved 03 March 2017

[4] ETSI EN 301 681, Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES) of Geostationary mobile satellite systems, including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) under the Mobile Satellite Service (MSS), operating in the 1,5 GHz and 1,6 GHz

frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

[5] R4-2321813, WF on IoT_NTN_extLband, Moderator (Inmarsat)

[6] R4-2414287, WF on Rel-19 FR1 NTN bands, Moderator (Inmarsat)