# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416053

Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on VSAT requirements for NR NTN Ku band

Agenda Item:	6.9.2

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 initially discussed the VSAT RF requirements for NR NTN Ku band. RAN4 reached the following way forward [1] and encouraged companies to provide more information for the discussion in NR NTN Ku band.

- Issue 1-1-1: Supported VSAT Types in the NR NTN Ku band work - The current NTN VSAT types in 38.101-5 are considered in this Ku band work item - Both mechanical and electrically steered antennas can be considered - The discussion on down selecting between FR1/FR2 approaches for the NTN Ku band will not be impacted by this WF. - Issue 1-1-2: Open issues to be discussed - Companies are encourage to provide the following information for the NTN Ku band in the next meeting: - Equivalent antenna aperture for the Ku band VSAT - Aspects on whether the common Tx/Rx antenna or separate Tx/Rx antenna can be assumed for VSAT in Ku band - Parameterized array antenna model for electronic steering antenna (the table 8.1.1-1 in TR 38.921 can be referred as an example format) - The range of TRP/EIRP for the Ku band - The UL/DL budget - Regulatory requirements

In this paper, we’d like to share our views on some of these issues.

Discussion

One of the open issues in last meeting is related to Aspects on whether the common Tx/Rx antenna or separate Tx/Rx antenna can be assumed for VSAT in Ku band. According to the WID [2], only full duplex FDD mode is considered. As for HD-FDD mode, since it will have impacts on the other working groups, this objective is not considered in current WID. If only full duplex FDD mode is considered in current WID, the implementation for common Tx/Rx antenna need to implement the duplexer. For mechanical steered antenna type, the common Tx/Rx antenna with duplexer can be achieved for the band plan (DL: 10.7~12.75GHz, UL: 14~14.5GHz). For phased array antenna type, it will increase the costs of implementation if the duplexer is implemented under each antenna element considering common Tx/Rx antenna architecture. Thus, probably the separate Tx/Rx antenna will be chosen for phased array antenna type.

Observation 1: For full duplex FDD mode under the band plan #1 (DL: 10.7~12.75GHz, UL: 14~14.5GHz), the common Tx/Rx antenna with duplexer can be assumed for mechanical steered antenna type (Parabolic antenna) and the separate Tx/Rx antenna will be chosen for phased array antenna type.

The isolation of duplexer for mechanical steered antenna type need more studies to evaluate the Rx interference from Tx RF chain. Similarly, the Tx/Rx antenna isolation for phased array antenna type need more studies to evaluate the Rx interference from Tx RF chain.

Observation 2: RAN4 need to study the isolation of duplexer for mechanical steered antenna type and the Tx/Rx antenna isolation for phased array antenna type in order to evaluate the Rx performance from the interference of Tx RF chain.

In last meeting, we raised an issue whether multi-band integration (Ku band and Ka band) can be supported by one VSAT. In reality, it’s possible to support both Ka and Ku bands in one VSAT device. However, RAN4 just started to discuss the VSAT RF requirements for Ku band. From standard perspective, it’s premature to consider the multi-band integration (Ku band and Ka band) for one VSAT device in Rel-19.

Observation 3: From standard perspective, it’s premature to consider the multi-band integration (Ku band and Ka band) for one VSAT device in Rel-19.

Summary

Observation 1: For full duplex FDD mode under the band plan #1 (DL: 10.7~12.75GHz, UL: 14~14.5GHz), the common Tx/Rx antenna with duplexer can be assumed for mechanical steered antenna type (Parabolic antenna) and the separate Tx/Rx antenna will be chosen for phased array antenna type.

Observation 2: RAN4 need to study the isolation of duplexer for mechanical steered antenna type and the Tx/Rx antenna isolation for phased array antenna type in order to evaluate the Rx performance from the interference of Tx RF chain.

Observation 3: From standard perspective, it’s premature to consider the multi-band integration (Ku band and Ka band) for one VSAT device in Rel-19.

- References

[1] R4-2413525, Way Forward for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL

[2] RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat