# 3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416059

# Hefei, China, October 14 – 18, 2024

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 HAPS

Agenda Item:	5.22.1

Document for:	Approval

Introduction

In the RAN#105 meeting, the WI [1] on introduction of HAPS additional operating bands was established in Rel-19. The main objectives are shown below.

The core part of this work item includes: - Assess whether existing ACLR and ACS requirements can be reused for the newly proposed HAPS bands. - The frequency ranges other than the 694-960 MHz should be prioritized. - Specify the following NR bands for HAPS operation in TS 38.104. - n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85, n90 Note 1: HAPS can be supported using TN functionality available in Rel-17. Note 2: The frequency range of the downlink in bands n2 and n25 is limited to 1930-1980 MHz. The frequency in band n41 is limited to 2500-2690 MHz.

In this paper, we’d like to share our views on this WI.

Discussion

In Rel-17, HAPS was considered as one use case and captured in the NR NTN WI [2]. The regulatory aspects for HAPS before WRC-23 were captured into the clause 5.3 of TR 38.863 [3]. As WRC-23 updated the regularity aspects for HAPS which were justified in the Rel-19 WI, it’s better to update the clause 5.3 of TR 38.863.

Proposal 1: RAN4 can update regulatory aspects for HAPS in the clause 5.3 of TR38.863 accommodating the conclusion in WRC-23.

In Rel-17, the adjacent coexistence studies were captured into the clause 6 of TR 38.863. 2GHz carrier frequency is assumed in the table 6.1-3. The coexistence simulation results were captured into the clause 6.4.7 of scenario 7 and clause 6.4.8 of scenario 8. Two companies provided the results. It’s concluded that the required ACLR and ACS for HAPS at 5%-tile throughput loss is 24.97dB and 30dB, which is much less than the requirements for aggressor TN to victim TN scenario.

Observation 1: In Rel-17, It’s concluded that the required ACLR and ACS for HAPS at 5%-tile throughput loss is 24.97dB and 30dB based on 2GHz carrier frequency assumption.

However, in order to reuse the current components and design as much as possible, RAN4 finally agreed to reuse current BS ACLR and ACS requirements for HAPS, i.e. 45dB ACLR and 46dB ACS.

As current ACLR and ACS requirements for HAPS are high enough to guarantee the adjacent channel coexistence with TN network, RAN4 can simply reuse the existing ACLR and ACS requirements for newly HAPS bands considering that the frequency carrier assumption for adjacent channel coexistence has few impacts.

Proposal 2: RAN4 can conclude that the existing 45dB ACLR and 46dB ACS requirements for HAPS can be reused for the newly proposed HAPS bands.

Proposal 3: RAN4 can consider the following spec updates to specify the newly proposed HAPS bands.

- 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NR operating band n1, n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85, n90, which are defined in Table 5.2-1, can be applied for HAPS operation. NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31, n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1. Table 5.2-1: NR operating bands in FR1 <Omit>

Summary

Proposal 1: RAN4 can update regulatory aspects for HAPS in the clause 5.3 of TR38.863 accommodating the conclusion in WRC-23.

Observation 1: In Rel-17, It’s concluded that the required ACLR and ACS for HAPS at 5%-tile throughput loss is 24.97dB and 30dB based on 2GHz carrier frequency assumption.

Proposal 2: RAN4 can conclude that the existing 45dB ACLR and 46dB ACS requirements for HAPS can be reused for the newly proposed HAPS bands.

Proposal 3: RAN4 can consider the following spec updates to specify the newly proposed HAPS bands.

- 5.2 Operating bands NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2. NR operating band n1, n2, n3, n5, n7, n8, n20, n25, n26, n28, n34, n38, n39, n41, n67, n85, n90, which are defined in Table 5.2-1, can be applied for HAPS operation. NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation. NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31, n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1. Table 5.2-1: NR operating bands in FR1 <Omit>

- References

[1] RP-242373, New WID Proposal: Introduction of HAPS additional operating bands, SoftBank Corp., Deutsche Telekom, NTT DOCOMO, KDDI

[2] 3GPP RP-213691: "Revised WID: Solutions for NR to support non-terrestrial networks (NTN)".

[3] 3GPP TR 38.863 V18.3.0, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.