3GPP TSG-RAN4 Meeting #112bis	R4-2416061

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:	LG Electronics

Title:	HPUE for intra-band UL CA

Agenda item:	6.1.1.1.2

Document for:	Discussion

# Introduction

In this paper, we provide our views on HPUE for intra-band contiguous/non-contiguous UL CA.

# Discussion

In last RAN4 meeting, the following WF was approved [1]. This paper discusses the requirement of HPUE for intra-band UL CA.

## 1.1 MPR evaluation methodology and assumption Online agreement: - MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19 - MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19 ## 1.2 PCMAX,C Adhoc agreement: - For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC - Whether or how to reflect the above agreement in spec is FFS - For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC - Whether or how to reflect the above agreement in spec is FFS ## 1.3 PCMAX Online agreement with adhoc modification: - For intra-band contiguous ULCA with 2Tx-TxD, the upper bound of Pcmax is 29dBm - For intra-band UL NC CA with dual PA architecture, the upper bound of Pcmax is 29dBm

## MPR evaluation of PC1.5 UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

For PC1.5 contiguous intra-band ULCA, RAN4 agreed to focus on TxD architecture and whether to define the requirements based on equal PSD or PSD imbalance with 6dB.

Figure 2-1 shows the PC1.5 TxD-architecture for intra-band contiguous UL CA.

Figure 2-1. PC1.5 TxD-Architecture (2PA1LO) : Equal PSD vs up to 6dB PSD imbalance

The existing MPR requirement was specified based on equal PSD, such as, MPR(A) in Figure 2-1. If up to 6dB PSD imbalance, such as, MPR(B), and MPR(C) in Figure 2-1, is meaningful in real field and to be specified as MPR, it needs to be considered with following condition.

- 6 dB ≤ { PCMAX,CC1 -  10*log10( LCRB1*SCS1)} - { PCMAX,CC2  -  10*log10( LCRB2*SCS2) }  ≤ 6 dB

It can be as follows.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

Here, the upper bound of Pcmax,c is 29dBm per CC.

#### Proposal 1: For TxD-Architecture of PC1.5 intra-band contiguous UL CA, consider following condition if up to 6dB PSD imbalance is assumed for MPR requirement.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

The upper bound of  PCMAX,CC1 or PCMAX,CC2 is 29dBm.

For PC1.5 non-contiguous intra-band ULCA, RAN4 agreed to focus on dualPA architecture with 2 LOs and whether to define the requirements based on equal PSD or PSD imbalance with 6dB.

Figure 2-2 shows the PC1.5 dualPA-architecture for intra-band non-contiguous UL CA.

Figure 2-2. PC1.5 dualPA-Architecture (2PA2LO) : Equal PSD vs up to 6dB PSD imbalance

The existing MPR requirement was specified based on equal PSD, such as, MPR(A) in Figure 2-2. If up to 6dB PSD imbalance, such as, MPR(B), and MPR(C) in Figure 2-2, is meaningful in real field and to be specified as MPR, it needs to be considered with following condition.

- 6 dB ≤ { PCMAX,CC1 -  10*log10( LCRB1*SCS1)} - { PCMAX,CC2  -  10*log10( LCRB2*SCS2) }  ≤ 6 dB

It can be as follows.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

Here, the upper bound of Pcmax,c is 26dBm per CC.

#### Proposal 2: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider following condition if up to 6dB PSD imbalance is assumed for MPR requirement.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

The upper bound of  PCMAX,CC1 and PCMAX,CC2 is 26dBm.

## PCMAX,C of PC1.5 UE for NR TDD intra-band UL contiguous and non-contiguous CA with 2Tx

RAN4 agreed as follows.

- For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC

  - Whether or how to reflect the above agreement in spec is FFS

- For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC

  - Whether or how to reflect the above agreement in spec is FFS

The agreement needs to be reflected in specification.

For PC1.5 intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c , 29dBm, can be specified as follows.

#### Proposal 3: For intra-band UL contiguous CA with TxD, specify the upper bound of PCMAX,C, 29dBm per CC.

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX. The configured maximum output power PCMAX,c on serving cell c shall be set as within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {29dBm, PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

For PC1.5 intra-band UL non-contiguous CA with dualPA, the upper bound of Pcmax,c, 26dBm, can be specified as follows.

#### Proposal 4: For intra-band UL non-contiguous CA with dualPA, specify the upper bound of PCMAX,C, 26dBm per CC.

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX. The configured maximum output power PCMAX,c on serving cell c shall be set as within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {26dBm, PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

For PC1.5 intra-band UL non-contiguous CA with dualPA, when only one single carrier is activated with PC1.5, the upper bound of Pcmax,c, 29dBm, can be specified as follows.

#### Proposal 5: For intra-band UL non-contiguous CA with dualPA, when only one single carrier is activated with PC1.5, specify the upper bound of PCMAX,C, 29dBm.

When only one single carrier is activated with PC1.5, PCMAX_L,f,c = MIN {29dBm, PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {29dBm, PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

# Use of generally known IM3 equations to extend the already existing TX CA MPR requirements for new PA combinations / power classes

For TX CA the transmitter performance and MPR have been defined in 38.101-1 for 20+20= 23dBm (PC3) and 23+23= 26dBm (PC2) and to ensure that both SEM and out-of-band performance requirements are met. The applicability of the existing requirements also for case with unequal PSD has been debated in RAN4.

We have provided our views to this in the proposals 1-4 above and see the need to further improve the coverage of the already existing requirements. However, when it comes to defining MPR for new CA PA combinations and power classes it may be possible to avoid time consuming simulations and analysis for each case by introducing a delta value to a selected reference. This delta value can be defined using the single carrier ACLR requirement as driving factor for PA and TX line-up dimensioning and linearity, in a bit similar manner as single carrier ACLR is used as calibration method in the MPR simulation.

## IM3 analysis

When two tones or carriers are transmitted with non-linear amplifier the level of spurious emissions, IM3 products, follow the well-known formula.

OIP3= Pout + 1/2*P, where P is the difference between the level two tone signal components and IM3 products.

OIP3=PTone+12∙∆P

Figure 3-1. Definition of OIP3 with equal tone or carrier power

There are also well known IM3 formulas for the case that power of the tones (or carriers) is not equal and there are shown below.

OIP3=2∙PLow+PHigh-IM3Low2     OIP3=2∙PHigh+PLow-IM3High2

Figure 3-2. Definition of OIP3 with unequal tone or carrier power

It is also known that P for two-tone signal is also a relatively good estimate for the ACLR with fully allocated (all RBs are transmitted) NR signal. Therefore, we can use the minimum ACLR performance requirement to estimate the OIP3 for the PA (or TX line up) that is required to support certain output power level with targeted linearity. Although the post PA losses are not relevant in case of relative comparison 2dB IL in RFFE between the PA and antenna is assumed here. IL higher than 2dB is often used in the analysis done for UEs supporting multiple modes and bands, but as the RFFE IL is not impacting the outcome of the relative analysis, lower IL= 2dB is used here. Block diagram of the simplified TX model that is used is shown below.

Figure 3-3. Simplified TX architecture

The 38.101-1 defines following ACLR requirement for different UE power classes:

ACLR and PA output power can be used in OIP3 equation to derive estimates for the required PA OIP3 as shown in table below.

|  | Power class 1 | Power class 2 | Power class 3 | Power class 5 |
| --- | --- | --- | --- | --- |
| Pout (dBm) | 31 | 26 | 23 | 20 |
| Post PA losses (dB) | 2 | 2 | 2 | 2 |
| PA Pout (dBm) | 33 | 28 | 25 | 22 |
| deltaP (dB) | 37 | 31 | 30 | 27 |
| PA OIP3 (dBm) | 48.5 | 40.5 | 37 | 32.5 |

The OIP3 can then be added back to same equations to estimate the level of IM3 products in case of two TX line-ups being used to generate high-output power non-contiguous ULCA signal. This simplified method does not take into account all transmitter imperfections that impact the level of emissions, but can be used as relative metric as shown below.

Figure 3-4. Examples with 23+23 and 26+26dBm PAs

Same approach can then be applied also to other cases where two transmit line-ups are used to generate UL CA signal using the formulas for both equal and un-equal power between the tones or carriers. This result indicates that there should not be a need to define MPR values individually for each non-contiguous CA power class (or PA combination), but a delta value to the already existing requirements can be used.

The results of such analysis are collected into the table below, where column MA,delta shows the additional MPR that is needed vs. the reference, which is here PC2 (23+23=26dBm) MPR values that are already defined in the 38.101-1. Total MPR would then be MA,total= MA + MA,delta.

|  | C1 | C2 | C1+C2 | MA,delta |
| --- | --- | --- | --- | --- |
| 23+23= 26dBm | 23 | 23 | 26.0 | 0 (Reference) |
| 23+26= 27.8dBm | 23 | 26 | 27.8 | 0.6 |
| 26+26= 29dBm | 26 | 26 | 29.0 | 1.0 |
| 26+31= 32.2dBm | 26 | 31 | 32.2 | 0.8 |
| 31+31= 34dBm | 31 | 31 | 34.0 | 0.5 |
| 20+20= 23dBm | 20 | 20 | 23.0 | 0.0 |
| 20+23= 24.8dBm | 20 | 23 | 24.8 | 0.0 |

#### Proposal 6: Study possibility to avoid MPR simulations and analysis for each CA output power combination through intoducing MA,delta value (MA,total= MA + MA,delta) ,which is used to offset the reference MPR requirement to new CA output power levels and PA combinations.

# Conclusion

In this contribution, we proposed our view on HPUE for intra-band UL CA. The proposals are as follows.

#### Proposal 1: For TxD-Architecture of PC1.5 intra-band contiguous UL CA, consider following condition if up to 6dB PSD imbalance is assumed for MPR requirement.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

The upper bound of  PCMAX,CC1 or PCMAX,CC2 is 29dBm.

#### Proposal 2: For dualPA-Architecture of PC1.5 intra-band non-contiguous UL CA, consider following condition if up to 6dB PSD imbalance is assumed for MPR requirement.

{ PCMAX,CC1 -  PCMAX,CC2} – 6 ≤ 10*log10( LCRB1*SCS1LCRB2*SCS2) ≤ { PCMAX,CC1 -  PCMAX,CC2} + 6

The upper bound of  PCMAX,CC1 and PCMAX,CC2 is 26dBm.

#### Proposal 3: For intra-band UL contiguous CA with TxD, specify the upper bound of PCMAX,C, 29dBm per CC.

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX. The configured maximum output power PCMAX,c on serving cell c shall be set as within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {29dBm, PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

#### Proposal 4: For intra-band UL non-contiguous CA with dualPA, specify the upper bound of PCMAX,C, 26dBm per CC.

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX. The configured maximum output power PCMAX,c on serving cell c shall be set as within the following bounds: PCMAX_L,f,c ≤ PCMAX,f,c ≤ PCMAX_H,f,c with PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {26dBm, PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

#### Proposal 5: For intra-band UL non-contiguous CA with dualPA, when only one single carrier is activated with PC1.5, specify the upper bound of PCMAX,C, 29dBm.

When only one single carrier is activated with PC1.5, PCMAX_L,f,c = MIN {29dBm, PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } PCMAX_H,f,c = MIN {29dBm, PEMAX,c, PPowerClass – ΔPPowerClass + ΔPPowerBoost}

#### Proposal 6: Study possibility to avoid MPR simulations and analysis for each CA output power combination through intoducing MA,delta value (MA,total= MA + MA,delta) ,which is used to offset the reference MPR requirement to new CA output power levels and PA combinations.

# References

[1] R4-2414277, “WF on HPUE for CA in TN”, Samsung

[2] R4-2410750, “WF on higher power UE”, Samsung

[3] RP-240828, “New WID : UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, ATT