3GPP TSG-RAN WG4 Meeting #112bis	R4-2416073

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on RRM requirements of NTN with channel BW less than 5MHz

Agenda Item:	6.8.3.4

Document for:	Discussion

# Introduction

In the WI NR Enhanced requirements and test methodology for NR and IoT NTN, the objective: RRM of NTN with channel BW less than 5MHz was discussed in the last meeting, the consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on those issues as follows.

# Discussion

## Issue 1-2-3: On top of which requirements in TS38.133 for NTN less than 5MHz

Moderator proposal: Companies to discuss the potential impacts based on the content in the following table. RRM impacts summary due to spectrum less than 5MHz RRM requirements; NTN requirements in TS38.133 v18.5.0; Possible impacts if BW is below 5MHz. IDLE/inactive mode mobility; 4.2C Cell Re-selection for NR UE for Satellite Access; TBD. RRC connection mobility control; 6.2C RRC Connection Mobility Control for Satellite Access; TBD. Handover; 6.1C Handover for SAN; TBD. UE Tx timing, MTTD/ MRTD, timer accuracy, TA accuracy; 7.1C, 7.2C, 7.3C; TBD. RLM; 8.1C Radio Link Monitoring for Satellite Access - 8.5C Link Recovery Procedures for Satellite Access; TBD. Others, if any.

Regarding the content in the WI, which is also cited below, only 3MHz is included in the WI scope. So, we understand that 12PRBs and 15PRBs both shall be supported in 3MHz by less than 5MHz for NTN.

- Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18

Observation 1: 3MHz with 12PRBs and 15PRBs is supported in 3MHz by less than 5MHz for NTN.

Proposal 1: To support less than 5MHz for NTN, the impacted requirements comprise the below.

- RLM/BFD, which shall follow less than 5MHz for TN requirements on below parameters:

  - PDCCH transmission parameters for in -sync evaluation for a UE operating on a cell with less than 5MHz BW

  - PDCCH transmission parameters for out-of-sync evaluation for a UE operating on a cell with less than 5MHz BW

  - PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW

  - Below configurations are supported.

| Channel BW | 3MHz |  |
| --- | --- | --- |
| [DL Transmission BW] | 12 PRBs | 15 PRBs |

- Cell identification, which shall follow less than 5MHz for TN requirements on below parameters:

  - Intra-frequency measurements: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1), adding 4 samples.

  - Inter-frequency measurements: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1), adding 3 samples.

- Handover, which shall follow less than 5MHz for TN requirements on the below parameter:

  - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

- Satellite switch with re-sync, which shall follow less than 5MHz for TN requirements on the below parameter:

  - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

It’s noted that since the NTN UE requires T∆ = Trs for time tracking to both known and unknown target cells, hence T∆ = 3*Trs is adequate to cover the tracking time required by less than 5MHz and the tracking time required by NTN both, without extending it further.

6.1C.1.2.2.1 Interruption time for RACH-based handover - T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = Trs.

Similarly, satellite switch with re-sync delays requirements also includes T∆, which are quoted as follows.

- Hard satellite switch with re-sycn delay - Dswitch_unchangedPCI = TRRC + Tinterrupt = TRRC + Tsearch + Tprocessing + T∆ + Tmargin + TIU - Soft satellite switch with re-sycn delay - Dswitch_unchangedPCI = TRRC + max(t-service - t-seviceStart, Tsearch + T∆ + Tmargin) + Tprocessing + TIU

Between handover satellite switch with re-sync, the common part in the two cases is that the UE performs the same fine time tracking to a neighbor satellite other than the serving satellite, such that the time tracking requirement to satellite switch with re-sync is identical with the requirement to handover. As a result, it’s rationale to use T∆ = 3*Trs for less than 5MHz for NTN to fulfill satellite switch with re-sync requirements.

## Issue 1-3-4: RLF/BFD/CBD

WF: FFS on - Proposal 1: whether RAN4 need to define hypothetical PDCCH transmission parameter for RLF/BFD/CBD - Yes - FFS - Proposal 2: - The legacy RLM OOS/IS evaluation period can be reused or be the baseline when defining the related requirements for less than 5MHz in NTN scenario

Proposal 2: Regarding RLM and BFD, less than 5MHz for NTN follows same hypothetical PDCCH transmission parameters for less than 5MHz for TN.

## Issue 1-3-5: Measurement(SSB index detection requirements)

WF: FFS on: - Proposal 1: Whether the time period for time index detection within NTN intra/inter-frequency measurements with/without MG shall be redefined. - Option 1: Yes - Option 2: FFS - Proposal 2a: - The new SSB index detection requirements can be defined by add number of SSB samples as Rel18 TN with less than 5MHz. - Proposal 2b: - RAN4 shall define the related requirements for less than 5MHz in NTN scenario with considering Klayer1_measurement , Kmulti_SMTC and Kgap

Proposal 3: Support Proposal 2a and 2b, ‘additional SSB sample as Rel18 TN with less than 5MHz’ and ‘Klayer1_measurement , Kmulti_SMTC and Kgap’ both  are applicable to less than 5MHz for NTN.

## Issue 1-3-6: L1-RSRP Measurement

WF: FFS on - Proposal 1: - For L1-RSRP measurement based on SSB there is no RRM impacts,

Proposal 4: Regarding L1-RSRP measurement based on SSB, there is no RRM impact for it.

# Conclusion

Observation 1: 3MHz with 12PRBs and 15PRBs is supported in 3MHz by less than 5MHz for NTN.

Proposal 1: To support less than 5MHz for NTN, the impacted requirements comprise the below.

- RLM/BFD, which shall follow less than 5MHz for TN requirements on below parameters:

  - PDCCH transmission parameters for in -sync evaluation for a UE operating on a cell with less than 5MHz BW

  - PDCCH transmission parameters for out-of-sync evaluation for a UE operating on a cell with less than 5MHz BW

  - PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW

  - Below configurations are supported.

| Channel BW | 3MHz |  |
| --- | --- | --- |
| [DL Transmission BW] | 12 PRBs | 15 PRBs |

- Cell identification, which shall follow less than 5MHz for TN requirements on below parameters:

  - Intra-frequency measurements: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1), adding 4 samples.

  - Inter-frequency measurements: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1), adding 3 samples.

- Handover, which shall follow less than 5MHz for TN requirements on the below parameter:

  - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

- Satellite switch with re-sync, which shall follow less than 5MHz for TN requirements on the below parameter:

  - T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 2: Regarding RLM and BFD, follow same hypothetical PDCCH transmission parameters for less than 5MHz for TN.

Proposal 3: Support Proposal 2a and 2b, ‘additional SSB sample as Rel18 TN with less than 5MHz’ and ‘Klayer1_measurement , Kmulti_SMTC and Kgap’ both  are applicable to less than 5MHz for NTN.

Proposal 4: Regarding L1-RSRP measurement based on SSB, there is no RRM impact for it.

# References

- R4-2413974, WF for RRM of NTN with channel BW less than 5MHz, vivo