3GPP TSG-RAN WG4 Meeting #112bis	R4-2416074

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on RRM requirements of Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3

Agenda Item:	6.27.3

Document for:	Discussion

# Introduction

The WI Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3 targeting release Rel-19 was approved, which can be found in [1].

In this contribution, we present our views on the underlying impacts to RRM requirements mandated by the objectives in the WI with respect to the recent progress in RAN2 and SA2.

# Discussion

## Topic #1: Store&Forward

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2, RAN4] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required

In RAN2# 126 meetings, the agreements below were reached.

- For the uplink/downlink messages transmission for MO, from RAN2 perspective the following steps are taken as baseline for S&F satellite operation (in case only eNB is on the satellite): - The UE sends uplink data signalling to eNB when service link is available and the eNB stores it. - When feeder link is available, the eNB sends the uplink data/NAS signalling to the CN. - The eNB (same or different) receives the downlink data/NAS signalling from the CN and stores it when feeder link is available (and service link is not available). - The eNB (same or different) sends the downlink data/signalling to the UE when service link is available again - S&F indication can be provided by SIB (FFS on the details). RAN2 assumes that no NAS indication is needed - RAN2 understands legacy UEs may be barred by legacy cellBarred and cellBarredNTN

In RAN2#127 meetings, the agreements below were reached.

- RAN2 adopts the SA2 study conclusions on the possible S&F architectures as the baseline for further discussion (RAN2 will only consider the full CN and spit-MME payload options) - RAN2 will consider both single satellite pass and multiple satellite pass scenarios - RAN2 will consider both MO and MT data within scope - UE is informed whether its serving satellite is currently operating in S&F via System Information broadcast (FFS if we also need a static indication that in general the NW supports the feature) - RAN2 does not further discuss whether legacy UEs will always need to be barred in a S&F network, at least not before further progress in SA2 discussion. If there will be a need for this, mechanisms to bar legacy UEs are already in place and no further impact on RAN2 specs in expected. - RAN2 agrees there will be a way to bar legacy UEs (using legacy cellBarred and/or cellBarred-NTN bit) and still allow R19 S&F capable UEs. FFS on the exact solution (e.g. new barring bits or whether this is linked to some other indication)

In terms of the bunch of agreements in RAN2, to our understanding, in the S&F operation, relying on regenerative payload, the satellite may temporarily lose the feeder link connection and/or the service link connection. The S&F operation may be applied during the time span during the unavailable feeder link connection and/or service link connection respectively. The below are some examples of such S&F operation.

- For MO data, when the feeder link is unavailable and the service link is available, the satellite may store the UL data and forward the UL data to the NTN gateway when the feeder link becomes available.

- For MT data, when the feeder link is available and service link is invalid or UE is out of coverage, the satellite may store the DL data and later forward the DL data to UE when service link is valid or the UE enters the satellite coverage.

Furthermore, the UE is able to know whether the serving satellite is currently operating in S&F via System Information broadcast. Besides, the UE is likely to be able to know when the S&F mode will end and what satellite shall be attached after S&F, based on the information provided by SA2, which is listed as follows.

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires.The Satellite IDs are based on the SIB information broadcasted by eNB.

From RRM perspective, the measurement behavior may be impacted accordingly. Depending on the case of the invalid feeder link or the case of the invalid service link and the case of the available data to be transmitted or received, the UE may be in RRC_CONNECTED state or RRC_IDLE/INACTIVATE state. But, in most of cases, the UE is in RRC_IDLE/INACTIVATE state in the S&F mode since of no continuous and amount of data from/to the UE. Hence, we start from the analysis on measurement and cell reselection in low activity RRC state in the below.

Firstly, the measurement behavior when the feeder link is unavailable may be impacted.

- It’s straightforward that the UE shall keep measuring SSBs since the UE shall monitor paging for the possible S&F based UL transmission if the UE has data to be transmitted in S&F mode.

- However, it is unclear if the UE can skip measurement or relax measurement or conduct cell reselection in case of no (more) dedicated UL transmission by the UE.

- Another issue is that resuming measurement after ending of the S&F mode is unclear.

- Yet another issue is whether cell reselection is valid or needs to be updated when the UE is in S&F mode.

Secondly, the measurement behavior when the service link is unavailable may be impacted.

- To our understanding, the measurement in this case is similar to the measurement in the discontinuous coverage in Rel-18 IoT NTN.

- Similar to the above, resuming measurement and camping on a new satellite after ending of the S&F mode is unclear, e.g., if there will be any newly introduced indication for the UE like t-ServiceStartNeigh in Rel-18 IoT NTN.

Furthermore, other than retaining being served by the serving satellite in the S&F mode, the UE may re-attempt the Attach/TAU procedure over some satellites with respect to a timer and satellite IDs indicated by NAS messages, it implies measurement and cell selection/reselection to such satellites may be impacted, However, how those NAS messages are interpreted and transferred may be determined by RAN2, since RRM requirements have not direct relationship to NAS messages.

Observation 1: UE is able to know whether its serving satellite is currently operating in S&F via System Information broadcast, but the UE could not predict when its serving satellite will operate in S&F in advance since no early indication by NW.

Observation 2: It’s likely that UE is able to know when its serving satellite will resume normal operating from S&F operating with respect to the indication by NW.

Observation 3: Other than retaining being served by a S&F mode satellite, UE may re-attempt the Attach/TAU procedure over some satellites with respect to the information indicated by NAS messages, it implies that measurement and cell selection/reselection to such satellites may be impacted if RAN2 takes these messages into account and provides them to UE.

Proposal 1: RAN4 to study the measurement behavior in S&F mode, which may cover the below cases:

- Measurement when the feeder link is unavailable

- Measurement when the service link is unavailable

Proposal 2: During the time span when the feeder link is unavailable, UE shall keep SSB measurement since the UE shall monitor paging for the possible DL/UL transmission if the UE has data to be received/transmitted in S&F mode.

Proposal 3: During the time span when the feeder link is unavailable, the UE may skip measurement or relax measurement in case of no (more) dedicated data transmission.

Proposal 4: During the time span when the feeder link is unavailable, RAN4 to study whether cell reselection is valid or needs to be updated.

Proposal 5: During the time span when the service link is unavailable, the measurement behavior is identical to the behavior in discontinuous coverage in Rel-18 IoT NTN.

Proposal 6: Regarding resuming measurement after ending of S&F mode, RAN4 may wait for further clarifications and agreements in other WGs.

# Conclusion

Observation 1: UE is able to know whether its serving satellite is currently operating in S&F via System Information broadcast, but the UE could not predict when its serving satellite will operate in S&F in advance since no early indication by NW.

Observation 2: It’s likely that UE is able to know when its serving satellite will resume normal operating from S&F operating with respect to the indication by NW.

Observation 3: Other than retaining being served by a S&F mode satellite, UE may re-attempt the Attach/TAU procedure over some satellites with respect to the information indicated by NAS messages, it implies that measurement and cell selection/reselection to such satellites may be impacted if RAN2 takes these messages into account and provides them to UE.

Proposal 1: RAN4 to study the measurement behavior in S&F mode, which may cover the below cases:

- Measurement when the feeder link is unavailable

- Measurement when the service link is unavailable

Proposal 2: During the time span when the feeder link is unavailable, UE shall keep SSB measurement since the UE shall monitor paging for the possible DL/UL transmission if the UE has data to be received/transmitted in S&F mode.

Proposal 3: During the time span when the feeder link is unavailable, the UE may skip measurement or relax measurement in case of no (more) dedicated data transmission.

Proposal 4: During the time span when the feeder link is unavailable, RAN4 to study whether cell reselection is valid or needs to be updated.

Proposal 5: During the time span when the service link is unavailable, the measurement behavior is identical to the behavior in discontinuous coverage in Rel-18 IoT NTN.

Proposal 6: Regarding resuming measurement after ending of S&F mode, RAN4 may wait for further clarifications and agreements in other WGs.

# References

- R4-2414039, Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, MediaTek