3GPP TSG-RAN WG4 Meeting #112bis	R4-2416075

Hefei, CN, October 14th-18th, 2024

Source:	Ericsson

Title:	Discussion on reply LS to RAN1 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2

Agenda item:	6.27.1

Document for:	Discussion

# 1	Introduction

In RAN2 #126 meeting, a LS was sent to RAN4 and RAN1 [1]:

… 1. Overall Description: In RAN2#126 meeting, the following agreements related to Msg3 transmission for uplink capacity enhancement for R19 IoT NTN have been achieved: Agreements: => RAN2 focusses the study on contention-based Msg3 transmission to complete an EDT-like transaction (FFS on the details of Msg3. FFS on the procedural steps, e.g. how much we reuse of EDT and PUR procedures. FFS on allocation of resources). => If an IoT NTN UE in IDLE state is to use the new R19 contention-based procedure, the UE needs to verify/update the uplink synchronization (e.g. get GNSS fix, acquire TA) just before sending msg3.. Based on the above agreements, RAN2 kindly asks RAN4 and RAN1: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? 2. Actions: To RAN4 and RAN1: ACTION: RAN2 respectfully requests RAN4 and RAN1 to take above RAN2 progress into account and provide feedback. …

With respect to the LS, RAN4 and RAN1 gave feedbacks respectively, the content is copied as follow.

In RAN4 #112 meeting, a LS reply by RAN4 [2] was sent to RAN2 and RAN1.

# 1. Overall Description: RAN4 would like to thank RAN2 for the LS R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. RAN2 asked for RAN4 following questions: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? Q2: If the answer for Q1 is no, from RAN4 and RAN1 perspective, how the required timing accuracy for Msg3 transmission can be satisfied in this case? RAN4’s feedback: For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view. Note 1: RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any. Note 2: For NB-IoT with 15kHz SCS, the timing error may be larger than half CP. # 2. Actions: RAN4 respectfully asks RAN2 to take the above information into account.

In RAN1 #118 meeting, a LS reply by RAN1[3] was sent to RAN2 and RAN4.

… 1. Overall Description: RAN1 would like to thank RAN2 for the LS in R1-2405786/R2-2405769 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2. Regarding RAN2’s questions, following is the RAN1’s response: Q1: Whether an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy the required timing accuracy for Msg3 transmission without Msg1/Msg2? RAN1’s response: From RAN1 perspective, assuming Note 1 and Note 2 are satisfied, an RRC Idle UE with a pre-compensated TA (i.e., the one used for Msg1 transmission during random access for IoT NTN) can satisfy requirement for Msg3 transmission/reception without Msg1/Msg2 at least in the following cases: - NB-IoT with 3.75kHz SCS. - eMTC CE mode A. Note 1: RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2 Note 2: RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively. Note 3: RAN1 may further discuss the cases of NB-IoT with 15kHz SCS and eMTC CE mode B 2. Actions: To RAN2: ACTION: RAN1 respectfully asks RAN2 to take the response into account. To RAN4: ACTION: RAN1 respectfully asks RAN4 to confirm whether RAN1’s assumptions in Note 1 and Note 2 are valid. …

In this paper we analyze the LS reply R1-2407548.

# 2	Discussion

In general, the responses in R1-2407548 and R4-2414114 are identical in essence.

One difference between the two LS replies is the cases satisfying (or not satisfying) requirement for Msg3 transmission/reception without Msg1/Msg2.

- In R4-2414114, NB-IoT with 3.75kHz SCS and eMTC CE mode A.

- In R1-2407548, The cases expect for NB-IoT with 15kHz SCS

Such difference could be interpreted as resulting from adopting different parameters in calculation comparing with CP length, which has not a common view. We don’t pursue perfectly alignment between RAN1 and RAN4 in this case since the calculation is a rough arithmetic operation which is more from theoretical point of view instead a rigorous analysis taking all practical parameters into account.

Another difference between the two LS replies is the assumption on NTA_Ref  .

- In R1-2407548, Note 1 states that ‘RAN1 assumes NTA=0 for Msg3 transmission without Msg1/Msg2. NTA herein shall be NTA_Ref’.

  - We understand that it’s the agreement by RAN1 regarding NTA_Ref  parameter.

  - Apparently, the above statement matches RAN4’s concern represented by the note1 in R4-2414114 ‘RAN4 assumed that NTA_Ref is not configured by the network for MSG3 transmission without Msg1/Msg2. RAN4 will follow the conclusion of the other working groups in the future, if any’. We don’t need to update RAN4’s conclusion and response with respect to RAN1’s assumption.

- In R1-2407548, Note 2 states that ‘RAN1 assumes that the RAN4 UL synchronization requirements specified in Table 7.20A.2-1 and Table 7.24A.2-1 of TS 36.133 apply to the Msg3 transmission without Msg1/Msg2 in NB-IoT over NTN and eMTC over NTN, respectively’.

  - Similarly, R4-2414114 contains such statement ‘For Msg3 transmission without Msg1/Msg2, UE can meet UE UL transmit timing requirements defined for IoT NTN in TS 36.133[attached in Annex] from UE RRM requirement definition point of view.’ We understand it is aligned with RAN4 conclusion and response.

Observation 1: LS reply R1-2407548 is aligned with LS reply R4-2414114, RAN4 doesn’t need to raise further LS reply.

# 3 	Summary

Observation 1: LS reply R1-2407548 is aligned with LS reply R4-2414114, RAN4 doesn’t need to raise further LS reply.

# References

[1] R2-2405769, LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

[2] R4-2414114, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE

[3] R1-2407548, Reply LS to RAN2 on UL synchronization for contention based Msg3 transmission without Msg1/Msg2, ZTE