3GPP TSG-RAN WG4 Meeting #112bis	R4-2416081

Hefei, CN, October 14th-18th, 2024

Source: 	Ericsson

Title:  	Discussion on testability and interoperability issues for beam management

Agenda Item:	6.17.4.2

Document for:	Discussion

# Introduction

In last meeting 3GPP TSG RAN WG4 Meeting #112, RAN4 discussed RRM requirements for AI/ML based beam management and the discussion summary is listed in [1]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to testability and interoperability.

# Discussion

## Sub-topic 2-2 Issues related to ground truth

Issue 2-1: Reported measurements and ground truth

It was agreed in RAN4#111 that the ground truth for the predicted RSRP is as below. Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded The agreement contains many FFS and there are many practical issues related to tests that need further discussion/consideration. Multiple companies brought up several issues that can be discussed separately, however, they are grouped under the same issue.

Below are some preliminary analyses to the FFS part in the SNR issue.

The purpose of defining SNR condition is to get a RSRP measurement accuracy by minimizing (most of) measurement errors due to low SNR. Hence, determining exact SNR level may rely on the configuration of RS periodicity, density, bandwidth, subcarrier spacing and channel model in order to obtain a level which is applicable to most typical field scenarios.

Recap the procedure deriving the accuracy requirement on the measured L1-RSRP in Rel-15, that simulation defined the achievable accuracy at certain SNR level, in a same way, it is rational to extend those simulations to a higher SNR level where the L1-RSRP estimation error can be less than a threshed of X percentile.

Generating such SNR level in the test environment is another issue to be addressed, a concern is that the realization of SNR level is not only about generating a proper signal level but also the noise floor level generated as a side effect needs to be considered. To realize high SNR, the probes in test environment shall be able to generate signals at high output power level over the air. An underlying problem is that the wanted signal level can sometimes become too high to be created in the OTA environment. In that case, the noise AWGN level may be reduced. If it is reduced too far then the noise floor becomes dominant, which is not predictable. Test environment vendors are encouraged to comment on the SNR level from a test environment perspective.

Observation 1: The achievable high SNR without distorting to measurement in test environment shall be studied. Test environment vendors may provide more information on it.

If the ideal measurement of RSRP is acquirable, then the test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Still, arranging the details on the sequence and test configuration may need further investigation. In essence, to our understanding, the UE is deemed to execute and report RSRP measurement and prediction with same test configurations except for different SNR conditions.

Proposal 1: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Arranging the details on the sequence and test configuration may need further investigation.

Proposal 2: To derive SNR condition, simulation study with agreed simulation configurations is expected.

Proposal 3: UE under test is able to perform RSRP measurement and prediction with the same test configurations but different SNR conditions.

A concern about error distribution is, since the ideal measurement of RSRP is believed to still have a residual error compared with the genie RSRP, the maximal offset between the predicted RSRP and ideal measurement RSRP may be

÷ predicted RSRP error÷ +÷ ideal measurement RSRP error÷

or

÷ predicted RSRP error - ideal measurement RSRP error÷

It depends on if the two errors are positive correlated or negative correlated or noncorrelated. Figure 1 also illustrates the difference with respect to different correlation assumptions. Since the prediction result is derived from the measurement result essentially, the correlation between the prediction error and measurement error may be valid. As a result, maybe the maximal offset between the predicted RSRP and ideal measurement RSRP is less than ÷ predicted RSRP error÷  + ÷ ideal measurement RSRP error÷.

Figure 1 Predicted RSRP and ideal measurement of RSRP in test

Observation 2: Error (offset) between predicted RSRP and ideal measurement RSRP may depend on whether the two errors are positive correlated, negative correlated or noncorrelated.

At least one company in the last meeting commented that since the ideal measurement of RSRP and the predicted RSRP both are provided by the UE under test. The UE may deliberately modify the two values to be aligned better in the report.

On the contrary, the test environment hasn’t any available approach to evaluate the authenticity of the measured and predicted values. We agree on the observation and the issue shall be solved. A simple way for the test environment is adding a procedure manually changing the beam index between the prediction duration and measurement duration. By this way, the UE reports the results from the corresponding prediction and measurement with the corresponding beam indexes, but only the test environment has knowledge of the beam index mapping between each other.

Proposal 4: How to avoid cheating in the reports on RSRP measurement and prediction is a valid issue, which shall be studied. A simple e for the test environment is adding a procedure manually switch the beam indexes between the prediction duration and the measurement duration which is unknown to UE under test.

By this way, the UE reports the results from the prediction and measurement with the corresponding different beam indexes for the same beam, while the UE doesn’t know the relationship between the beam indexes, only the test environment has the knowledge of beam index mapping between each other.

## Sub-topic 2-3 Channel models in tests

The issue about the channel model to be used in prediction tests has been discussed in previous meeting without consensus. Multiple contributions discuss the channel model needs and issues in emulation. The channel model is an important component of the test setup. Issue 2-3: Channel models - Proposals - Option 1: CDL - Option 2: TDL - Option 3: AWGN - Option 4: Uma/Umi - Option 5: simplified CDL - CDL channel model with a small number of clusters. CDL simplification needs to be further discussed - Option 6: others - Recommended WF - Option 5 To be discussed whether more details about how to simplify CDL (or how to apply spatial filtering) can be agreed

In the last meeting, at least one company concerned that AI/ML model has not the capability to identify and further utilize the spatial information of TX beams, given that the only input of AI/ML model is the received power strength, e.g., RSRP. For this reason, the test configuration and channel model can be simplified as follows, also demonstrated in Figure 2:

- Option 1: A combined signal containing set A/set B beams with different power levels is generated by TE and provided by one probe.

- Option 2: Each beam in set A/set B is stimulated by one probe in TE, in an iteration manner, set A/set B beams with different power level and/or AoA are provided by one probe.

Then, the designed signals are provided to the UE under test through a AWGN channel.

Figure 2 Simplified channel models

Either Option1: the above designed signal or Option2: set A/set B beams configured with different power levels and/or AoAs simplifies the reception, the information at the input of AL/ML model after UE reception which can be acquired by AI/ML model only contains the beam IDs and the corresponding beam RSRPs.

According to our understanding, the above recommendations could be used to evaluate some specific AI/ML model functions and performance, but not for a full-scope evaluation. On top of that, at least the below issues shall be taken into consideration when we consider the real test purpose of AI/ML model.

- Issue 1: We have to consider the possibility that the UE could potentially utilize whatever information it can have in a multiple-AoA channel, e.g., the physical angles between its Rx beams, to improve its prediction. We agree that such information may not be the direct input of AI/ML model, for example, such information may be handled by a processing part prior to the AI/ML model in UE. But for a UE in the practical field deployment, the AI/ML model and processing part actually both work, and the prediction result is determined by the union of the AI/ML model and processing part.

- Issue 2: A possibility is that the UE could filter out beams in a multiple-AoA channel from the interference to improve its prediction. In a same manner, the filtering entity doesn’t belong to AI/ML model, for example, such beam filtering may be handled by a processing part prior to the AI/ML mode in UE. But for a UE in field, the AI/ML model and processing part both work, and the prediction result is determined by the union of the AI/ML model and processing part both.

- Issue 3: A common understanding is the test conditions shall be close to the field deployment conditions, in order to guarantee the validity and robustness of conformance test. Such simplified test configuration and channel model may make the conformance test to be a basic function test which cannot prove the performance especially the performance in field. In other words, the simplified channel in fact builds a dedicated test model, the UE passing the test model cannot guarantee the AI/ML model validity and performance in the practical field deployment. We think that isn’t the target that we pursue.

- In Issue 1 and Issue 2, a multiple-AoA channel may introduce extra errors to both prediction and measurement. Up to UE implementation, the error among UEs may be different, but such error shall be kept in a range, which is reflected by the final prediction error. If we ignore the error in the aforementioned simple test, it is impossible to ensure similarity between test conditions and field deployment conditions.

Since of the above issues, we doubt if a simplified AWGN channel together with a single signal with the varying power and phase can really reflect the expected effect of the conformance test.

Observation 3: The prediction accuracy in filed deployment is decided or dominated by the union of AI/ML model and other parts in UE, not only AI/ML model.

Observation 4: A simplified AWGN channel together with power changed beams may be too simple to reflect the expected effect of conformance test, instead, it more like a function test for AI/ML model, which cannot reflect field deployment conditions.

Observation 5: A CDL channel reflects more accurate field deployment conditions.

Proposal 5: RAN4 shall identify the test purpose before selecting the channel model.

- If the test is to imitate field deployment conditions, a CDL channel is preferred.

- If the test only examines the function of AI/ML model, a simplified AWGN channel together with power varying signal through OTA may work.

We prefer CDL channel, as it imitates the field deployment conditions, which is aligned with our target.

## Issue 2-4: Test setup needs

- Proposals - Option 1: - set B Tx beams: baseline 8; set A Tx beams: baseline 64 - Does the TE need to broadcast all Tx beams in the same test? - AoA - AoD - UE rotation during the test - multiple TRP (or reflections from another direction) - Option 2: - other parameters - Recommended WF - To be discussed

Referring to TR38.843, the maximum number of Set A is supposed to be 256, The number of Set B is normally set in a ratio to the number of Set A, which typically is 1/4, for BM-case1; the number of set B equals to the number of set A for BM-case2. In some simulations, it was set as 32 or 64 beams in Set A and 4, 8, 16, or even 32 in Set B.

Since the issue to be determined is determining the maximum number of Set A and Set B, other than directly proposing the exact number for test in RAN4, the number may rely on the maximal number of Set A/Set B that a UE supports mandatorily and the maximal number of Set A/Set B that a UE supports optionally depending on UE capability. From test perspective, a mandatorily supported maximal number of Set A/Set B may be chosen for test.

Proposal 6: RAN4 to agree that the maximal number of Set A/Set B is the maximal number of Set A/Set B that a UE supports mandatorily, and the exact number is left for RAN1 to decide.

We’d like to draw attention to the exact number of Set A/Set B in test, not the maximal number. Before determining the number of Set A/Set B, first of all, we’d like to raise a question: what AI/ML model performance or characteristics could/shall be verified by AI/ML model test and what is the effect caused by the beam number in it? In our opinion, the evaluation could at least verify the following concerns, taking the same number of outputs, e.g., Set A, as a refernce:

- Prediction scaling, i.e., the capability of using fewer inputs to predict same outputs.

- Prediction model computational power/complexity, i.e., the capability of handling more inputs to predict same outputs.

Since the two concerns aren’t in the same direction, to get the two diverse evaluation results, eventually more than one set of number of Set A/Set B shall be defined in perspective.

Proposal 7: For BM-case1, at least two sets of number of Set A/Set B are provided for test. Below are two examples:

- For verifying prediction model computational power/complexity, one example is that number of set A = X, number of set B = X/4.

- For verifying prediction scaling, one example is that number of set A = X, number of set B = X/16

Different accuracy requirements may be applied for different sets of number of set A and set B.

Where X may be the maximum number of Set A that a UE supports mandatorily.

Proposal 8: For BM-case2, the number of set A and set B may be the maximum number of set A and set B that a UE supports mandatorily.

Proposal 9: RAN4 to define the number of future time instances to predict, i.e., prediction window, for BM-case 2.

## Sub-topic 2-5 Beam management Test setup

Some companies provided analysis on the available test setups, pros/cons and capabilities. To progress the discussion, a more in depth analysis of the test setup capabilities is needed Issue 2-5: OTA Test setup for beam management - Proposals - Option 1: Take FR2 MIMO OTA test setup as baseline for further study analysis - Option 2: Multi-AoA IFF based system with a sparse probe layout as baseline - Option 3: existing IFF or enhance IFF test systems with single AoA for AI/ML BM test - Option 4: existing RRM test setup with 2 AoA - Option 5: continue discussion, discuss how to handle the test setup study - Option 6: others - Recommended WF - To be discussed It should be discussed how to continue the study on the test setup, whether a plenary recommendation can be made to continue the study under the FR2 OTA enhancements or in this WI or in some other SI

Regarding the issue, no agreement was achieved in the last meeting, various points of view were put forth, CDL is preferred because of the capability to deal with multiple-AoA while TDL is preferred due to its simplicity.

Referring to TR38.901, 3D Multi-Probe Anechoic Chamber (MPAC) for FR2 MIMO OTA testing is a good start point as reference methodology. And CDL channel realized by MPAC which is mainly for MIMO evolution may be a proper reference channel for AI/ML because of similarity between each other. Although TDL channel can emulate multipath effect, it is not suitable for multi-antenna transmission scheme, the reason has been interpreted in other issues.

Proposal 10: Take 3D Multi-Probe Anechoic Chamber (MPAC) for FR2 MIMO OTA testing as the start point of reference methodology for AI/ML tests.

Taking CDL channel as reference, generating a CDL channel is derived from at least the following parameters.

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- Coupling of rays within a cluster for both azimuth and elevation

- Cross polarization power ratios

- Channel coefficient

For BM-case1, which predicts the best beams in Set A with respect to Set B, which is different from Set A, at the same time domain with respect to spatial correlation between Set B and Set A. The channel with same parameters but with some random distributions and variations are applied for the training and inference respectively.

Therefore, generating 2 channel instants, channel #1 for training and channel #2 w ith same parameters for inference is applicable to BM-case1, which is demonstrated in Figure 3.

Figure 3 Channel update for BM-case 1

For BM-case2, which harnesses historical predictable non-random variation of Set B to anticipate Set A (same as Set B) for one or more future time instances with respect to temporal correlation between Set B and Set A. From test perspective, it needs some trackable changes, e.g., UE monotonous movements which reflect the temporal correlation as prerequisite of the temporal prediction.

In practice, UE travelling direction and UE velocity may determine the real temporal correlation, but the temporal correlation in test must rely on a set of consequent changes of channel parameters. At the least, establishing a set of channel instants from channel #1 to channel #m for training and channel #m+1 to channel #n for training. These channels contian a set of parameters which update sequentially and gladfully from training to inference, as depicted in Figure 4. In which, the number of m and n is determined by the measurement and prediction windows in BM-case2.

Figure 4 Channel update for BM-case 2

Wherein, the parameters needing updates between channels may at least comprise:

- Departure angles (AOD, ZOD)

- Arrival angles (AOA, ZOA)

- UE position and/or rotation

- Channel coefficient

In an illuminative example, for BM-case 2, the updates of parameters may include the below:

- AOD in channel #1 is 10 degrees,

- AOD in channel #2 is 20 degrees,

- …,

- AOD in channel #k is (10*k) degree

in order to mimic the UE’s movement.

Proposal 11: Regarding channel generation in tests (for each test configuration, a test case may contain more than one test configuration):

- For BM-case1, generating 2 channel instants including channel #1 for training and generating channel #2 with same parameters for inference.

- For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

  - Generating a set of channel instants, from channel #1 to channel #m, for training, and

  - Generating a set of channel instants, from channel #m+1 to channel #n for inference

Wherein,

  - The channels instants comprise sequent and grade updates of a set of parameters.

  - The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

  - Others aren’t precluded

## Sub-topic 2-6 Data sets

Issue 2-6:	Datasets for training/testing

Agreement: Vendors may take into account the test environment/conditions defined by RAN4 when training the UE - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training FFS on proposals to augment training data to avoid overfitting of UE models to the test environment RAN4 to strive to make the test conditions similar to field deployment conditions

The rationale behind the agreements on the issue is that the coverage of datasets for training/testing shall be broad adequately to meet or be close to the practical field deployment scenario as much as possible. In this manner, we suggest considering defining the test environment/conditions which can demonstrate the robustness of the AI/ML model.

Proposal 12: In order to avoid overfitting of UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

Since the training data are uniquely affected by the specific UE hardware and digital processing units, the error of training data could be left for implementation from each UE vendor, the effect to the predict accuracy may be covered by the overall measurement error.

# Conclusion

Observation 1: The achievable high SNR without distorting to measurement in test environment shall be studied. Test environment vendors may provide more information on it.

Observation 2: Error (offset) between predicted RSRP and ideal measurement RSRP may depend on whether the two errors are positive correlated, negative correlated or noncorrelated.

Observation 3: The prediction accuracy in filed deployment is decided or dominated by the union of AI/ML model and other parts in UE, not only AI/ML model.

Observation 4: A simplified AWGN channel together with power changed beams may be too simple to reflect the expected effect of conformance test, instead, it more like a function test for AI/ML model, which cannot reflect field deployment conditions.

Observation 5: A CDL channel reflects more accurate field deployment conditions.

Proposal 1: Test procedure shall at least comprise the below three steps:

- Acquire the predicted RSRP under the required corresponding SNR condition

- Acquire the ideal measurement of RSRP under high SNR condition

- Compare the two RSRPs acquired in the above steps

Arranging the details on the sequence and test configuration may need further investigation.

Proposal 2: To derive SNR condition, simulation study with agreed simulation configurations is expected.

Proposal 3: UE under test is able to perform RSRP measurement and prediction with the same test configurations but different SNR conditions.

Proposal 4: How to avoid cheating in the reports on RSRP measurement and prediction is a valid issue, which shall be studied. A simple way for the test environment is adding a procedure manually switch the beam indexes between the prediction duration and the measurement duration which is unknown to UE under test.

Proposal 5: RAN4 shall identify the test purpose before selecting the channel model.

- If the test is to imitate field deployment conditions, a CDL channel is preferred.

- If the test only examines the function of AI/ML model, a simplified AWGN channel together with power varying signal through OTA may work.

We prefer CDL channel, as it imitates the field deployment conditions, which is aligned with our target.

Proposal 6: RAN4 to agree that the maximal number of Set A/Set B is the maximal number of Set A/Set B that a UE supports mandatorily, and the exact number is left for RAN1 to decide.

Proposal 7: For BM-case1, at least two sets of number of Set A/Set B are provided for test. Below are two examples:

- For verifying prediction model computational power/complexity, one example is that number of set A = X, number of set B = X/4.

- For verifying prediction scaling, one example is that number of set A = X, number of set B = X/16

Different accuracy requirements may be applied for different sets of number of set A and set B.

Where X may be the maximum number of Set A that a UE supports mandatorily.

Proposal 8: For BM-case2, the number of set A and set B may be the maximum number of set A and set B that a UE supports mandatorily.

Proposal 9: RAN4 to define the number of future time instances to predict, i.e., prediction window, for BM-case 2.

Proposal 10: Take 3D Multi-Probe Anechoic Chamber (MPAC) for FR2 MIMO OTA testing as the start point of reference methodology for AI/ML tests.

Proposal 11: Regarding channel generation in tests (for each test configuration, a test case may contain more than one test configuration):

- For BM-case1, generating 2 channel instants including channel #1 for training and generating channel #2 with same parameters for inference.

- For BM-case2, plural channel instants with a series of changes of the channel parameters are necessary to reflect temporal correlation between channels. At least, it includes:

  - Generating a set of channel instants, from channel #1 to channel #m, for training, and

  - Generating a set of channel instants, from channel #m+1 to channel #n for inference

Wherein,

  - The channels instants comprise sequent and grade updates of a set of parameters.

  - The numbers of m and n depend on the measurement window and prediction window of BM-case2.

- The channel parameters may comprise one or more than one of

  - Departure angles (AOD, ZOD)

  - Arrival angles (AOA, ZOA)

  - UE position and/or rotation

  - Channel coefficient

  - Others aren’t precluded

Proposal 12: In order to avoid overfitting of UE models to the test environment only, the test (including data sets) shall be able to demonstrate the robustness of the AI/ML model and match the practical field deployment conditions.

# References

- R4-2414323 WF on AI/ML, Qualcomm