3GPP TSG-RAN WG4 Meeting #112bis		R4-2416082

Hefei, China, 14th – 18th October 2024

Agenda item:	6.4.2

Source:	Intel Corporation

Title:	Discussion on the remaining issues for Rel-19 less than 5MHz UE RF requirements

Document for:	Discussion

# Introduction

The Rel-19 NR channel BW less than 5MHz for FR1 Phase 2 work item was updated in June 2024 in [1]. The work item Core part objectives are provided below.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

The idea of the objectives is to first define generic RF requirements for inter-band NR-CA and NR-DC n100-n101 band combinations considering less than 5MHz bandwidth in FDD band n100 and 5 MHz or 10 MHz in TDD band n101, and specify general RF requirements/applicability for inter-band CA/DC.

In the last WG meeting, a WF in [2] was approved capturing the agreements on the RF scope of the item. The remaining issues in the scope of RF requirements in this item are whether and how to capture sync raster configurations and how the UE capabilities of Rel-19 can be designed and applied from RAN4 perspective. In this paper we provide our analysis on the issues and propose to send an LS to RAN2 to help work on the signalling aspects of the Rel-19 UE features.

# Discussion

## Signalling aspects for UE support of NR CA/DC combos with less than 5MHz CBW and support of 12 or 20 RB transmission bandwidth

Background

In RAN4 #112 the discussion on UE capabilities took place and the following agreements were reached [2]:

Issue 2-2: UE capability signaling - Options for further discussion - Option 1: Request RAN2 to introduce UE capability signaling to enable less than 5MHz CBW operation for CA/DC - The following Rel-18 capabilities for NR less than 5MHz CBW operation need to be extended or new capabilities defined to enable CA/DC operation scenario with one or more component carriers with less than 5MHz CBW - [support12PRB-CORESET0-r18] - [support3MHz-ChannelBW-Asymmetric-r18] - [support3MHz-ChannelBW-Symmetric-r18] - [support5MHz-ChannelBW-20PRB-CORESET0-r18] - [support12PRB-CORESET0-GSCN-41637-r18] - UE capability signalling granularity is FFS - The respective UE capabilities shall be early-implementable from Rel-18 - The details including whether to update existing Rel-18 capabilities or introduce new Rel-19 capabilities are up to RAN2 - Option 2: Do not introduce new UE capability signaling to enable less than 5MHz CBW operation for CA/DC - RAN4 to define BCSs for band combination CA_n100A-n101A or combinations with other band where 3 MHz is supported as usual where 3MHz / 5MHz channel bandwidths are listed for each band. BCS5 with supportedMinBandwidthUL-r17 set to 5 MHz can be used to indicate that a UE supports CA channel BWs other than 3 MHz for a given band. - RAN2 specs need to remove “This feature is only applicable to single-carrier operation from Rel-19. RAN2 to confirm no NBC issues. - No new RAN2 UE capability signaling is required.” - Other options are not precluded

General considerations to all CA/DC band combinations with less than 5MHz BW configurations

In Rel-18 the below UE feature groups are introduced as optional features for UE to indicate its support for less than 5MHz features and are applicable to the single carrier case.

| IE | Note |
| --- | --- |
| [support12PRB-CORESET0-r18] | General for 12PRB reception support. Per Band indication. |
| [support3MHz-ChannelBW-Asymmetric-r18] | General for asymmetric bandwidth support. Per Band indication. |
| [support3MHz-ChannelBW-Symmetric-r18] | General for symmetric bandwidth support. Mandatory support when UE supports support12PRB-CORESET0-r18. Per Band indication. |
| [support5MHz-ChannelBW-20PRB-CORESET0-r18] | Specific for band n100 20PRB BW support. Per UE indication since it only applies to band n100. |
| [support12PRB-CORESET0-GSCN-41637-r18] | Specific for band n100 12PRB BW support. Per UE indication since it only applies to band n100. |

The list of supported CA/DC band combinations which UE supports is indicated via the supportedBandCombinationList IE in the RF-Parameters IE. Further under this IE, the UE is able to indicate its supported BCS to each supported band combination.

We believe it is unnecessary to create new additional UE capabilities in Rel-19 to facilitate the use of CA/DC with bandwidths smaller than 5 MHz. The current system for signaling UE CA/DC capabilities should be sufficient to handle the 3 MHz and 5 MHz bandwidths for band n100 in both CA_n100-n101 and DC_n100-n101 configurations, as well as other BCs that involve bandwidths under 5 MHz. For example, a UE that indicates it supports the Rel-18 capabilities for 3 MHz or 5 MHz bandwidths, as mentioned earlier, and that it can handle CA/DC band combinations involving bands which have support of bandwidths under 5 MHz, is expected to fulfil all the RF requirements specified in Rel-19 WI. Conversely, if a UE does not signal support for Rel-18 capabilities with bandwidths less than 5 MHz, it is not expected to support the corresponding bandwidths and features in the indicated band combinations.

Proposal 1: No new Rel-19 signalling is needed for the work item and the UE indicating support for 3MHz (Rel-18) or 5MHz in its supported combinations and indicating support of 12PRB or 20PRB receptions in the corresponding band within the CA/DC band combination, is expected to meet all the RF requirements specified in this work item.

Specific to band n100 and GSCN 41537 and 41638

Even for the specific band combination CA/DC_n100-n101, the existing Rel-18 UE features work well together with the general CA/DC combo signalling framework. The UE is supposed to support 12PRB reception when configured with CA/DC_n100-n101 combinations as long as it indicates support both to support12PRB-CORESET0-GSCN-41637-r18 and to the combo.

Single carrier applicability constraints

In accordance with the latest TS 38.306 the existing Rel-18 UE features for less than 5 MHz CBW are restricted to be applicable to the single carrier case and technically are not supposed to be used for CA/DC. In this sense to simplify the UE capabilities framework and reuse Rel-18 signalling for CA/DC case, we propose to remove the related constraints and enable non-single-carrier operation. Furthermore, we note that such approach would allow implementations of CA/DC band combinations with NR < 5 MHz CBW starting from Rel-18 from capabilities signalling perspective.

support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHzChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation.

We propose to send an LS to RAN2 and recommend removing the single carrier constraints for NR < 5 MHz features from the Rel-18 specification and enable reuse of the respective IEs for CA/DC scenarios. In the annex to this document, we provide a draft LS to RAN2.

Our general view on the relationship between Rel-18 less than 5MHz feature and that of this item in Rel-19 is that they should be coupled from UE implementation perspective. We propose removing the constraints mentioned above directly from Rel-18 RAN2 specifications for all the listed 5 features. Given that Rel-18 work has been completed just recently, we do not expect any NBC issues with such change and would encourage companies to check this from implementation perspective.

Observation: No NBC issues are identified with respect to removing the single carrier constraints for NR < 5 MHz CBW features from the Rel-18 specifications.

Proposal 2: Send an LS to RAN2 and recommend to remove the single-carrier constraints for Rel-18 UE capabilities for NR < 5MHz features in the Rel-18 specs.

In case NBC issues are identified (in RAN4 or RAN2), an alternative approach would be to introduce a new indication for Rel-19 UEs which support the Rel-19 features. To minimize the signalling overhead we consider using a simplified 1 bit indication in Rel-19 for the UE inform the NW that it can support an extension of  all existing Rel-18 NR < 5MHz features for CA/DC scenarios. In this way we don’t need to re-introduce all existing features in Rel-19 with CA/DC applicability.

## SCell SSB transmissions on the new sync raster for band n100

In the last working group meeting the following agreement on band n100 SCell SSB transmissions was made [2]:

- For band n100 - SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-3 - SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency position same as defined in TS 38.101-1 Table 5.4.3.1-3 - Whether and how to capture the respective notes in the specification can be decided subject to the decisions on UE capability signaling in the WI RF maintenance stage. - The candidate CR text - Option 1: “SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel bandwidth or 20 PRB transmission bandwidth configuration within 5 MHz channel bandwidth can be configured only in band n100 with SS Block frequency position defined in Table 5.4.3.1-3.” - Other options not precluded

The idea of this agreement is to make sure that the network guarantees SCell SSB is transmitted on the sync raster 41637 when 12PRB is configured and on the sync raster 41638 when 20PRB is configured for the UE in band n100 when SCell is added to the UE.

The leftover discussion following the agreement is whether and how to capture the agreement in the specifications of UE RF requirements under the Rel-19 work item. In our understanding, the corresponding normative descriptions should be captured in the sync raster sub-clause so it is obvious enough to provide reference to the network deployments of SCells in band n100.

Below we provide TP to the specification:

Table 5.4.3.1-3: Additional GSCN parameters for band n100 SS Block frequency position SSREF (MHz); GSCN; Note. 920.73; 41637; Only applicable for 12 PRB transmission bandwidth configuration within 3 MHz channel with punctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. 921.45; 41638; Only applicable for 20 PRB transmission bandwidth configuration within 5 MHz channel with unpunctured PBCH defined in TS 38.211 [6] clause 7.4.3.1.. NOTE 1: UE may assume that SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel bandwidth or 20 PRB transmission bandwidth configuration within 5 MHz channel bandwidth can be configured only in band n100 with SS Block frequency position defined in Table 5.4.3.1-3..

Proposal 3: Add normative note in the sync raster sub-clause table 5.4.3.1-3 to reflect the agreement that SCell SSB transmissions in band n100 are expected to be on the new sync raster defined in the table.

A company draftCR is submitted in [3] to implement the changes proposed. Any endorsed change will be consolidated into our big draftCR for UE RF requirements in [4] after this meeting.

# Conclusion

In this paper we provide our analysis on the issues and propose to send an LS to RAN2 to help work on the signalling aspects of the Rel-19 UE features.

Proposal 1: No new Rel-19 signalling is needed for the work item and the UE indicating support for 3MHz (Rel-18) or 5MHz in its supported combinations and indicating support of 12PRB or 20PRB receptions in the corresponding band within the CA/DC band combination, is expected to meet all the RF requirements specified in this work item.

Observation: No NBC issues are identified with respect to removing the single carrier constraints for NR < 5 MHz CBW features from the Rel-18 specifications.

Proposal 2: Send an LS to RAN2 and recommend to remove the single-carrier constraints for Rel-18 UE capabilities for NR < 5MHz features in the Rel-18 specs.

Proposal 3: Add normative note in the sync raster sub-clause table 5.4.3.1-3 to reflect the agreement that SCell SSB transmissions in band n100 are expected to be on the new sync raster defined in the table.

A company draftCR is submitted in [3] to implement the changes proposed. Any endorsed change will be consolidated into our big draftCR for UE RF requirements in [4] after this meeting.

# References

- RP-241622, Revised WID: NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- R4-2414451, WF on UE RF requirements for NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- R4-2416084, draftCR on SCell SSB transmissions for band n100 with 12 or 20PRB bandwidth, Intel Corporation

- R4-2416083, Draft Big CR for less than 5MHz UE RF requirements in Rel-19, Intel Corporation

# Annex

3GPP TSG RAN WG4 Meeting #112bis		TDoc

Hefei, China, 14th – 18th October 2024

Title:	LS on UE capabilities signalling for Rel-19 NR_FR1_lessthan_5MHz_BW_Ph2 WI

Response to:

Release:	Rel-19

Work Item:	NR_FR1_lessthan_5MHz_BW_Ph2

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Zhang, Meng

meng.zhang@intel.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

# Overall description

RAN4 has had extensive discussions on the UE RF requirements for Rel-19 less than 5MHz UE configured with CA and DC band combinations. UE RF requirements are introduced as in the endorsed big draftCR in R4-2414319.

RAN4 agreed that no new Rel-19 signalling is needed for the UE RF requirements introduced in NR_FR1_lessthan_5MHz_BW_Ph2 work item. UE indicating support for 3MHz (Rel-18) or 5MHz in its supported combinations and indicating support of 12PRB or 20PRB receptions (by existing Rel-18 features) in the corresponding band within the CA/DC band combinations, is expected to meet all the RF requirements specified in this work item in Rel-19.

Furthermore, RAN4 recommends updating the description of existing Rel-18 UE features for less than 5MHz bandwidth support and remove the constraint of a single carrier operation unless any NBC issues are identified. The specific UE feature groups are listed below:

- [support12PRB-CORESET0-r18]

- [support3MHz-ChannelBW-Asymmetric-r18]

- [support3MHz-ChannelBW-Symmetric-r18]

- [support5MHz-ChannelBW-20PRB-CORESET0-r18]

- [support12PRB-CORESET0-GSCN-41637-r18]

# Actions

To RAN2

ACTION: 	RAN4 asks RAN2 to kindly consider the agreement in RAN4 and to remove the single-carrier constraints for Rel-18 UE capabilities for NR < 5MHz features in the Rel-18 specs.

# Dates of next TSG RAN WG4 meetings

3GPP RAN WG4 #113		18th – 22nd November 2024		Orlando, USA

3GPP RAN WG4 #114		17th – 21st February 2025			Athens, Greece