3GPP TSG RAN WG4 Meeting #112-bis	R4-2416090

Hefei, China 14 Oct. 2024 – 18 Oct. 2024

Agenda Item:	6.4.3

Source: 	Ericsson

Title:	Further Discussion on RRM requirements for NR CA/DC in less than 5 MHz

Document for:	Discussion

# Introduction

In 3GPP RAN4#112, we reached the following agreements on less than 5 MHz Phase 2. WF can be found in [1], copied below for reference.

Issue 2-1-1: SCell activation and deactivation delay requirements: Agreement - Specify new SCell activation delay requirements to support less than 5MHz bandwidth in the target SCell in FR1. Agreement - Start from single SCell activation delay requirements. Issue 2-1-2: Measurement requirements for deactivated SCC: Agreement - Specify SSB index identification delay requirements for UE with deactivated SCell measurements on a target cell with less than 5MHz SSB BW in FR1, taking into consideration the impacts from PBCH puncturing. Issue 2-2-1: PSCell addition and release, conditional PSCell addition delay requirements: Agreement: - Specify new PSCell addition delay requirements for PSCell configured with less than 5MHz channel bandwidth. Issue 2-2-3: Handover with PSCell, conditional handover including target MCG and SCG: Agreement: - SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. Issue 2-2-4: SCG activation delay requirements: Agreement: - FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCG activation requirement requirement. Issue 2-3-2: CGI reading: Agreement: - not consider the CGI reading related requirement in Rel-19. Issue 2-3-4: requirements applicability: Agreement: - the features/scenarios not considered in R18 less than 5MHz RRM shall not be discussed in R19 CA/DC with less than 5MHz band, e.g., CSI-RS based L1/L3/RLM/BFD/CBD measurement.

Since this was the first meeting for this WI, most of the agreements are on a high level, and we believe more detailed discussion is needed before we define a work split to write the CRs. Moreover, the following open issues need discussion in this meeting.

Issue 2-1-3: Whether intra-band CA is in the scope: - Proposals - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19. - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations. Issue 2-1-4: IDLE mode CA measurement reporting: - Proposals - Option 1: For 3MHz channel bandwidth, N2=[3+3] if the NR inter-frequency carrier for idle mode CA/DC measurement reporting is in FR1. Issue 2-2-2: PSCell change, conditional PSCell change delay requirements: - Proposals - Option 1: RAN4 to discuss the impacts of less than 5MHz, in particular PBCH puncturing, to PSCell change delay requirements. Issue 2-3-1: NW indication on PBCH BW in MO configuration and/or HO command: - Proposals - Option 1: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell. - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command. Issue 2-3-3: EMR requirements: - Proposals - Option 1: RAN4 to Study the Cell detection requirements in EMR measurements for less than 5MHz. - Option 2: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading. Further discuss: - Option 2: RAN4 to study impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading. - Option 3: not define IDLE mode EMR requirement

This paper further discusses the open issues and detailed specifications needed for the agreements.

# Discussion

## Scope of Intra-Band CA

According to Rel-19 WID [2], intra-band CA is not within the scope of this less than 5MHz CA/DC WI. Most bands supporting 3 MHz are narrow; intra-band would not be relevant in those cases. In the RAN4#112 meeting, we discussed these two options.

  - Option 1: Consider intra-band CA (both contiguous and non-contiguous) within the scope of the RRM requirements in Rel-19.

  - Option 2: Wait until RF room has concrete conclusion on intra-band CA combinations.

From the discussion, it is understood that some operators are interested to support this feature in this WI. But RF room did not have any concrete discussion on intra-band. If there is no additional effort identified, we can support option 1. In that case the WID should be revised and approved.

RAN4 to support option 1, provided that no additional effort is needed and WID should be revised and approved.

Additionally, considering the time constraint, contiguous and non-contiguous intra-band CA should be considered as a second priority. In other words, after completing the inter-band work, if time permits, we can specify contiguous and non-contiguous intra-band CA.

RAN4 should set contiguous and non-contiguous intra-band CA as a second priority, after completing the inter-band work, if time permits, we can specify contiguous and non-contiguous intra-band CA.

## SCell configured with less than 5MHz channel bandwidth Impact on CA/DC procedures

NR CA/DC are specified in chapter 8 of TS 38.133, and we look at the requirements needed to support UE with less than 5MHz capability. In the last meeting, SCell activation delay requirements were agreed upon to support less than 5MHz channel bandwidth (CBW) in the target SCell in FR1, primarily for single SCell. In this section, we discuss SCell activation and activation delay requirement aspects.

### SCell activation delay requirements

In Rel-18, for CBWs less than 5 MHz, it is specified that due to the PBCH puncturing, the UE needs additional SSB occasions to read the SSB. To understand the impact of reduced bandwidth on the SCell activation requirement in a similar context, let's examine the SCell activation delay requirements specified in clause 8.3.2 of TS 38.133. We first focus on the 4 cases relevant to FR1 SCell activation delay. These cases are:

Case 1: FR1 known SCell with meas. period <= 2400ms

Case 2: FR1 known SCell with meas. period > 2400ms

Case 3: FR1 unknown SCell with single SSB or TCI indication is provided in same PDU as SCell activation MAC CE

- Case 3a: active serving cell(s) on the same band

- Case 3b: without active serving cell(s) on the same band

Case 4: FR1 unknown SCell with multiple SSB and TCI indication is not provided in same PDU as SCell activation.

- Case 4a: semi-persistent CSI-RS is used for CSI reporting

- Case 4b: periodic CSI-RS is used for CSI reporting.

Fig 1. Scell activation delay illustration for FR1 Cases 1:3

Fig 2. Scell activation delay illustration for FR1 Case 4

As discussed in Rel-18, UE needs additional SSB to read SSB index or to decode the MIB. When we look at the above figures, we think AGC does not need to read the SSB index or MIB. UE needs to read the SSB index in case of TCI state activation is involved or to measure and report L1-RSRP. UE needs to decode MIB for SFN acquisition and SSB index reading in some scenarios. As we do not need to transmit RACH during the SCell activation phase, MIB decoding is not needed. Since the cell search, TCI state activation or L1-RSRP measurement is involved in case 3b, case 4a and 4b, we think these requirements may need additional SSB samples. As one can see from Fig 1 and fig 2, in case 3b, 4a and 4b, the SSB-based cell search will take place, which is required to capture the impacts of PBCH puncturing.

The PBCH Puncturing impacts only case 3b and case 4.

To capture the PBCH puncturing impact, RAN4 should capture the following texts in clause 8.3.2.

If the SCell is unknown and belongs to FR1, and if one of the following conditions is met

-	 ‘ssb-PositionInBurst’ indicates only one SSB is being actually transmitted, or

-	 ‘ssb-PositionInBurst’ indicates multiple SSBs and TCI indication is provided in same MAC PDU with SCell activation,

provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

TFirstSSB_MAX + TSMTC_MAX + Trs + 5ms, if the following conditions are met,

-	the SCell is contiguous to an active serving cell in the same band, and

-	its ssb-PositionInBurst is same as the one of contiguous FR1 active serving cell, and

-	its SMTC offset is same as the one of contiguous FR1 active serving cell, and

-	its RTD with contiguous FR1 active serving cell is smaller than or equal to 260ns, and its reception power difference with contiguous FR1 active serving cell is smaller than or equal to 6dB;

Otherwise,

-	TFirstSSB_MAX + TSMTC_MAX + 2*Trs + 5ms, where unknown target cells operating with 20 PRB SSB BW.

TFirstSSB_MAX + TSMTC_MAX + 4*Trs + 5ms, where unknown target cells operating with 12 PRB SSB BW.

And otherwise, provided that the side condition Ês/Iot ≥ -2dB is fulfilled, Tactivation_time is:

If  unknown target cells operating with 20 PRB SSB BW.

6ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

If unknown target cells operating with 12 PRB SSB BW

6ms + TFirstSSB_MAX + TSMTC_MAX + 3*Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP), if semi-persistent CSI-RS is used for CSI reporting,

-	3ms + TFirstSSB_MAX + TSMTC_MAX + 3*Trs + TL1-RSRP, measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay), if periodic CSI-RS is used for CSI reporting.

For case 3b, SSB index reading is needed. It can be performed during the fine timing acquisition phase as agreed in Rel-18.

For case 4a and 4b, SSB index reading is needed during the cell search phase as UE need to perform L1-RSRP.

### Multiple SCell activation

In RAN4#112, it is agreed that, primarily, this work item will focus on Single SCell activation. However, it is noted that multiple SCell activation has a similar impact of reduced bandwidth to single SCell activation. Due to time constraints, RAN4 should limit the WI scope to Single SCell activation.

RAN4 to decide and limit the scope of the WI to Single SCell considering the time constraint.

## PSCell configured with less than 5MHz bandwidth in NR-DC

### PSCell addition delay

In TS 38.133, the requirements for NR dual connectivity (DC) specify the delay within which the UE should be able to configure a PSCell. To understand the impact of reduced bandwidth on the addition delay of the PSCell, let's examine the PSCell addition delay requirements as outlined in clause 8.9.2.

Upon receiving PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell in FR1 or FR2 no later than in slot n+Tconfig_PSCellNR slot length: where: Tconfig_PSCell = TRRC_delay + Tprocessing + Tsearch + T∆ + TPSCell_ DU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20ms when target cell is in FR1 and Tprocessing = 40 ms when target cell is in FR2. Tsearch is the time for AGC settling and PSS/SSS detection. If the target cell is known, Tsearch = 0 ms. If the target cell is unknown and the target cell Ês/Iot ≥ -2dB, Tsearch = 3*N* Trs ms. N = 1 when target cell is in FR1, N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2 T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms for a known or unknown PSCell. TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

In Rel-18, for less than 5 MHz, it is specified that due to reduced BW UE takes more time for SSB reading and fine time tracking and acquiring full timing information of the target cell. We introduced new requirements for T∆ for HO requirement. In the above requirements, PSCell activation for NR-DC will also be impacted by the shortened BW. In order to capture that RAN4 should capture the following texts in clause 8.9.2.

T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

RAN4 to capture the following text for PSCell activation delay for NR-DC.

- T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW.

- T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

### Conditional PSCell addition delay

Conditional PScell addition delay requirements are specified in clause 8.9A.2. Below is the text copied from the spec.

Upon receiving conditional PSCell addition in subframe n, the UE shall be capable to transmit PRACH preamble towards PSCell no later than in subframe n + Tconfig_PSCell_Addition_Conditional: Where: Tconfig_PSCell_Addition_Conditional = TRRC_delay + TEvent_DU + Tmeasure + TUE_preparation + Tprocessing + T∆ + TPSCell_ DU + 2 ms TRRC_delay is the RRC processing delay defined in Clause 11.2 in 36.331 [16], which is the corresponding RRC message embedded in E-UTRA RRC message, otherwise it is the RRC procedure delay defined in clause 12 in TS 38.331 [2] for processing the conditional PSCell addition command. TEvent_DU is the delay uncertainty which is the time from when the UE successfully decodes a conditional PSCell addition command until a condition exists at the measurement reference point which will trigger the conditional PSCell addition. Tmeasure is the measurements time stated in clause 8.9A.2.1. TUE_preparation is the UE preparation time for conditional PSCell addition, and starts after UE realizes the condition of PSCell addition is met and identity of the PSCell is determined. TUE_preparation is up to 10 ms. Tprocessing is the SW processing time needed by UE, including RF warm up period. Tprocessing = 20 ms when PSCell is in FR1, and Tprocessing = 40 ms when PSCell is in FR2. T∆ is time for fine time tracking and acquiring full timing information of the target cell. T∆ = 1*Trs ms. TPSCell_ DU is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TPSCell_ DU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. Trs is the SMTC periodicity of the target cell if the UE has been provided with an SMTC configuration for the target cell in PSCell addition message, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms.

We can see that due to the reduced BW T∆ will be impacted. fine time tracking and acquiring full timing information of the target cell require more time.

RAN4 to capture the following text for Conditional PSCell activation delay for NR-DC.

- T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW.

- T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

### Handover with PSCell from NR-DC to NR-DC

The text copied from TS 38.133 shows that the Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2. as clause 6.1.1.2.2 already have captured the impact of reduced BW the HO with PSCell-PCell and PSCell-PSCell can be reused without any changes in the spec.

##### 6.1.5.4.1 HO with PSCell – PCell Interruption time The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When handover with PSCell from NR-DC to NR-DC is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: - Tsearch, TIU, T∆ and Tmargin are the same as defined in clause 6.1.1.2.2. - Tprocessing is the SW processing time needed by UE, including RF warm up period. For FR1-FR1 NR-DC to FR1-FR1 NR-DC or FR1-FR2 NR-DC to FR1-FR2 NR-DC, Tprocessing = 30 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 25 ms. For FR1-FR1 NR-DC to FR1-FR2 NR-DC or FR1-FR2 NR-DC to FR1-FR1 NR-DC, Tprocessing = 50 ms if SMTC of the target unknown PSCell is configured in targetcellSMTC-SCG-r16 but not configured in reconfigurationWithSync. Otherwise, Tprocessing = 45 ms.

The HO requirements with PSCell-PCell and PSCell-PSCell can be reused for NR CA/DC in less than 5MHz.

### NR Conditional Handover Requirements

Like NR handover, NR conditional handover (CHO) also aims to change the NR PCell to another NR cell. In Rel-18, we considered Rel-15 as a baseline. Therefore, the CA/DC requirements introduced in Rel-16 are not considered in the less than 5MHz WI for Rel-18. CHO was also not considered in Rel-18. In Rel-19, RAN 4 should consider and define CHO requirements.

RAN4 should define conditional handover requirements to support NR CA/DC for less than 5 MHz.

Furthermore, as specified in clause 6.1.1.2.2 Rel-18, interruption time for fine time tracking and acquiring full timing information of the target cell T∆ should be specified as:

- T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where,

- T∆ = Trs for target cells operating with 20 PRB SSB BW.

- T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

RAN4 to capture the following text for NR Conditional Handover.

- T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where,

- T∆ = Trs for target cells operating with 20 PRB SSB BW.

- T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

### Measurement requirements for deactivated SCC

In Release 18, based on simulation results, for different SNR side condition, number of SSB extended for SSB index plus MIB reading was agreed. We present the agreements in table below.

| SNR side condition (Es/Iot) | Number of SSB extended for SSB index + MIB reading |
| --- | --- |
| Es/Iot ≥ -2 dB | 2 |
| Es/Iot ≥ -4 dB | 3 |
| Es/Iot ≥ -6 dB | 4 |

Intra-frequency cell detection side condition is -6dB, and 4 SSB samples are extended for intra-frequency cell detection. Inter-frequency cell detection side condition is -4dB, and 3 SSB samples are extended for inter-frequency cell detection.

In Rel-19 WI scope, RAN4 supposed to specify the requirements for inter-band CA. Though the SCell is in inter-band, since it is a serving cell, SCells measurement is considered as intra-frequency measurement. Following the simulation results summarized in above table, we think time index detection needs to be extended by 4 samples for SCell when the SCell is operating in less than 5 MHz.

SSB time index detection for inter-frequency CA is extended by 4 samples.

The requirement which needs to be captured in spec is shown below.

Table1: Time period for time index detection, deactivated SCell with 12 PRB SSB (FR1)

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | Ceil(3+4 x Kp) x measCycleSCell x CSSFintra |
| DRX cycle≤ 320ms | Ceil(3+4 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra |
| DRX cycle> 320ms | Ceil(3+4 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra |
| NOTE 1: The requirements also apply to deactivated SCG SCell. |  |

### SCG activation

Similar to the PSCell activation delay requirement specification, reduced CBWs may also impact SCG activation time. Below are the requirements copied from clause 8.17 of TS 38.133.

Upon receiving SCG activation command in slot n, the UE shall be capable to transmit PRACH preamble or PUCCH or PUSCH towards PSCell no later than in slot n+Tactivation_timeNR slot length , where: Tactivation_time = TRRC_delay + Tprocessing + Tsearch + T∆ + TIU + 2 ms TRRC_delay is the RRC procedure delay as specified in TS 38.331 [2]. Tprocessing is the SW processing time needed by UE, including RF warm up period. When PSCell is activated from deactivated state, if any PSCell parameter is modified, Tprocessing = 20ms. Otherwise, Tprocessing = 5 ms. Tsearch is the time for AGC settling and PSS/SSS detection. For RACH based PSCell activation, if the FR1 or FR2 PSCell is known, Tsearch = 0 ms. If the FR2 PSCell is unknown and Es/Iot ≥ -2 dB, then Tsearch = 24* Trs ms. If the target cell is an unknown FR1 PSCell and Es/Iot ≥ -2 dB, then Tsearch =3* Trs ms. For RACH-less based PSCell activation, if bfd-and-RLM is configured and TCI state is known, Tsearch = 0 ms. There are no requirements if TCI state is unknown. T∆ is time for fine time tracking and acquiring full timing information of the target PSCell. T∆ = 1*Trs ms. TIU: When RACH based PSCell activation is configured, it is the delay uncertainty in acquiring the first available PRACH occasion in the PSCell. TIU is up to the summation of SSB to PRACH occasion association period and 10 ms. SSB to PRACH occasion associated period is defined in Table 8.1-1 of TS 38.213 [3]. When RACH-less based PSCell activation is configured, it is the uncertainty in acquiring the first PUSCH transmission occasion [or SR on PUCCH]. Trs is the SMTC periodicity of the PSCell if the UE has been provided with an SMTC configuration for the target cell in SCG activation command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs = 5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5.

The shortened BW has impact on T∆. Hence, RAN4 should capture the following texts in clause 8.17.

T∆ is time for fine time tracking and acquiring full timing information of the target cell, where,

T∆ = Trs for target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

RAN4 to capture the following text for SCG activation delay.

- T∆ = Trs for target cells operating with 20 PRB SSB BW.

- T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

## Other RRM issues

### NW indication on PBCH BW in MO configuration and/or HO command

It is agreed on the RAN4# 112 RF session that no new signalling from the network to inform UE of the number of PRB transmissions is required [2].

Issue 2-3: Network signaling on Scell transmission bandiwdth • Agreement (online session) o No new signalling from network to inform UE the number of PRB transmission is required in the RF scope of this item

PRB-related information is already possible to get from the existing signalling. Hence, we believe that RAN4 RRM should not introduce new network signalling on PRB transmission in alignment with RF.

RAN4 RF discussed and conclude to not introduce new network signalling on PRB transmission.

RAN4 RRM should not introduce new network signalling on PRB transmission in alignment with RF.

### Impact on Early Measurement Reporting (EMR)

EMR is intended to configure a candidate SCell addition. However, SCell activation has evolved from Release 15 to Release 18 with reduced delays, among other improvements. If the UE has measured this candidate cell through EMR or idle/inactive inter-frequency measurements, and the SSB has been detected, the activation delay should be reduced. We don't see the harm in including EMR for less than 5MHz. Hence, we support option 2.

RAN4 to discuss and study the impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading.

In Rel-18, time index detection requirements are specified for CBWs less than 5MHz. SSB index reading on a target cell with less than 5MHz SSB BW in FR1 impacts cell detection requirements. Hence, Table 4.4.2.2-1 should be checked for any impact on EMR due to reduced BW.

### Potential impact on other RAN WG (RAN1/RAN2).

According to R1-2405566, the Feature Groups (FG) in Rel-18 only apply for single-carrier operation to support less than 3MHz channel bandwidth in DL and UL. Thus, depending on the agreements reached on the support CA/DC in less than 5MHz CBW, other working groups (WGs) i.e. RAN1 and RAN2 will need to be informed for them to identify any specification impact or update on the UE Feature list to reflect the support of this feature.

RAN4 to send LS to inform potential impacts to other WGs in the November meeting after the RRM requirements are finalized.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 aspects for NR CA/DC less than 5 MHz and made the following observations and proposals.

Observation 1	The PBCH Puncturing impacts only case 3b and case 4.

Observation 2	RAN4 RF discussed and conclude to not introduce new network signalling on PRB transmission.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	RAN4 to support option 1, provided that no additional effort is needed and WID should be revised and approved.

Proposal 2	RAN4 should set contiguous and non-contiguous intra-band CA as a second priority, after completing the inter-band work, if time permits, we can specify contiguous and non-contiguous intra-band CA.

Proposal 3	For case 3b, SSB index reading is needed. It can be performed during the fine timing acquisition phase as agreed in Rel-18.

Proposal 4	For case 4a and 4b, SSB index reading is needed during the cell search phase as UE need to perform L1-RSRP.

Proposal 5	RAN4 to decide and limit the scope of the WI to Single SCell considering the time constraint.

Proposal 6	RAN4 to capture the following text for PSCell activation delay for NR-DC.

T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 7	RAN4 to capture the following text for Conditional PSCell activation delay for NR-DC.

T∆ = Trs for both known and unknown target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 8	The HO requirements with PSCell-PCell and PSCell-PSCell can be reused for NR CA/DC in less than 5MHz.

Proposal 9	RAN4 should define conditional handover requirements to support NR CA/DC for less than 5 MHz.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where,

T∆ = Trs for target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

Proposal 10	RAN4 to capture the following text for NR Conditional Handover.

T∆ is time for fine time tracking and acquiring full timing information of the target cell. Where,

T∆ = Trs for target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

Proposal 11	SSB time index detection for inter-frequency CA is extended by 4 samples.

Proposal 12	RAN4 to capture the following text for SCG activation delay.

T∆ = Trs for target cells operating with 20 PRB SSB BW.

T∆ = 3*Trs for target cells operating with 12 PRB SSB BW.

Proposal 13	RAN4 RRM should not introduce new network signalling on PRB transmission in alignment with RF.

Proposal 14	RAN4 to discuss and study the impact of reduced PBCH BW of 12PRBs for 3MHz channel bandwidth on EMR measurement with SSB index reading.

Proposal 15	RAN4 to send LS to inform potential impacts to other WGs in the November meeting after the RRM requirements are finalized.

# References

- R4- 2413883, WF for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2, Intel

- RP- 241622 “Revised WID: NR channel BW less than 5MHz for FR1 Phase 2”, Intel

- 3GPP TS 38.133: "NR; Requirements for support of radio resource management"