3GPP TSG-RAN WG4 Meeting #112bis	R4-2416092

# Hefei, China, October 14th – 18th 2024

Source:	Ericsson

Title:	Simulation results for Rel-19 MMSE-IRC BS demodulation requirements

Agenda item:	6.16.3

Document for:	Information

# Introduction

In RAN4#112 meeting, companies agreed to reuse LTE signal/interference model and receiver model as baseline for Rel-19 NR MMSE-IRC BS requirement discussion [1]. In this contribution, the trial simulation results are delivered.

# Simulations

In this simulation, the DIP values in LTE MMSE-IRC BS interference profile [2] are converted to INR values based on the formula in [3]. The trial simulations for HPUE considered for HetNet deployment assuming INR1 is 7dB higher than INR1 from LTE profile.

2.1	Simulation assumptions

Table 2.1-1 Mapping from LTE DIP values to INR values

| Scenario | Interferer number | (DIP1, DIP2) [dB] | (INR1, INR2) [dB] |
| --- | --- | --- | --- |
| HomNet | 1 interferer | (-1.11, NA) | (5.36, NA) |
|  | 2 interferers | (-1.11, -10.91) | (7.29, -2.5) |
| HetNet | 1 interferer | (-0.43, NA) | (9.83, NA) |
|  | 2 interferers | (-0.43, -13.78) | (12.38, -0.97) |

Table 2.1-2: Test parameters

| Parameter |  |  |  | Value |
| --- | --- | --- | --- | --- |
| Transform precoding |  |  |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  |  |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ |  | Maximum number of HARQ transmissions |  | 4 |
|  |  | RV sequence |  | 0, 2, 3, 1 |
| DM-RS |  | DM-RS configuration type |  | 1 |
|  |  | DM-RS duration |  | single-symbol DM-RS |
|  |  | Additional DM-RS position |  | pos1 |
|  |  | Number of DM-RS CDM group(s) without data |  | 2 |
|  |  | Ratio of PUSCH EPRE to DM-RS EPRE |  | -3 dB |
|  |  | DM-RS port(s) |  | {0} |
|  |  | DM-RS sequence generation |  | NID0=0, nSCID =0 |
| Time domain resource assignment |  | PUSCH mapping type |  | A |
|  |  | Start symbol |  | 0 |
|  |  | Allocation length |  | 14 |
| Frequency domain resource assignment |  | RB assignment |  | Full applicable test bandwidth |
|  |  | Frequency hopping |  | Disabled |
| TPMI index for 2Tx two layer or 4Tx four layer spatial multiplexing transmission |  |  |  | 0 |
| Code block group based PUSCH transmission |  |  |  | Disabled |
| Interference (Note 2) | Set 1 (HomNet) | 1 interferer | INR1 (dB) | 5.36 |
|  |  | 2 interferers | INR1 (dB) | 7.29 |
|  |  |  | INR2 (dB) | -2.5 |
|  | Set 2 (HetNet) | 1 interferer | INR1 (dB) | 9.83, HPUE: 16.83 |
|  |  | 2 interferers | INR1 (dB) | 12.38, HPUE: 19.38 |
|  |  |  | INR2 (dB) | -0.97 |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL patterns. NOTE 2: The receiver model and the interference model are defined in Annex A. |  |  |  |  |

2.2	Simulation results

Table 2.2-1: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.19 | 2.53 |
|  |  | TDLB100-100 |  | 5.36 | 2 | -1.77 | 0.02 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 5.73 | 11.13 |
|  |  | TDLB100-100 |  | 5.36 | 16 | 6.39 | 9.02 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.56 | 10.6 |
|  |  | TDLB100-100 |  | 5.36 | 20 | 6.08 | 8.43 |

Table 2.2-2: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 2 Interferers

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1.17 | 5.09 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 2 | -0.01 | 1.93 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 7.55 | 13.65 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 16 | 7.84 | 10.97 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 6.67 | 13.17 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 20 | 7.0 | 10.29 |

Table 2.2-3: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -0.83 | 2.55 |
|  |  | TDLB100-100 |  | 5.36 | 2 | -1.56 | 0.05 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 6.24 | 11.39 |
|  |  | TDLB100-100 |  | 5.36 | 16 | 6.54 | 9.13 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.75 | 10.61 |
|  |  | TDLB100-100 |  | 5.36 | 20 | 6.11 | 8.43 |

Table 2.2-4: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 10 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 2 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1.60 | 5.06 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 2 | 0.27 | 2.02 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 8.13 | 13.94 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 16 | 8.0 | 11.07 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 7.0 | 13.14 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 20 | 7.06 | 10.34 |

Table 2.2-5: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 50 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.47 | 2.05 |
|  |  | TDLB100-100 |  | 5.36 | 2 | -2.31 | -0.53 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 6.3 | 11.57 |
|  |  | TDLB100-100 |  | 5.36 | 16 | 6.44 | 9.11 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.86 | 11.05 |
|  |  | TDLB100-100 |  | 5.36 | 20 | 6.23 | 8.67 |

Table 2.2-6: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 50 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 2 Interferers

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 0.9 | 4.59 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 2 | -0.54 | 1.38 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 8.27 | 14.13 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 16 | 7.95 | 11.02 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 7.08 | 13.56 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 20 | 7.23 | 10.55 |

Table 2.2-7: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 100 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.48 | 1.95 |
|  |  | TDLB100-100 |  | 5.36 | 2 | -2.37 | -0.59 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 6.39 | 11.45 |
|  |  | TDLB100-100 |  | 5.36 | 16 | 6.33 | 8.89 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 6.26 | 11.08 |
|  |  | TDLB100-100 |  | 5.36 | 20 | 6.32 | 8.64 |

Table 2.2-8: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 100 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 2 Interferers

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 0.93 | 4.48 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 2 | -0.60 | 1.31 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 12 | 8.35 | 14.0 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 12 | 7.78 | 10.8 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 7.47 | 13.57 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 20 | 7.29 | 10.54 |

Table 2.2-9: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 20 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.68 | 2.04 |
|  |  | TDLB100-100 |  | 5.36 | 2 | -2.02 | -0.26 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 6.23 | 11.56 |
|  |  | TDLB100-100 |  | 5.36 | 16 | 6.38 | 9.05 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.59 | 10.68 |
|  |  | TDLB100-100 |  | 5.36 | 20 | 6.0 | 8.35 |

Table 2.2-10: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 20 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 2 Interferers

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 0.66 | 4.57 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 2 | -0.27 | 1.64 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 8.19 | 14.11 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 16 | 7.8 | 10.95 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 6.76 | 13.22 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 20 | 7.02 | 10.26 |

Table 2.2-11: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 40 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 1 Interferer

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 9.83 | 2 | -1.32 | 2.15 |
|  |  | TDLB100-100 |  | 5.36 | 2 | -2.32 | -0.53 |
|  | 4 | TDLA30-10 | TDLC300-100 | 9.83 | 16 | 6.14 | 11.33 |
|  |  | TDLB100-100 |  | 5.36 | 16 | 6.34 | 8.93 |
|  | 8 | TDLA30-10 | TDLC300-100 | 9.83 | 20 | 5.59 | 10.43 |
|  |  | TDLB100-100 |  | 5.36 | 20 | 5.96 | 8.24 |

Table 2.2-12: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 40 MHz channel bandwidth, 30 kHz SCS with Transform Precoding Disabled with 2 Interferers

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 2 | 1.08 | 4.70 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 2 | -0.54 | 1.37 |
|  | 4 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 16 | 8.04 | 13.9 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 16 | 7.74 | 10.82 |
|  | 8 | TDLA30-10 | TDLC300-100 | (12.38, -0.97) | 20 | 6.83 | 12.97 |
|  |  | TDLB100-100 |  | (7.29, -2.5) | 20 | 6.91 | 10.14 |

Table 2.2-13: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 1 Interferer for HPUE

| TX | RX | Propagation conditions and correlation matrix |  | INR1 [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | 16.83 | 2 | 1.84 | 8.5 |
|  | 4 | TDLA30-10 | TDLC300-100 | 16.83 | 16 | 7.68 | 17.16 |
|  | 8 | TDLA30-10 | TDLC300-100 | 16.83 | 20 | 6.54 | 16.38 |

Table 2.2-14: Minimum requirements for PUSCH with 70% of maximum throughput, Type A, 1 Transmission Layer, 5 MHz channel bandwidth, 15 kHz SCS with Transform Precoding Disabled with 2 Interferers for HPUE

| TX | RX | Propagation conditions and correlation matrix |  | (INR1, INR2) [dB] | MCS In Table 1 | SNR [dB] |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Tested signal | Interferer 1&2 |  |  | IRC | MRC |
| 1 | 2 | TDLA30-10 | TDLC300-100 | (19.38, -0.97) | 2 | 4.37 | 11.0 |
|  | 4 | TDLA30-10 | TDLC300-100 | (19.38, -0.97) | 16 | 9.88 | 19.69 |
|  | 8 | TDLA30-10 | TDLC300-100 | (19.38, -0.97) | 20 | 7.85 | 18.94 |

# Conclusions

The simulation results for NR MMSE-IRC PUSCH are delivered.

# References

[1] 	R4-2413570, ‘WF on NR demodulation performance: Phase 5’, China Telecomm

[2]	36.104 v13.0.0

[3]	R4-2416091, Discussion on NR MMSE-IRC PUSCH requirements, Ericsson