3GPP TSG-RAN WG4 Meeting #112bis	R4-2416093

# Hefei, China, October 14th – 18th 2024

Source:	Ericsson

Title:	Discussion on general issue of Rel-19 SCM for demodulation

Agenda item:	6.14.1

Document for:	Discussion

s

# Introduction

In RAN4#112 meeting, companies discussed some detailed limitations in current TDL model and corresponding demodulation tests, and also raised many questions on potential approaches for spatial channel model. The agreed WF captured agreements on work plan, skeleton of TR and candidate options for future discussions.

The agreed TR skeleton is:

Content of Technical Report

Agreement:

RAN4 can use the following as a starting point for the TR skeleton structure.

- Deployment Scenarios

  - Identification of limitation of current channel models, and impact to UE processing

- Spatial Channel Modelling Approaches

  - Feasibility of CDL models in TR38.827

    - Advantages and disadvantages

  - Feasibility of extended TDL models

    - Advantages and disadvantages

- Test Setup and considerations

  - Choose set of models from modified CDL models and/or TDL models

    - Results Collection

  - Investigate potential test cases based on the consensus of SCM methodology

  - Investigate the complexity of conformance tests.

- Feasibility of introducing RAN4 performance requirements (stability, repeatability, alignability)

- Document Conclusions

In this contribution, some unclear parts in agreement and detailed content of TR will be furtherly discussed.

# Discussion

In agreed WF, it mentioned that “Choose set of models from modified CDL models and/or TDL models for result collection” after comparing CDL and TDL extension approaches. There is an issue that the channel model profile should be agreed before link level simulations, but there is no existing TDL extension model profile available and CDL model profiles in 38.827 might need some modifications. According to agreed work plan, it is a too tight schedule to evaluate new channel model profile (especially for TDL extension approach, i.e., with per-tap correlation matrix) and do calibration among companies. In that case, some expected simulations for TDL extension might not happen eventually.

For CDL models in 38.827, it is also hard to tell in advance which model is very suitable for target test case (i.e., 8T8R) unless companies have some simulation results. But there are many model profiles in 38.827 and also the profile might need modifications (i.e., delay spread or Doppler shift), corresponding simulation effort would also be huge. If there are modifications for existing CDL model profile in 38.827, the corresponding reason and methodology of modification should be clarified in TR.

It would be better to agree CDL or TDL profile after feasibility analysis and run simulation campaign to check agreed profile. It might need several rounds to have stable profiles and simulation assumptions. In that case, whole effort needed for this SI would be much higher than original expectation.

If we could focus on CDL model extension (i.e., 38.827 models) at the first and could agree a set of models finally, then it could be easier to have corresponding TDL models and correlation matrix based on agreed CDL models. Then RAN4 could continue to compare the performance and complexity between CDL and TDL models by then if there is time left.

There is no available TDL extension model profile and corresponding simulations might not happen due to tight work plan.

The simulation effort for model comparison, selection and potential modifications would be very high even only focus on CDL models in 38.827.

It could be easier to have TDL models and correlation matrices once RAN4 have agreed CDL models.

Regarding the effort mentioned above, RAN4 discussion might need to concentrate on one approach such as CDL models in 38.827 at the first due to available channel model profile. As for TDL extension approach, it could start after companies have agreement on CDL approach.

Proposal 1	RAN4 discussion focus on CDL approach in 38.827 at the first, and then continue with TDL extension approach investigation after companies have agreements on CDL approach.

As for TR content, it would be better to add sections to capture agreed channel model profile before link level simulations. One more section might be needed for CDL extension if CDL model in 38.827 need any modification. TDL extension (i.e., with per-tap correlation matrix) might also need more detailed description on channel coefficients and correlation matrix derivation regarding no available models now.

The test case selection could be agreed prior to the link level simulations. The simulation can be done only when the simulation purpose is clear. And for conformance test complexity, at least two items could be considered. One is test instrument implementation including limitation of cluster/tap number, profile size (i.e., impacted by correlation matrix size) etc. The other is test setup description to identify the procedure of conducted test and virtual cable test mentioned in SID.

In summary, we propose to modify the TR skeleton as following (new added item is marked with red; switching order item is marked with yellow):

- Deployment Scenarios

  - Identification of limitation of current channel models, and impact to UE processing

- Spatial Channel Modelling Approaches

  - Feasibility of CDL models in TR38.827

    - Advantages and disadvantages

    - Modifications if possible

    - Agreed CDL model profiles

  - Feasibility of extended TDL models

    - Advantages and disadvantages

    - Evaluation procedure description

    - Agreed TDL model profiles

- Test Setup and considerations

  - Investigate potential test cases based on the consensus of SCM methodology

  - Choose set of models from agreed CDL models and/or TDL models

    - Results Collection

  - Investigate the complexity of conformance tests.

    - Test instrument implementation

    - Test setup description

- Feasibility of introducing RAN4 performance requirements (stability, repeatability, alignability)

- Document Conclusions

Proposal 2	Take modification on TR skeleton as following:

- Deployment Scenarios

  - Identification of limitation of current channel models, and impact to UE processing

- Spatial Channel Modelling Approaches

  - Feasibility of CDL models in TR38.827

    - Advantages and disadvantages

    - Modifications if possible

    - Agreed CDL model profiles

  - Feasibility of extended TDL models

    - Advantages and disadvantages

    - Evaluation procedure description

    - Agreed TDL model profiles

- Test Setup and considerations

  - Investigate potential test cases based on the consensus of SCM methodology

  - Choose set of models from agreed CDL models and/or TDL models

    - Results Collection

  - Investigate the complexity of conformance tests.

    - Test instrument implementation

    - Test setup description

- Feasibility of introducing RAN4 performance requirements (stability, repeatability, alignability)

- Document Conclusions

# Conclusions

In the previous sections we made the following observations:

Observation 1	There is no available TDL extension model profile and corresponding simulations might not happen due to tight work plan.

Observation 2	The simulation effort for model comparison, selection and potential modifications would be very high even only focus on CDL models in 38.827.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 discussion concentrate on one approach such CDL models in 38.827 or reconsider the time plan for this SI.

Proposal 2	Take modification on TR skeleton as following:

- Deployment Scenarios

  - Identification of limitation of current channel models, and impact to UE processing

- Spatial Channel Modelling Approaches

  - Feasibility of CDL models in TR38.827

    - Advantages and disadvantages

    - Modifications if possible

    - Agreed CDL model profiles

  - Feasibility of extended TDL models

    - Advantages and disadvantages

    - Evaluation procedure description

    - Agreed TDL model profiles

- Test Setup and considerations

  - Investigate potential test cases based on the consensus of SCM methodology

  - Choose set of models from agreed CDL models and/or TDL models

    - Results Collection

  - Investigate the complexity of conformance tests.

    - Test instrument implementation

    - Test setup description

- Feasibility of introducing RAN4 performance requirements (stability, repeatability, alignability)

- Document Conclusions

# References

[1] 	R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, Nokia

[2]