3GPP TSG-RAN WG4 Meeting #112bis	R4-2416094

# Hefei, China, October 14th – 18th 2024

Source:	Ericsson

Title:	Discussion on methodology of NR SCM for demodulation

Agenda item:	6.14.2

Document for:	Discussion

# Introduction

In RAN4#112 meeting, companies discussed some detailed limitations in current TDL model and corresponding demodulation tests, and also raised many questions on potential approaches for spatial channel model. The agreed WF captured agreements on work plan, skeleton of TR and candidate options for future discussions.

In this contribution, the open issues, such as comparison among existing TDL model, TDL extension and CDL extension, how to choose and configure test cases, conformance test limitations, etc., will be furtherly discussed.

# Discussion

## 2.1	General

In this part, we would like to discuss the high-level methodology of SCM implementation based on test method proposed in WID. During the last meeting, it seems companies don’t have a common understanding on this part, and the discussions are scattering and trapped in some detailed configurations. It would be worthy to get align on the whole methodology at the first step.

Generally, the whole link level channel in RAN1 and RAN4 demodulation perspective could also be called as “radio channel” which include Tx/Rx antenna and the directional propagation channel, see figure 2.1-1 below.

Figure 2.1-1 Radio channel and directional propagation channel

The directional propagation channel is basically defined by propagation path and the corresponding channel properties include delay dispersion, angular dispersion and polarization cross-scattering (XPR). Radio channel together with multipath fading impact also include delay spread, Doppler dispersion/spectrum, spatial frequencies and angular spread. The table 2.1-1 summarize the multipath channel relations [3].

Table 2.1-1 Multipath channel properties and impact

| Channel properties | Domain impact |
| --- | --- |
| Frequency domain |  |
| Time delay | Baseband frequency |
| Time dispersion | Frequency-selective fading |
| Channel impulse response | Channel transfer function |
| Delay spread | Coherent bandwidth |
| Time domain |  |
| Doppler frequency | Time |
| Doppler dispersion | Time-variant fading |
| Doppler spectrum/spread | Coherent time |
| Spatial domain |  |
| Spatial frequencies, “beam space” | Spatial positions |
| Angular dispersion | Spatial fading |
| Angular spectrum | Coherent length |
| Angular spread | Antenna correlation |

In SID RP-241610, there are two methods are mentioned for conformance test, conducted test and OTA with virtual cable test. However, the test setup should be the similar which include BS emulator, channel emulator and DUT, see figure 2.1-2 below. For FR1 conducted test, it is same as current test setup. For FR1/FR2 OTA test, the difference is the cable between the test equipment and BS receiver in conducted is replaced by an anechoic chamber with a test antenna. Both Tx and Rx RF antenna ports impact is captured in the channel emulator.

Figure 2.1-2 Test setup for FR1 DL conducted test

The purpose of this kind of test setup is split demodulation performance test from RF port impact which would impact the whole link level channel according to antenna element locations. It could be considered as a “black box” as some companies proposed. It could be compared with figure 2.1-1 for link level channel description.

It is possible to consider whole radio channel as a black box for a link between Tx and Rx.

The antenna configuration is decided by vendors according to their own technical considerations. It can’t be same between all vendors. In that case, the channel models in real network would be very much different and can’t be estimated. From demodulation chip perspective, its implemented algorithm should be independent of a certain antenna configuration. In 3GPP, what we have done is just use some typical configuration assumptions we believe to create channel model (i.e., existing TDL and ULA low correlation matrix) and doing the test. To DUT, the channel model actually is always treated as a “black box”. To apply spatial channel model is to have more practical model which can be used to check the blind points in legacy MIMO tests.  It is possible to take the “black box” concept for spatial channel model either. The performance will be tested under a specific channel condition which is designed for a specific test purpose (i.e., per layer performance impact, PMI accuracy etc.) with proper assumption for whole link. The demodulation branches would be directly fed with received signal which is expected before receiver equalization.

From demodulation perspective, algorithm performance could be independent from a specific antenna configuration at a certain level.

Figure 2.1-3 CDL model in 38.827 and “black box” SCM model

In 38.827, the CDL models are created based on assumptions per deployment scenario (i.e., Uma, Umi, etc.) including frequency, BS antenna configurations, desired angular directions, spread and UE speed etc. Since the 38.827 is aiming to UE OTA RF tests, it doesn’t use virtual cable setup but measure the UE antenna radiation pattern at the first place. If we use CDL models in 38.827 for demodulation, a typical UE antenna radiation pattern could be calculated and directly applied to CDL channel models. Of course, the channel model coefficients would be changed in that case.

Proposal 1 	Take spatial channel model including both BS and UE antenna configuration as a “black box” for demodulation tests. The corresponding configurations should be decided based on typical test case scenario.

Now the question is how to choose proper configurations for a certain spatial channel model? We believe there are at least three key issues should be prioritized, availability, test expectation, typical deployment and testability.

The availability means the channel model profile is existing in specification or not. It will impact the investigation effort and time schedule of discussion.

The test expectation mentioned here is what we are expecting from spatial channel model, such as per layer SNR difference, obvious spatial fading related to an angular distribution etc. As companies mentioned in the previous meeting, big difference of per layer SNR is seen in real network and the performance is expected to be verified by operators. And a slow variant angular distribution is also expected for a high layer transmission/reception (i.e., low UE speed). These aspects should be considered for a channel model selection.

The deployment would decide product configurations and cell features, and then impact the expected channel characteristics, such as the antenna configuration could impact on channel correlation, cell range could impact delay spread and UE speed could impact Doppler spread etc. For high layer MIMO performance, the typical deployment could be dense urban scenario with plentiful paths for spatial multiplexing. In that case, large BS and UE antenna might be useful to have feasible SINR. Furthermore, relatively small delay spread and low Doppler shift are also typical for high layer transition.

The testability would decide the test setup, target SNR and cluster angular distribution etc., and then also could impact the channel model characteristics, such as cluster number and channel profile size etc. TE vendors could give input on the feasible cluster number and target SNR value supported by instruments. If profile size is very large, for example per tap TDL correlation matrix for large antenna, the channel file implementation and storage might be a problem and corresponding specification could also be very tricky.

In summary, to have a good channel model for a specific test case, we need to consider at the least these four aspects jointly and then decide the assumptions for channel coefficients evaluation. Otherwise, it is hard to say we have a feasible channel model to check expected feature performance under a real network like scenario.

Proposal 2	Jointly consider at least availability, test expectation, deployment and testability for channel model evaluation and decision.

## 2.2	Existing TDL model, CDL extension and TDL extension Comparison

In this section, we would compare the three aspects mentioned above among existing TDL channel models, CDL extension models in 38.827, TDL extension models in [4] and TDL extension method in R4-2412328 [5]. The comparison is based on the proposed 8T8R test case in WF [1].

### 2.2.1	Existing TDL model

The existing TDL models in 38.901 are generated from CDL models (see channel coefficient formula below) assuming ideal isotropic antenna at both Tx and Rx. In that case, its spatial filtering is constant in all directions and can’t reflect the antenna radiated beam properties in the real network. Normally, BS antenna element is directional and could give different radiated gain along different directions.

The existing correlation matrices in 38.101-4 are defined for following combinations:

- ULA correlation:

  - Both gNB and UE are ULA with 1, 2, 4 and 8 antennas.

  - Low correlation (α=β=0): 1x2, 1x4, 2x1, 2x2, 2x4, 4x1, 4x2, 4x4

  - Medium correlation (α=0.3, β=0.9): 1x2, 2x1, 2x2, 2x4, 4x2, 4x4

  - Medium correlation A (α=0.3, β=0.3874): 1x4, 2x4, 4x4,

  - Medium correlation B (α=0.3, β=0.005154): 2x8

  - High correlation (α=β=0.9): 1x2, 2x1, 2x2, 4x1, 4x2, 4x4,

  - Most of PDSCH demodulation requirements are defined with ULA low correlation matrix. The Rel-18 8Rx PDSCH requirements are defined by 2x8 ULA medium B correlation, 4x8 and 8x8 ULA low correlation.

- X-pol correlation: only medium and high correlation are defined.

  - 1D X-pol antenna array at gNB side, with 2, 4, 8 antenna arrays.

  - 2D X-pol antenna array at gNB side with 16 antenna arrays.

  - 1D X-pol antenna array at UE side with 1, 2 and 4 antenna elements.

  - Medium correlation (α1=α2=0.3, β=0.6, γ=0.2): 2x2

  - High correlation (α1=α2=0.9, β=0.9, γ=0.3): 4x2, 2x4, 4x4, 8x2, 16x2

  - Only some CSI reporting requirements are defined with X-pol correlation.

In principle, a proper defined correlation matrix could introduce different propagation gain along different directions. But low correlation matrices have zero cross-correlation between antenna arrays and will lead to same superposition results in all directions. As many companies pointing out, the existing 8Rx with 8 layers requirements are defined by existing TDL model and ULA low correlation matrix and show same average SNR for all layers. In our simulations [3], small difference can be seen between two CWs for 8 layers transmission with TDLC300-100 ULA low if we assume non-isotropic antenna elements.

Figure 2.2-1 8Tx8Rx PDSCH with TDL model and non-isotropic BS antenna element assumption

On the other hand, the correlation value in existing correlation matrices might be too high between adjacent antenna arrays which would lead to very bad spatial differentiation. Then they are hard to be used for high layer transmission requirement definition due to bad performance achieved. Antenna correlation value is basically aligned with the zero-rank Baser function of the 1st kind if we assume PAS (probability of angular spread) of an impinging signal is Laplacian distribution [7]. In that case, the correlation value could be high is angular spread is small even the antenna separation is large, otherwise the correlation decreases quickly with the increasing of separation, see figure below [6]. For a CDL model, per-cluster angular spread is defined, and per-tap correlation matrix for TDL model might be more precise.

Figure 2.2-2 Correlation vs antenna separation respective to angular spread

For test expectation consideration, the spatial characteristic in TDL model shows low angle preference along time and frequency domain if no proper spatial filter and correlation matrix are applied. In that case, it might not be suitable for high layer transmission performance verification dependent on channel angular distribution.

Existing correlation matrix seems not suitable for high layer transmission performance verification.

For deployment considerations, different delay spread values and Doppler shifts are considered with existing TDL model. To simplify the specification edition, limited TDL model combined with fixed delay spread is defined for demodulation requirements, such as TDLA10, TDLB100, TDLC300. The Doppler spectrum use Jakes model and applied for all taps.

As for testability consideration, the number of taps in existing TDL model are combined to reduce complexity if the relative delay is same. The tap resolution of test instrument is also considered during model profile evaluation. With beam steering method, the PMI test under time variant arrival angle directions can be achieved.

### 2.2.2	CDL extension model in 38.827

Companies have delivered many observations for 38.827 in the last RAN4 meeting. In this section, we would revisit this method based on three consideration aspects.

For test expectation consideration, CDL models in 38.827 could deliver different SNR per layer and relatively high time/frequency spatial fading consistency according to simulation results. The comparison between TDL model, CDL model in 38.900 and CDL model in 38.827 is showed below. FR1 CDLC Uma model in 38.827 shows smaller performance difference between CW than CDLC model in 38.900. Both show very bad performance at 70% normalized throughput.

Figure 2.2.2-3 TDLC300-100 low vs CDLC in 38.900 vs FR1 CDLC Uma in 38.827

Furthermore, changing the desired angle directions (i.e., μϕ,desired in formula 7.7-5 in 38.901) or adding additional beam steering weights could support multiple channel direction tests. Of course, the channel model profile will be different.

It is possible to support time variant beam directions for CDL model by changing desired angle directions or additional beam steering.

For deployment consideration, 38.827 models take typical angular spread and antenna configurations per deployment such as Uma, Umi, etc. Thus, the channel coefficients are more practical and closer to real network. But it should be pointed out that, all ZoA are fixed to 90o and ZoD are centralized, and cluster ZSA is fixed to 0 would lead to very high correlation in zenith direction at receiver side. This is an artificial configuration for OTA test. Corresponding transmission beams are also directed to azimuth plane along the strongest clusters in channel model. From demodulation test setup described in SID, this processing seems not so necessary. Another point is about the Doppler shift value configured in 38.827 that fixed UE speed 30km/h for FR1, 3km/h for indoor and 12km/h for FR2. For some test cases, such as 8 layers in Uma, low UE speed and plentiful multipaths are necessary, so 30km/h UE speed would not be suitable. Thus, some modifications might be needed. Furthermore, the Doppler spectrum is different among clusters due to different arrival angles.

Fixed angular spread and typical antenna configuration is applied.

Artificial ZoA and ZoD is used for OTA test purpose.

Relative high UE speed is assumed for FRI CDL models.

Doppler spectrum is per cluster.

For testability consideration, the number of clusters might be high, but the 38.827 has concluded that at least 24 clusters should be feasible for TE implementation.

The model profile in 38.827 seems feasible for test instrument implementation.

### 2.2.3 	TDL model + per tap correlation matrix

There is no available TDL model and corresponding per-tap correlation matrix in existing specifications. If this approach is investigated in this study, the channel coefficients should be evaluated at the first.

In [4], the general procedure of applying per-tap correlation matrix to LTE TDL model is described. The basic thinking is to calculate correlation values between antenna arrays based on proper assumptions on antenna element radiation pattern, array separation, cluster angular spread and XPR under a certain deployment scenario. The angular spread information could be gotten from selected CDL model. The calculation of correlation value could refer to [7].

To compare the performance and complexity, CDL models in 38.827 can be considered for start point. According to methodology described in chapter 7.7.4.2 and 7.7.5.1 of 38.901, following general procedures could be used:

- Scaling the angles: use angle scalar values in 38.827.

- Choose spatial filters for Tx and Rx: use BS antenna element radiation pattern assumptions in 38.827. Define UE antenna element radiation pattern according to test case and deployment.

- Define pointing direction.

  - Based on the dominant cluster direction: according to CDL models in 38.827.

  - Based on arbitrary direction: according to expecting test setup.

- Calculate TDL tap power values by CDL cluster power and spatial filter.

- Calculate polarization correlation matrix Γ: Use XPR values in 38.827.

- Calculate BS and UE side correlation matrix RBS and RUE:

  - Define antenna array: BS array use assumptions in 38.827. Define new UE array according to test case and deployment.

  - Calculate per-tap correlation matrix according to original CDL cluster departure or arrival angle value and array locations (mapping in LCS.

- Calculate per-tap total correlation matrix by RBS⨂Γ⨂RUE

- Calculate Doppler spectrum.

  - Option 1 is using one Jakes Doppler spectrum for all taps as existing TDL model.

  - Option 2 is using per-tap Doppler spectrum based on cluster directions.

Note that, it is proved that an equivalent per channel spatial correlation matrix can be derived from per-path spatial correlation matrix (with the pre-condition that channel responses between paths and between subcarriers are independent, ) to simplify the channel model [8]. But the specified conversion method needs further evaluation.

It could be straight forward to have TDL with per-tap correlation model according to a CDL model.

It could be possible to have equivalent per channel correlation matrix by per-tap correlation matrix.

For test expectation consideration, we could expect obvious spatial fading distribution from a TDL model with proper per-tap correlation matrix. In that case, we can assume this kind of model can fulfil the expectation on obvious different SNR per layer. For some special test cases, such as PMI case, time variant beam directions could also be achieved if we change the desired mean angular value in the first step above. But it would lead to multiple TDL channel model profiles.

For the deployment consideration, this kind of model could be defined by realistic assumptions from 38.901 or 38.827 regarding to typical deployment.

As for testability consideration, the main issue might be the correlation matrix size and corresponding implementation.

- The first thing is the number of taps. If the CDL models in 38.827 could use up to 24 clusters and TE vendors don’t have any problem on it, we can assume TDL models could also have up to 24 taps. But still, it could be possible to consider reduce tap number by combining taps with same delay spread under a proper method.

- The second thing is the size of correlation matrix if per-tap correlation is applied. Assuming the BS antenna is planner antenna with M antenna arrays and UE antenna with N arrays, then the number of correlation coefficients in one matrix could be M^2 x N^2. By some optimization method such as only store unique coefficients, the size could be reduced to 2xMxN-M-N+1. If the total number of taps is L, the total number of correlation coefficients could be Lx(2xMxN-M-N+1). For example, the BS antenna have 32 arrays and UE antenna have 4 arrays, and tap number is 24, then there are at least (2x32x4-32-4+1)*24= 5304 coefficients. Equivalent per-channel correlation matrix might help to reduce the complexity and make this kind of model practical for conformance tests.

- Another issue is about the per-tap Doppler spectrum is used if we want to compare the simulation results between CDL model and TDL model. The converging simulation time could be very long and impractical, such as 100s or longer. Regarding all existing TDL models use Jakes spectrum for all taps, the performance difference from per-tap Doppler spectrum might be acceptable. We could follow current approach to use one spectrum for all taps, especially for low UE speed scenario.

Per-tap correlation matrix size could be very large due to tap number and antenna array size.

It would need a very long time to get converging results for Doppler impact comparison between CDL model and per-tap Doppler spectrum.

### 2.2.4	Existing TDL model + extended beam steering method

In R4-2412328, a method is proposed to add Rx beam steering weight on top of existing BS beam steering weight, TDL model and correlation matrix to have separatable arrival angle of beams at the UE receiver side.

For the test expectation consideration, it is a simple way to achieve different SNR per layer by proper beam steering and power split. It can also realize the time varying beam direction configuration. But one limitation could be the low spatial angle preference along frequency domain inheriting from existing TDL model. One issue mentioned in the contribution is that the TDL channel rank can’t be increased by only Tx beam steering method. But it could be achieved by changing correlation matrix. As for the performance showed by this kind of channel, more results are needed to check if it can align the observation in real network.

From deployment consideration, this method can be considered as more artificial than other methods. The steered beam directions have exactly identical separations which can’t happen in real network. For study purpose, same strong beam directions as 38.827 could be used as reference for further comparison. But it could be still a question when configure angle and power for a new case.  Other deployment relevant channel properties could be same as existing TDL models.

As for testability consideration, this method could be similar as existing TDL models with additional implementation on Rx beam steering weight impact.

For this method, another question might be how to decide the beam steering direction and corresponding power split? For direction, one possible way might follow similar way used in 38.827 aiming same ZoA for all strong beams.

It is simple and artificial way to have different arrival angles by extended beam steering method.

It is not clear about the principle of configuring beam directions and corresponding power.

### 2.2.5	Summary

Here we summarize main points from analysis on all methods above and give recommendation for the study.

Table 2.2.1-1 Summary of channel properties comparison

| Channel properties | Existing CDL models | Existing TDL models | CDL models in 38.827 | TDL extension by per-tap correlation | Existing TDL model + extend beam steering |
| --- | --- | --- | --- | --- | --- |
| Availability | Available | Available | Available | Not Available | Not Available |
| Test expectation | Show SNR difference between layers. Medium and long spatial time and frequency domain preference. Fixed beam direction. | No SNR difference between layers. Short spatial time and frequency domain preference. Fixed beam directions. Beam directions could be time variant by steering weight. | Show SNR difference between layers. Medium and long spatial time and frequency domain preference. Fixed beam directions. Beam directions could be time variant by setting desired direction. | Show SNR difference between layers. Medium and long spatial time and frequency domain preference. Beam directions could be time variant. | Show SNR difference between layers. Short spatial time and frequency domain preference. Fixed beam directions. Beam directions could be time variant. |
| Deployment | Ideal antenna configuration. No specific delay spread is used. No specific doppler spectrum is used. | Ideal spatial filter assumption. High correlation per channel between adjacent arrays. Fixed delay spread is used. Fixed Doppler spectrum is applied for all taps. | Practical spatial filter assumption. Practical but fixed BS antenna array assumption. Typical angular spread is applied. Artificial ZoD and ZoA is used. Fixed delay spread is applied per scenario. | Practical spatial filter assumption. Per-tap correlation is applied based on practical antenna array assumption. Typical angular spread is applied. Fixed delay spread is applied per scenario. Possible using fixed Doppler spectrum for all taps. | Ideal spatial filter assumption. High correlation per channel between adjacent arrays. Fixed delay spread is used. Fixed Doppler spectrum model is applied for all taps. Artificial beam directions and power. |
| Testability | Not feasible due to many random parameters. | Feasible setup. Small channel model profile size. Simple beam steering function by artificial weight. | Feasible setup. Medium channel model profile size. Relatively complicated beam steering method by changing desired direction. | Feasible setup. Large channel model profile size. Simple beam steering function by artificial weight. | Feasible setup. Medium channel model profile size. Simple beam steering function by artificial weight. |
| Recommendation | Baseline for comparison. | Baseline for comparison. | Prioritize for study. | Pending until CDL extension method has conclusion. | Pending until CDL extension method has conclusion. |

Proposal 3	Prioritize study on CDL model extension method. Pending TDL approaches until CDL method has conclusion.

## 2.3	Test case selection and configurations

As we proposed in another contribution [2], the CDL extension approach could be prioritized for the study to avoid too extensive discussions on different approaches. In that case, we only discuss the suitable test cases and configurations for CDL approach.

### 2.3.1 Test cases

Test Cases for SU-MIMO

Way Forward:

As a starting point:

- 8Rx:

  - 8 Layer (8T8R)

    - 8 Layer PDSCH TPUT (Options on precoder choice to be presented at next meeting)

    - FFS: 8 Layer PMI Reporting (CB Type 1, single panel)

  - FFS: 4 Layer (4T8R)

    - FFS: 4 Layer PDSCH TPUT

    - FFS: 4 Layer PMI Reporting (CB Type 1, single panel)

- FFS: 4Rx:

  - FFS: 4 Layer (4T4R)

    - FFS: 4 Layer PDSCH TPUT

Interested companies to bring views at the next meeting (for comparison and initial alignment).

Test Cases for MU-MIMO

Way Forward:

Companies to bring proposals at the next meeting.

Some companies have concerns about the Mu-MIMO performance and would introduce separate test cases for it in this SI. Normally, Su-MIMO scenario is used to check the basic receiver demodulation algorithm performance (i.e., MMSE) while Mu-MIMO scenario is used to check the intra-cell inter-user interference cancelation performance (i.e., MMSE-IRC). Thus, the focus is IRC algorithm performance not channel model in Mu-MIMO tests. Furthermore, in Mu-MIMO test, only one UE as the served UE is tested. In that case, it would be no difference from Su-MIMO from the DUT point of view. As for channel model itself, the model used for Su-MIMO could also be applied for Mu-MIMO scenario. Regarding the analysis above, it seems no necessary to consider Mu-MIMO test case specially.

Proposal 4	Only consider Su-MIMO scenario for Rel-19 SCM study.

For the test cases in Su-MIMO, both 4Rx and 8Rx are discussed in the last meeting. In 38.827, it is indicated that how to use FR1 model for 4x4 OTA tests, such as how to choose the two strongest beams for transmission. In that case, it could be straightforward to check 4Tx4Rx with 4 layers for SU-MIMO at the first. But 8Rx with 8 layers could be a more interested case due to two codewords could be transmitted by quite different beams and achieve different SNR per layer. That’s why it is prioritized after discussion. If 8Tx8Rx case can achieve a feasible channel model for demodulation, it seems no much benefit to study 4Tx4Rx case. As for 4Tx8Rx, it seems also useful to check the test setup and corresponding channel model for it.

Proposal 5	Take 8Tx8Rx with 8 layers and 4Tx8Rx with 4 layers for the study as the first.

For PMI tests, TDL model + beam steering method is used in current requirements. If we go to spatial model such as CDL model, the channel profile modification and test complexity should be evaluated at the first step. Generally, the PMI test need to let transmission beam angle time variant to check the UE PMI feedback. Thus, the CDL model need timely variant by changing mean angle of departure and arrival angles. It might lead to multiple CDL model profiles and hard to be handled by a simple function like beam steering. In that case, PMI test discussion could be pended until the CDL model study for throughput test has its milestone.

Proposal 6	The PMI test case discussion could be pended until throughput test case have conclusion.

### 2.3.2 Configurations

For test configuration, here we propose some configurations for 8Tx8Rx PDSCH with existing TDL model and FR1 3.5GHz CDL model in 38.827 as start point.

UE Antenna Configuration

Agreement:

UE Antenna Configuration, for DL, for 38.827 : Cross Pol (1, n/2, 2), i.e. [ + + + +].

Where ‘n’ is number of Rx with half lambda spacing, with omni antenna element radiation pattern.

It was agreed to use X-pol UE antenna configuration for 8Rx in the previous meeting. In that case, it would be suitable to consider X-pol correlation matrix for TDL model rather than ULA correlation matrix regarding to comparison with CDL model. As the start point, medium X-pol correlation matrix could be considered. In our simulation, X-pol medium correlation shows better performance than ULA medium A (still can’t reach peak throughput for one CW by fixed PMI).

In normal PDSCH simulations, the BS antenna array needs to be configured and then decide the CSI codebook. There are 128 antenna elements used in FR1 3.5GHz CDL model coefficients evaluation. In that case, the model has its “inner BS antenna configuration” which independent of the antenna configuration used in simulator, say “outer antenna configuration”. For simulation, it could be possible to mapping outer antenna configuration to inner antenna configuration by virtual sub-array method. For example, in 8Tx8Rx case, there are 8 CSI ports in outer antenna configuration and 128 antenna elements in inner antenna configuration. By applying virtual sub-array for inner antenna to 4 sub-array per polarization (i.e., 4x1 type), simulator could map 8 CSI ports to 8 virtual sub-arrays for simulations.

The next issue is how select the strongest beam for transmission with CDL model. In 38.827, it is indicated that the beam direction of 1st and 2nd strongest beam, but for 3rd and 4th strongest beam, a similar approach as 1st and 2nd could be followed that only choose beams with same ZoD. It is also basically aligned with the 3rd and 4th strongest cluster AoD and ZoD. In our simulations, we use follow beam directions:

- Strongest beam: AoD: -6.99°, ZoD: 99.99°

- 2nd strongest beam: AoD: -21.99°, ZoD: 99.99°

- 3rd strongest beam: AoD: -35.98°, ZoD: 99.99°

- 4th strongest beam: AoD: -50.97°, ZoD: 99.99°

For codebook, our 8 layers after PMI precoding (using Rel15 codebook with N1=4 and N2=1) will be mapped to the 4 strongest cross-polarized beams above.

It is also observed in our simulation that the TRS/CSI-RS period would impact the final performance when using CDL model. In that case, it’s better to get alignment on it. Companies could take per slot period for CSI-RS as baseline to check the target SNR by a certain CDL model. Once the SNR with per slot period is low enough and then use more practical period, such as 20ms, for further comparison.

Longer TRS/CSI-RS period will have very bad performance with CDL models.

All other parameters could reuse Rel-18 configuration as the start point.

Proposal 7	Take following assumptions for 8Tx8Rx 8 layers PDSCH simulations with FR1 CDL models in 38.827.

| Parameter |  | Value |
| --- | --- | --- |
| Simulated Channel |  | PDSCH |
| Duplex mode |  | TDD 7D1S2U, S=6:4:4 |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |
| UE receiver type |  | SU MMSE-IRC |
| (BB) Antenna configuration |  | 8 Tx / 8 Rx |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS (in Table 1) |  | 17 (for both CWs) for low correlation 7 for medium correlation |
| Channel model |  | FR1 CDL in TR38.827 (i.e., Uma, Umi) (TDL with X-pol medium) |
| Active DL BWP index |  | 1 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | N/A |
|  | PRB bundling size | N/A |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
|  | Transmission Rank | Fixed to 8 (2CW) |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) |
|  | DMRS ports indexes | {1000-1007} for Rank8 |
| Codebook configuration | CodebookType | typeI-SinglePanel |
|  | CodebookMode | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) | (4,1) for 8 Tx refers to CSI-RS ports |
|  | (CodebookConfig-O1,CodebookConfig-O2) | (4,1) |
|  | RI Restriction | 10000000 rank 8 |
| PDSCH & PDSCH DMRS Precoding | Common parameters | Wideband |
|  | Random | Random precoder |
| CSI-RS configuration | Periodicity | 1 slot |
| Analog impairments |  | None (No TO, no FO, no PA model, no TxEVM, no RxEVM, no IQ imbalance, etc.) |
| Number of HARQ Processes |  | 4 |
| Maximum HARQ transmissions |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  | 2 Specific to used TDD pattern |
| UE speed |  | 30km/h (used in 38.827, could be 3km/h for comparison) |

## 2.4	Conformance test consideration

As discussed in 2.1, basic test setup is mentioned in SID and TE vendors have confirmed the feasibility of these setup [9], but the remaining issues are the channel profile implementation and potential high target SNR of the tests.

For channel profile implementation, both CDL extension and TDL extension have limitations as pointed out in 2.2. But it could also be possible to simplify the model, such as reduce the UE speed to smaller value i.e., 3km/h, to support high layer transmission better. By using low UE speed, it could apply same Doppler spectrum for all taps. Another simplification is replacing per tap correlation matrix by per channel correlation matrix for all taps in TDL model if possible.

For time variant beam directions, the method for CDL and TDL extension should be furtherly investigated regarding the complexity of the function.

As for potential high target SNR achieved by a test case under spatial channel model, TE vendor input is needed to give a limitation on it which can be used for decision making.

Proposal 8	Take smaller UE speed for high layer transmission in FR1 to have better target SNR.

Proposal 9	In TDL method discussion, use same Doppler spectrum for all taps rather than   per tap Doppler spectrum if the UE speed is low.

Proposal 9	In TDL method discussion, furtherly evaluate the possibility of replacing per tap correlation matrix by per channel correlation matrix for all taps.

Proposal 10	Further evaluate the method for time variant beam directions in CDL model.

Proposal 11	TE vendors give input on target SNR limitation.

# Conclusions

In the previous sections we made the following observations:

Observation 1	It is possible to consider whole radio channel as a black box for a link between Tx and Rx.

Observation 2	From demodulation perspective, algorithm performance could be independent from a specific antenna configuration at a certain level.

Observation 3	Existing correlation matrix seems not suitable for high layer transmission performance verification.

Observation 4	It is possible to support time variant beam directions for CDL model by changing desired angle directions or additional beam steering.

Observation 5	Fixed angular spread and typical antenna configuration is applied.

Observation 6	Artificial ZoA and ZoD is used for OTA test purpose.

Observation 7	Relative high UE speed is assumed for FRI CDL models.

Observation 8	Doppler spectrum is per cluster.

Observation 9	The model profile in 38.827 seems feasible for test instrument implementation.

Observation 10	It could be straight forward to have TDL with per-tap correlation model according to a CDL model.

Observation 11	It could be possible to have equivalent per channel correlation matrix by per-tap correlation matrix.

Observation 12	Per-tap correlation matrix size could be very large due to tap number and antenna array size.

Observation 13	It would need a very long time to get converging results for Doppler impact comparison between CDL model and per-tap Doppler spectrum.

Observation 14	It is simple and artificial way to have different arrival angles by extended beam steering method.

Observation 15	It is not clear about the principle of configuring beam directions and corresponding power.

Observation 16	Longer TRS/CSI-RS period will have very bad performance with CDL models.

Based on the discussion in the previous sections we propose the following:

Proposal 1 	Take spatial channel model including both BS and UE antenna configuration as a “black box” for demodulation tests. The corresponding configurations should be decided based on typical test case scenario.

Proposal 2	Jointly consider at least availability, test expectation, deployment and testability for channel model evaluation and decision.

Proposal 3	Prioritize study on CDL model extension method. Pending TDL approaches until CDL method has conclusion.

Proposal 4	Only consider Su-MIMO scenario for Rel-19 SCM study.

Proposal 5	Take 8Tx8Rx with 8 layers and 4Tx8Rx with 4 layers for the study as the first.

Proposal 6	The PMI test case discussion could be pended until throughput test case have conclusion.

Proposal 7	Take following assumptions for 8Tx8Rx 8 layers PDSCH simulations with FR1 CDL models in 38.827.

Proposal 8	Take smaller UE speed for high layer transmission in FR1 to have better target SNR.

Proposal 9	In TDL method discussion, use same Doppler spectrum for all taps rather than   per tap Doppler spectrum if the UE speed is low.

Proposal 9	In TDL method discussion, furtherly evaluate the possibility of replacing per tap correlation matrix by per channel correlation matrix for all taps.

Proposal 10	Further evaluate the method for time variant beam directions in CDL model.

Proposal 11	TE vendors give input on target SNR limitation.

# References

[1] 	R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, Nokia

[2]	R4-2416093, General issue on SCM for NR demodulation, Ericsson

[3]	R4-2416095, Simulation results for SCM for NR demodulation, Ericsson

[4]	Henrik Asplund, Jonas Medbo, Bo Goransson, Jonas Karlsson, Johan Skold, ‘A simplified approach to applying the 3GPP spatial channel model’,  the 17th Annual IEEE International Symposium on Personal, Indoor and Mobile Radio Communications (PIMRC'06), 2006

[5]	R4-2412328, Discussion on Spatial Channel Modelling, MediaTek Inc.

[6]	Henrik Asplund, David Astely, etc., ‘Advanced Antenna Systems for 5G Network Deployments, bridging the gap between theory and practice’, Academic Press

[7]	R4-070879, LTE MIMO correlation matrices, Agilent Technologies

[8]	R4-071317, Discussion on per channel versus per path correlation, Agilent Technologies

[9]	R4-2413272, On Rel-19 spatial channel model for demodulation performance requirements, Keysight Technologies UK Ltd

[10]	38.901 v17.1.0

[11]	38.827 v16.8.0

[12]	38.101-4 v18.5.0