3GPP TSG-RAN WG4 Meeting #112bis	R4-2416095

# Hefei, China, October 14th – 18th 2024

Source:	Ericsson

Title:	Simulation results for Rel-19 SCM for demodulation

Agenda item:	6.14.2

Document for:	Information

# Introduction

In RAN4#112 meeting, companies discussed some detailed limitations in current TDL model and corresponding demodulation tests. Some test case assumptions are agreed for initial simulations to compare existing TDL model and CDL model in 38.827.

In this contribution, comparing results are delivered.

# Simulation

The test case is 8Tx8Rx with 8 layers PDSCH demodulation. The antenna configuration is based on agreed configuration in WF.

## 2.1	Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Simulated Channel |  | PDSCH |
| Duplex mode |  | FDD DL only |
| FR / Carrier frequency |  | FR1 / 3.5 GHz |
| UE receiver type |  | SU MMSE |
| (BB) Antenna configuration |  | 8 Tx / 8 Rx |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS |  | 17 (for both CWs) for low correlation 7 for medium correlation |
| Channel model |  | CDLC in TR38.900 CDL-C UMa in TR38.827 TDLC300-100 low TDLC300-100 medium A TDLA30-10 medium A |
| Active DL BWP index |  | 1 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | N/A |
|  | PRB bundling size | N/A |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
|  | Transmission Rank | Fixed to 8 (2CW) |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) |
|  | DMRS ports indexes | {1000-1007} for Rank8 |
| Codebook configuration | CodebookType | typeI-SinglePanel |
|  | CodebookMode | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) | (4,1) |
|  | RI Restriction | 10000000 rank 8 |
| PDSCH & PDSCH DMRS Precoding | Common parameters | Wideband |
|  | Random | Random precoder |
| Analog impairments |  | None (No TO, no FO, no PA model, no TxEVM, no RxEVM, no IQ imbalance, etc.) |
| Number of HARQ Processes |  | 4 |
| Maximum HARQ transmissions |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  | 2 1 |
| UE speed |  | 30 km/h |

## 2.2	Result figures

### 2.2.1 	TDL model

Figure 2.2.1-1 TDLC300-100 low MCS17 with random and fixed precoding

The performance difference between two CW is small when use non-isotropic BS antenna element assumption.

Figure 2.2.1-2 TDLC300-100 ULA medium A MCS7 with random precoding

Figure 2.2.1-3 TDLA30-10 ULA medium A MCS7 with random precoding

Very large performance difference between two CWs, and both CW can’t reach peak throughput.

Correlation matrix medium A only works for ULA antenna which is not a proper configuration for cross-polarization antenna assumptions.

Figure 2.2.1-4 Compare TDLC300-100 and TDLA30-10 X-pol medium MCS7 with random and fixed precoding

TDLA_30_10 performs better as compared with TDLC_300_100 at MCS7 with medium X-pol correlation matrix, still unacceptable level of meaningful performance when comparing normalized throughput, but normalized per CW throughput gives edge to one CW over the other.

### 2.2.2	CDL model results

Figure 2.2.2-1 CDLC in 38.900

Figure 2.2.2-2 FR1 CDLC Uma in 38.827

Both CDLC models in 38.900 and 38.827 could show different performance per CW and average throughput of two CW can’t reach peak value.

Figure 2.2.2-3 TDLC300-100 low vs CDLC in 38.900 vs FR1 CDLC Uma in 38.827

FR1 CDLC Uma model in 38.827 shows smaller performance difference between CW than CDLC model in 38.900. Both show very bad performance at 70% normalized throughput (target SNR is over 30dB).

Figure 2.2.2-5 Different TRS/CSI-RS period comparison with FR1 CDLC Uma in 38.827

The period of TRS/CSI-RS could have obvious impact on performance. Larger CSI-RS period would decrease the total average throughput and especially degrade CW performance transmitted by relative weak beams.

# Conclusions

In the previous sections we made the following observations:

Observation 1	The performance difference between two CW is small when use non-isotropic BS antenna element assumption.

Observation 2	Very large performance difference between two CWs, and both CW can’t reach peak throughput.

Observation 3	Correlation matrix medium A only works for ULA antenna which is not a proper configuration for cross-polarization antenna assumptions.

Observation 4	TDLA_30_10 performs better as compared with TDLC_300_100 at MCS7 with medium X-pol correlation matrix, still unacceptable level of meaningful performance when comparing normalized throughput, but normalized per CW throughput gives edge to one CW over the other.

Observation 5	Both CDLC model in 38.900 and 38.827 could show different performance per CW and average throughput of two CW can’t reach peak value.

Observation 6	FR1 CDLC Uma model in 38.827 shows smaller performance difference between CW than CDLC model in 38.900. Both show very bad performance at 70% normalized throughput (target SNR is over 30dB).

Observation 7	The period of TRS/CSI-RS could have obvious impact on performance. Larger CSI-RS period would decrease the total average throughput and especially degrade CW performance transmitted by relative weak beams.

Based on the discussion in the previous sections we propose the following:

- No table of figures entries found.

# References

[1] 	R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, Nokia

[2]	R4-2416094, Discussion on methodology of NR SCM for demodulation, Ericsson