# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416118

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Source: 	OPPO

Agenda item:	6.15.2.1

Document for:	Discussion

Introduction

In last meeting, a WF[1] was approved on NR_RRM_Ph5_Part1. For Topic#1 FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor, the following agreements have been achieved.

Applicability requirement: Updated Agreement in online session: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: Target and serving carrier frequency can be the same or different. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion. UE Power class: Agreement in online session: Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion. Other clarification on WID: Agreement in online session: Remove {FFS: “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”}. Issue 1-2-1: FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature Agreement in online session: - The conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode. And it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception. Issue 1-2-2: FFS: UE’s mobility status, e.g., whether HST is precluded or not Agreement in online session: - RAN4 to consider UE in non-HST case as first priority. - Note: whether or how HST case could use the outcome of the WI discussion can be FFS after concluding on non-HST case. These extra FFS parts will NOT delay the WI completion. Issue 1-2-6: FFS: UE has prior knowledge on the cell to be measured Agreement: On top of the UE capability of supporting Multi-Rx, no additional conditions of prior knowledge for target cell is needed Issue 1-2-7: FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment Agreement: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment Issue 1-2-10: FFS: Simultaneous operation between L3 and L1 measurements Agreement: Do not consider simultaneous Multi-Rx operation for both L1 and L3 measurement on the same symbols in this WI. Issue 1-6: scheduling/measurement restriction relaxation Agreement: RAN4 to not consider simultaneous operation between L3 measurement and data reception on the same OFDM symbols.

In this contribution, we will provide our views on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor.

Discussion

Power consumption issue

- Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. - Option 2 (HW): - RAN4 shall firstly identify the promising scenario(s) for L3 measurement delay reduction enabled by multi-Rx with clear/significant benefits, which could help to converge the discussion. - One possible scenario to be considered is when there is strong demand of mobility performance (e.g. UE at cell edge or the link is about to break). - It shall not be assumed that UE supporting this feature shall activate multiple panels all the time for all L3 measurement. RAN4 shall discuss the entering and/or exiting conditions for enhanced L3 measurement enabled by multi-Rx taking the targeting scenario into account. - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. - Option 4 (vivo): Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. - RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). [Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed.

In our view, two issues should be discussed for UE power consumption due to FBS under with multi-Rx operation: how to define entering and exiting rules of FBS, and how UE can meet the conditions and corresponding requirements.

Not that all to be discussed in this WID is related to UE supporting multi-Rx reception. The UE which can do FBS without multi-Rx on operation is not in the scope.

For entering and exiting rules of FBS, we can discuss the different conditions or configuration to enable or disable FBS. Option 3 is most straightforward to us that UE can follow the network’s indication. Additional criteria UE needs to meet are not necessary to introduce. On the contrary, they can be set as side conditions of RRM requirements to be defined for UE performing FBS. For option 1 that L3 measurement with FBS can be activated/deactivation following on R18 mechanism can also work, but the disadvantage is that R18 conditions may need revised as we already agreed that it does not assume that the condition of in active mode is same as that for Rel-18 multi-Rx simultaneous reception and the configuration of R18 multi-Rx operation could not be reused directly.

Generally, for and exiting rule, we support option 3 to introduce R19 indication by network, and L3 signalling could be fine.

In addition, for exiting rule, we share the similar concerns in option 2/4/8 that UE is allowed to turn the panels off even during conducting FBS procedure due to network triggering. Once the delay requirements of FBS can be met, UE still has the chance to fall back to single panel or deactivate multi-Rx for some while. We are not sure it is mandatory to ensure network can know this immediately, but UE assistance information can be also beneficial for network scheduling optimization. In our view, either the legacy signalling like the overheating indication, or new UAI indication of UE preference is feasible. We can leave these options to RAN2 decision.

For FBS entering and exiting rule, support option 3 to introduce R19 indication by network, and L3 signalling is preferred. Additional criteria are not necessary to introduce, but can be set as side conditions for requirements UE needs to meet.

Excepting network configuration for FBS, UE assistance information can be beneficial, and either the legacy overheating indication or new UAI indication is feasible and it should be up to RAN2 decision.

Searcher assumption

Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier [Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT)

We are open to consider the enhanced searcher assumption for R19 UE supporting multiple-Rx simultaneous reception for L3 delay enhancement. If UE performs RRM measurement with two panels activated, two searchers can be occupied by this single carrier. But the impact on UE baseband and processing needs to be clarified before we agreed on that, e.g., whether to introduce additional processing time.

Open to support the enhanced searcher assumption that two searchers are occupied by this single carrier for L3 RRM measurement with two UE’s panels activated. Clarify the impact on processing time.

Scenarios and requirements for L3 measurement delay reduction by optimizing Rx BSF

- Issue 1-3: Scenarios to use L3 measurement delay reduction by optimizing Rx BSF - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 5: Handover - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection

As agreed in last meeting, scenario 1/2/3/4/5/7 can be considered as first priority. After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the baseline scenarios 1/2/3/4/5/7, RAN4 can discuss whether and how the solutions(s) can be extended to the other scenarios. The discussion on other scenarios will NOT delay the WI completion.

Issue 1-4: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF [Way forward]: FFS: - Proposal 1 (CATT): - For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Proposal 2 (Apple, CTC): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Proposal 3 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Proposal 4 (LGE): - If UE supports multi-Rx reception simultaneously for L3 measurement and signal quality for serving cell is low, L3 measurement delay reduction by optimizing Rx BSF could be applied to scenario 1 to 4. - RAN4 to discuss how to reduce M values for SSB based intra- / inter-frequency measurements for UE supporting multi-Rx simultaneous reception - Proposal 5 (Ericsson): - As a result of FBS, RX beam sweep factor can be defined as [4]. - Proposal 6 (Ericsson): - RAN4 to study the requirements relevant to switching between different operations, including: - Transition time period when enabling and disabling FBS in Rel-19. - Transition time period when switching between FBS in Rel-19 and multi-Rx in Rel-18. - Proposal 7 (Samsung): - To define the delay requirements for intra-frequency and inter-frequency measurement w/o MG for L3 multi-Rx measurement, suggest to take N=4, M=20 as the baseline - Whether to/How to define the other values can be based on SLS results (if necessary)

Besides the different values of RX beam sweep factors, the corresponding conditions are also considered. Taking FR2 intra-frequency L3 measurements as example, the legacy M considers the beam sweep factor as 8 for different UE power classes. The new factor M can be decided by both UE capability and side conditions. Different values can apply for different cases.

Discuss the reduced scaling factors due to Rx BSF, considering both UE capabilities and applicable conditions.

Conclusions

In this paper we provided our views on FR2-1 SSB based L3 measurement delay reduction for optimizing BSF. The followings are provided.

- For FBS entering and exiting rule, support option 3 to introduce R19 indication by network, and L3 signalling is preferred. Additional criteria are not necessary to introduce, but can be set as side conditions for requirements UE needs to meet.

Excepting network configuration for FBS, UE assistance information can be beneficial, and either the legacy overheating indication or new UAI indication is feasible and it should be up to RAN2 decision.

Open to support the enhanced searcher assumption that two searchers are occupied by this single carrier for L3 RRM measurement with two UE’s panels activated. Clarify the impact on processing time.

Discuss the reduced scaling factors due to Rx BSF, considering both UE capabilities and applicable conditions.

Reference

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple