# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416120

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.3

Source:	OPPO

Title:	On fast SCell activation with EMR

Document for:	Discussion

- Introduction

In last meeting, RAN4 initially discussed the R19 fast SCell activation for UE supporting Rel-18 EMR, based on the latest WID[1]. A WF[2] was approved on fast SCell activation.

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this contribution, we provide our views on fast SCell activation for UE supporting Rel-18 EMR.

- Discussion

#### Issue 1-1-2: Applicability of fast SCell activation delay requirements Agreement: - The fast SCell activation delay requirements are defined for the case when - the UE supports Rel-18 eEMR: - Including both EMR and cell reselection measurement, - Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are configured - FFS Including the case when measIdleValidityDuration-r18 and / or measReselectionValidityDuration-r18 are not configured - and the UE has reported valid results on the SCell to be activated before SCell activation command.

According to clause 4.7.3 in spec TS 38.133, The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

For R19 UE and network, UE may support R18 eEMR but network doesn’t configure the R18 signaling e.g., measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured. The case should not be precluded when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and UE has reported valid results on the SCell to be activated before SCell activation command. For UE supporting the capability of R19 fast Scell activation with EMR, a new indication of valid measurement results of idle/inactive mode can be introduced, or reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

Proposal 1: RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Proposal 2: To support the case in proposal 1, we suggest two options：1)a new indication of valid measurement results of idle/inactive mode can be introduced, or 2)reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

EMR/eEMR was introduced for RRC_IDLE/RRC_Inactive mode, and the existing known/unknown SCell condition during SCell configuration/activation does not consider whether UE has valid EMR for the target SCell or not, which may cause unnecessary long delay requirement for the target SCell activation in RRC connected mode. In this WID, EMR and SCell setup/activation can be considered together to reduce the total delay. Two issues were raised in last meeting and we will discuss them one by one in the followings.

The first issue to be discussed is the side condition. For EMR or re-selection in RRC_IDLE/RRC_Inactive mode, SSB Ês/Iot>=-4dB. However, in some cases of SCell activation, SNR >=-2dB. The mismatch of the SNR condition needs to be solved. Based on current EMR reporting or existing measurement result report rule, network cannot differentiate which one is more reliable measurement result to support fast SCell activation as known SCell.The validity of measurement is mainly limited to check the freshness of the measurement, but not applicable to the justification of known or unknown SCell.

For EMR+fast SCell activation, we think a unified and consistent SNR side condition can be set, e.g., SNR>=-2dB, to guarantee the EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode.

Proposal 3: RAN4 to define a unified SNR side condition for EMR+ fast SCell activation.

The second issue to be discussed is the time duration for known SCell activation, including both the period for valid report and SSB remaining detectable.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown. The requirement for unknown SCell applies provided that the activation commands for PDCCH TCI, PDSCH TCI (when applicable), semi-persistent CSI-RS for CQI reporting (when applicable), and configuration message for TCI of periodic CSI-RS for CQI reporting (when applicable) are based on the latest valid L1-RSRP reporting.

In principle, we think the current valid measurement report in the known/unknown condition can cover the EMR report besides the legacy L3 report, based on the definition of EMR and validation in R18. They brings no difference.

Proposal 4: The legacy time duration condition (e.g., max(5*measCycleSCell, 5*DRX cycles) for FR1 ) for valid report defined in current known/unknown SCell conditions can apply for fast SCell activation with valid EMR report.

For time duration for SSB remaining detectable, e.g., max(5*measCycleSCell,  5*DRX cycles) for FR1 known SCell, the legacy values are defined based on SSB measurement during the SCell activation procedure in the RRC connected mode. For fast SCell activation with EMR, the SSB measurement happens in the IDLE/Inactive mode, and therefore the measurement period can be changed to that for inter-frequency NR cells measurement in IDLE/Inactive mode, i.e.,

- measurement period in section 4.4.2.2, if a UE supporting measValidationReportEMR-r18 and configured with measIdleCarrierListNR-r16 by higher layers.

- measurement period in section 4.2.2.4, if UE supporting measValidationReportReselectionMeasurements-r18 and idleInactiveNR-MeasReport-r16.

Proposal 5: The legacy time duration condition for SSB remaining detectable can be replaced with the measurement period for inter-frequency NR cells in the idle/inactive mode.

- Conclusion

In this contribution, we discuss the RRM requirement for fast SCell activation with EMR.

Proposal 1: RAN4 to include the case when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 are not configured and define requirements of fast SCell activation with EMR.

Proposal 2: To support the case in proposal 1, we suggest two options：1)a new indication of valid measurement results of idle/inactive mode can be introduced, or 2)reusing the R18 validity check rule with fulfilling a default value of ValidityDuration.

Proposal 3: RAN4 to define a unified SNR side condition for EMR+ fast SCell activation.

Proposal 4: The legacy time duration condition (e.g., max(5*measCycleSCell, 5*DRX cycles) for FR1 ) for valid report defined in current known/unknown SCell conditions can apply for fast SCell activation with valid EMR report.

Proposal 5: The legacy time duration condition for SSB remaining detectable can be replaced with the measurement period for inter-frequency NR cells in the idle/inactive mode.

References

[1] RP-240830, NR Radio Resource Management (RRM) Phase 5, Apple, CATT

[2] R4-2414041, WF on NR_RRM_Ph5_Part2 - fast SCell activation, CATT