# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416121

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	On RRM requirements of on-demand SSB SCell operation

Source: 	OPPO

Agenda item:	6.22.2.1

Document for:	Discussion

Introduction

According to the WID, 3 objectives could be related to RAN4 work. Due to on-demand SIB1 and adaptive SSB/RACH/Paging depend on RAN1/2 progress, RAN4 can first focus to specify the requirements for on-demand SSB(OD-SSB) SCell operation.

- On-demand SSB SCell operation, for RRC_CONNECTED

- On-demand SIB1, for RRC_IDLE/RRC_INACTIVE

- Adaptive SSB/RACH/Paging

In last meeting, a WF was approved and the following agreements are achieved on OD-SSB requirements:

Issue 1-1-1: OD-SSB work plan - Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed. Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement. Issue 1-1-3: Deactivated SCell measurement requirement - Agreement: - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement. - Includes both FR1 and FR2-1 Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion.

In this contribution, we will provide discussion on RRM requirements for On-demand SSB SCell operation.

Discussion

RAN4 has agreed to define requirements for OD-SSB based deactivated Scell measurement and Scell activation. And RAN4 will first focus on the MAC CE based single SCell activation case as starting point.

Requirements for Case 1

Deactivated SCell L3 measurement part Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

RAN4 needs to further discuss whether to consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. From our side, we think it is a valid case to be discussed. However, dynamic OD-SSB on/off transmission may not bring too much network energy saving gain but cause more UE power consumption compared to legacy deactivated SCell measurement based on meascycleSCell.

Observation 1: RAN4 to decide whether to consider the scenarios #2 and #2A together under case #1.

For RRM requirements, we think the OD-SSB is mainly to speed up SCell activation delay, and it may provide some extra information of to be activated SCell for network to decide whether to activate this Scell.

Base on this, one possible way is UE supporting one shot OD-SSB measurement during an effective time windows before Scell activation, and the other is network is assumed to activate this SCell together with OD-SSB indication or within the period equal to max(5*measCycleSCell, 5*DRX cycles) after the reception of OD-SSB indication. Under these conditions, we think it is possible to support faster L3 deactivated Scell measurement before Scell activation. For example, max(OD-SSB periodicity, SMTC) could be proper from UE side considering the balance of all serving cells’ measurement and UE power consumption. Otherwise, legacy meascycleSCell or SMTC can be reused. But RAN4 needs to wait for more information on this because RAN1 and RAN2 are still discussing the timer or duration of OD-SSB indication.

During Scell activation we think the similar logic of R18 faster Scell activation can be reused here so that OD-SSB periodicity can be used to replace current Trs or Tfirst_SMTC.

After SCell activation, the L3 measurement on normal activated SCell should be unchanged due to UE power saving. Any enhancement could be out of scope of R19 NES about OD-SSB.

In summary, for Case 1, we only expect to enhance requirements for SCell activation related procedure/purpose, but not L3 activated or deactivated SCell measurement themselves.

Proposal 1: For Case 1, RAN4 to consider the enhancement due to OD-SSB during Scell activation procedure. Not consider enhancement of L3 measurement due to OD-SSB after Scell activation. FFS whether and how to enhance L3 deactivated SCell measurement before SCell activation.

Two main issues may need to discuss on OD-SSB based SCell activation. One is about the known/unknown conditions, and the other is the SCell activation delay.

If RAN4 decided not to consider OD-SSB based enhancement before SCell activation, RAN4 only considers the new impact of OD-SSB transmission after Scell activation command. The target SCell is unknown since there is no always-on SSB before SCell activation command. RAN4 can specify SCell activation requirements of unknown case for Case #1. And we see no big difference of requirements between Scenario 2 and 2A.

Otherwise, RAN4 also needs to firstly discuss known or unknown Scell before Scell activation, and the definition of known/unknown would be updated at least including the effective time windows for reporting and SSB detection.

Proposal 2: For case 1, RAN4 specifies Scell activation requirements assuming the target SCell is unknown with high priority. Not differentiate requirements between Scenario 2 and 2A.

Requirements for Case 2

For Case 2 with always-on SSB on the Scell, OD-SSB can be configured/triggered on the same frequency as always-on SSB, which could have different time offsets and periodicities.

Similar to Case 1, we prefer not to enhance the L3 measurement on deactivated or activated Scell only, but consider the dependency on SCell activation. The legacy L3 deactivation Scell measurement before OD-SSB transmission can be kept unchanged.

During SCell activation, whether and how to combine the measurement results of two kinds of SSB need discussion. For the same SSB frequency, time offset and index, it is more feasible for UE to receive and process these SSBs. If UE combines the measurement results of OD-SSB and always-on SSB, we think the additional UE capability and restrictions, e.g., the available duration of OD-SSB transmission. RAN4 needs to wait for more information on this because RAN1 and RAN2 are still discussing the timer or duration of OD-SSB indication.

Proposal 3：For Case 2, RAN4 to discuss how to handle OD-SSB and always-on SSBs on the same frequency and spatial domain of deactivated SCell firstly.

From our side, we slightly prefer to use the minimum of periodicities of OD-SSB and always-on SSB to speed up the measurement during Scell activation.

Proposal 4：For Case 2, prefer to use the min of OD-SSB and always-on SSB periodicities to define the requirement during Scell activation.

Before/after SCell activation, UE does not need to conduct Scell measurement based on OD-SSB periodicity, but the legacy meascycleSCell/SMTC or the longer one between meascycleSCell/SMTC periodicity and OD-SSB periodicity.

Proposal 5:  For Case 2, consider the legacy meascycleSCell/SMTC or the longer one between meascycleSCell/SMTC periodicity and OD-SSB periodicity before/after SCell activation

For known/unknown definition in case 2, as always-on SSB transmission is before/together with the SCell activation command, current definition of known/unknown based on normal SSB can be reused. When OD-SSB transmission is performed after SCell activation command, RAN4 will specify OD-SSB+always-on SSB based SCell activation procedure for both known and unknown cases. The requirements can be decided based on proposal 3 and 4. And we think there is no need to specifically differentiate Type 2/2a scenarios.

By the way, if companies want to update the definition of known/unknown Scell, the issue in observation 1 should be clarified first. At least we do not see the strong intention to enhance the L3 measurement on deactivated or activated Scell only in this WI.

Proposal 6：Define Scell activation requirements of Case 2 for both known and unknown case.

Conclusions

In this paper we provided our views.

Observation 1: RAN4 to decide whether to consider the scenarios #2 and #2A together under case #1.

Proposal 1: For Case 1, RAN4 to consider the enhancement due to OD-SSB during Scell activation procedure. Not consider enhancement of L3 measurement due to OD-SSB after Scell activation. FFS whether and how to enhance L3 deactivated SCell measurement before SCell activation.

Proposal 2: For case 1, RAN4 specifies Scell activation requirements assuming the target SCell is unknown with high priority. Not differentiate requirements between Scenario 2 and 2A.

Proposal 3：For Case 2, RAN4 to discuss how to handle OD-SSB and always-on SSBs on the same frequency and spatial domain of deactivated SCell firstly.

Proposal 4：For Case 2, prefer to use the min of OD-SSB and always-on SSB periodicities to define the requirement during Scell activation.

Proposal 5: For Case 2, consider the legacy meascycleSCell/SMTC or the longer one between meascycleSCell/SMTC periodicity and OD-SSB periodicity before/after SCell activation

Proposal 6：Define Scell activation requirements of Case 2 for both known and unknown case.

Reference

- RP-240170, Revised WID for Enhancements of network energy savings for NR

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson