# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416127

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on other RRM requirements of R19 NR NTN

Source: 	OPPO

Agenda item:	6.26.4.2

Document for:	Discussion

Introduction

In last RAN4 meeting, a WF on RRM requirements for Rel-19 NR NTN phase3 was approved [1], including downlink coverage enhancements, support of regenerative payload, and support of (e)RedCap UEs with NR FR1-NTN.

In this paper, we will provide our views on DL coverage enhancement.

Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

Discussion

- Sub-topic 2-1 SSB periodicity enhancement - Issue 2-1-1: RRM impact of SSB periodicity enhancement <Way Forward> : Further discuss the following proposal - Proposal 1 (CATT, Apple, Xiaomi, Samsung, HW, QC): - RAN4 to wait for more progress from RAN1/2 on SSB periodicity extension and other possible system level enhancements to discuss the RRM impacts of DL coverage enhancement. - Proposal 1a (Samsung): - If SSB periodicity is extended, almost all RRM requirements should be impacted including idle mode/inactive mode and connected mode such as: cell (re)-selection, L3 measurement/L1 measurement and so on. Need further RAN1 conclusion on whether SSB periodicity is changed. - Proposal 1b (Ericsson): RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of SSB/SMTC periodicity. - RAN4 to check if RRM requirements could be impacted by SSB periodicity extension to a value larger than 20ms during cell search, including the remaining physical channels and signals (e.g., SIB, paging, etc.).

RAN4 discussed the impact of SSB periodicity extension in last meeting. IF RAN1/2 concluded on these, the related measurement requirements may need to be updated, e.g., timing, cell (re)-selection, L3 measurement/L1 measurement.

For example, if 320ms SSB periodicity is supported, RAN4 may need to discuss if existing timing accuracy requirements should apply, and if the existing measurement requirements apply with beam hopping.

Proposal 1: If SSB periodicity extension was agreed, RAN4 to study and update the measurement requirements, e.g., timing, cell (re)-selection, L3 measurement/L1 measurement.

- Sub-topic 2-2 Network energy saving - Issue 2-2-1: RRM impact of DTX/DRX <Way Forward> : Further discuss the following proposal - Proposal 1 (Apple): - RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES. - Proposal 2 (Samsung): - All the measurements related to DL-RS should be impacts in “off” state. RAN4 to discuss and define new RRM requirements to enhance DL coverage. Need further progress from other WGs. - Proposal 3 (Ericsson): - RAN4 to check if RRM requirements could be impacted by scaled measurement time delay due to extension of DRX periodicity. - Sub-topic 2-3 Dynamic and flexible power sharing - Issue 2-3-1: RRM impact of dynamic and flexible power sharing <Way Forward> : Further discuss the following proposal - Proposal 1 (LG): - After RAN1 concludes the solution for dynamic and flexible power sharing among satellite beams or different satellite beam patterns/size, RAN4 should check whether the outcome from RAN1 is affect RRM requirements. - Proposal 2 (Xiaomi): - RAN4 to study the RRM impact of satellite beams activation/deactivation scheme after RAN1/2 reaching more concrete conclusions. - Sub-topic 2-4 Other RRM impacts - Issue 2-4-1: Beam switching related issues <Way Forward> : Further discuss the following proposal - Proposal 1 (Ericsson): RAN4 to check if beam steering latency other than 0 may impact RRM requirements. - Proposal 2 (Ericsson): RAN4 to check if timing offset/error (between beam switching timing and DL/UL framework) may impact RRM requirements. - Proposal 3 (Ericsson): If applicable, the requirements for quasi-earth_fixed cell and earth_ moving cell both shall be studied with respect to the beam hopping scheme.

For others, we think DTX/DRX, Dynamic and flexible power sharing could have limited impacts on RRM requirements of NTN. RAN4 can start the work after RAN1/2 reached more concrete conclusion.

Conclusions

In this paper we provided our views on other RRM requirements for R19 NTN.

Proposal 1: If SSB periodicity extension was agreed, RAN4 to study and update the measurement requirements, e.g., timing, cell (re)-selection, L3 measurement/L1 measurement.

Reference

- R4-2414037, WF on RRM requirements for Rel-19 NR NTN phase3, CATT