3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416134

Hefei, China, October 14th – 18th 2024

Agenda Item:	6.20.1

Source:	Ericsson

Title:	SBFD general aspects

Document for:	Approval

# 1	Introduction

In this contribution, we present our views and proposals on general aspects for SBFD capable BS regarding band support for SBFD capable BS, SBFD sub-band configuration, and specification structure. Comprehensive analysis of each SBFD BS RF TX/RX requirement and potential new requirements for SBFD capable BS are detailed in separate submissions [1-3].

# 2	Discussion

2.1	Band support

#### Issue 1-2-1: Band/channel bandwidth support for SBFD - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

In RAN4#112, whether SBFD should be a feature as band specific or general to all TDD bands was discussed in the AdHoc session, and the agreement was captured in the WF [4] as listed above.

It was tentatively agreed that the supported channel bandwidth shall be no less than X = [20] MHz, and we need to further discuss the potential to increase the value of X beyond X = [20].

In SBFD subband configuration, guardbands play a crucial role in preventing interference between adjacent subbands. However, excessively wide guardband can consume valuable bandwidth. Considering the case that the channel bandwidth is low, after excluding the guardband, the remaining PRBs for both DL and UL subbands may be too little and become insufficient. Such a situation is not practical in real-world deployments. In RAN4#112, it was proposed to deploy SBFD with 30 MHz channel bandwidth for mid bands. Assume that UL subband size is 20% of channel bandwidth, i.e., 6 MHz, and guardbands consume a few RBs, there are not many resources left for DL. In this case, both the UL and DL subbands are narrow, while vendors need to spend huge effort in the front end to achieve sufficient isolation from self-interference, inter-sector interference, as well as inter-site interference. From a spectrum efficiency point of view, it is better to deploy dynamic TDD, i.e., change DL slots to UL slots, rather than SBFD when channel bandwidth is narrow, e.g., 30 MHz.

In our view, a general principle should be applied when specifying SBFD: The channel bandwidth should be no less than 50 MHz.

Wide guardband can consume valuable bandwidth. When the channel bandwidth is low, after excluding the guardband, the remaining PRBs for both DL and UL subbands may become insufficient. Such a situation is not practical in real-world deployments.

A general principle should be applied when specifying SBFD: The channel bandwidth should be no less than 50 MHz.

When it comes to how to implement the principle in the spec, we notice that the majority companies support option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. More directly, we think it is better to clearly state which TDD bands support SBFD feature, while the condition “with channel bandwidth >= X MHz” can be explicitly shown in Transmission bandwidth configuration in Clause 5.3.

To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1.

2.2	SBFD subband configuration

#### Issue 1-2-4: DL/UL subband configurations support for SBFD - Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility. - BS transmission configuration in BS RF requirements (for information only): Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264. #### Issue 1-2-5: Restriction on guardband size for SBFD - Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

2.2.1	Clarification on declaration and flexibility

There are three options identified in the WF [4] from RAN4#112, which were distinguished by their level of flexibility. From a standardization perspective, it is important to exercise caution when allowing flexibility in technical standards/specifications. In our view, SBFD subband configurations need to be captured in RAN4 specification. Manufacturer can declare which configurations defined in specification are supported, and all supported SBFD configurations shall be tested, due to the fact that for SBFD capable BS, achieving different configurations may require adapting hardware components, which may involve variations in analog and/or digital filter characteristics. Most importantly, the BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL subband/UL subband/guardband setting other than those that have been declared and tested.

The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested.

Based on the AdHoc discussion in RAN4#112, the three (four) options can be down selected to the following two options.

- Option 1: fixed configurations/UL subbands/DL subbands/guardbands.

- Option 2b: fixed UL subbands and vendor declared DL subbands and guardbands.

The pros and cons of the two options are analyzed in table 1, and a critical issue on interoperability is identified: Different vendors' BSs use varying sizes of DL subbands/UL subbands/guardbands may cause interference. Consequently, an operator must either deploy the entire network with a single vendor or ensure same SBFD subband configuration through the flexible configurations between different vendors.

Table 1: Pros and Cons of SBFD subband configuration options

|  | Pros | Cons |
| --- | --- | --- |
| Option 1 | - An operator can deploy the network with different vendors. - From a standardization perspective, test models can be clearly defined. - From a regulator perspective, no flexibility and manufacturer declaration is allowed. - Reduce SBFD aware UEs’ complexity. - A limited number of configurations simplify the standardization work, and make sure test models are under control. | - Cannot cover all possibilities, but desired configurations by operator can be added in SBFD enhancement WI. - Difficult to define guardbands for subbands. |
| Option 2b | - Vendor can select guardband freely, and guardband is up to implementation. | - Interoperability issue: one operator has to deploy the whole SBFD network with the same vendor, or needs to ensure a common configuration is available by having the vendors collaborate. - Importance of defining RF requirements in RAN4 specifications are weakened. - Regulator will not accept manufacturer declaration. |

To have a clear test scope and test parameters (test models etc. need to be designed for each configuration), we should have a limited set of SBFD configurations. Although RAN1 allows more flexibility in selecting bandwidths and subcarrier spacings (SCSs), in RAN4, we have restricted these parameters to limited sets. Likewise, we propose limiting SBFD configurations to a predefined set.

Another aspect which needs to be considered is whether SBFD aware UEs need to support all configurations. If UEs need to support every possibility, it is UEs’ complexity as well. To ease the burden of conformance testing and reduce the complexity for SBFD aware UEs, we explore the following ways to streamline SBFD subband configurations.

SBFD configurations shall be captured in Rel-19 RAN4 specification and restricted to a limited set.

Based on our observation, it is reasonable to fix the size of UL subband as described in Option 2b to be able to reuse the FRCs defined in TS 38.104 [5]. Option 2b has two level of flexibilities: one is the size of the UL subband of a supported channel bandwidth; another is the location of the UL subband. In our view, UL should be around 20% of supported channel bandwidth, since feasibility and co-existence only studied 20%, i.e., for one supported channel bandwidth, only one fixed UL subband size is allowed. On one hand, there is no need to support narrower sizes of UL subband, as it doesn’t gain much to deploy SBFD in this case. On the other hand, if the size of UL subband is too wide, the DL subbands will be impacted. Moreover, the UL subband should be placed right in the middle of the channel bandwidth, as we do not see any benefit from flexible placement of the UL subband.

When the two level of flexibilities are out of scope, the only remaining differences between Option 1 and Option 2b are the selection of the guardbands and the sizes of DL subbands. As mentioned in table 1, a critical issue is interoperability between different vendors. Therefore, we propose to proceed with Option 1 and in the following text we will provide our recommendations on the selection of the guardbands and the sizes of DL subbands.

For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth.

Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband.

Select Option 1 to specify DL/UL subband configuration in BS RF requirements, due to interoperability issues between different vendors.

2.2.2	Proposal of SBFD subband configuration

It has been agreed that only DUD and DU configurations are specified for the SBFD subband configuration.

As justified in band support topic in this T-doc, we propose to support SBFD with channel bandwidth no less than 50 MHz. In the SI phase, the feasibility and co-existence study was based on the assumption that UL subband occupies 20% BS channel bandwidth. Hence, we propose to continue with the assumption that UL subband occupies 20% BS channel bandwidth for FR1 in the WI phase. As justified by companies in RAN4#112, it is more important to fix the UL subband size than DL subband size: Thus, to be able to reuse the existing channel bandwidths for UL subbands, the percentage is relaxed to 20% - 25% to design suitable SBFD subband configurations for all channel bandwidths between 50 MHz and 100 MHz.

Reuse the existing channel bandwidths for SBFD UL subbands.

SBFD-capable BS channel bandwidths are down selected to 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz and 100 MHz for FR1.

As for sub carrier spacing (SCS), for FR1, according to UE demodulation requirement, 30 kHz SCS is selected for TDD and 15 kHz SCS is selected for FDD. Considering SBFD is supported in TDD operation, 30 kHz shall be the SCS for SBFD slots in FR1.

During the online discussion in RAN4#112, 15 kHz SCS was also proposed to be supported for SBFD. In our observation, 15 kHz SCS is only supported by channel bandwidth no larger than 50 MHz, which are too narrow to be considered for SBFD.

15 kHz SCS is only supported by channel bandwidth no larger than 50 MHz, which are too narrow to be considered for SBFD.

For FR1, use 30 kHz as the SCS for SBFD slots.

The most important part is the selection of guardband size between DL and UL subbands. Different BS classes may require different techniques to suppress interference during SBFD operation, for example, WA BSs would require analogue filtering, digital filtering, and high linearity of the receiver, etc., while for LA BSs there is no need to build analogue filter and highly linear receiver. The guardband size can thus be different for different BS classes. For SBFD-capable WA BSs, wider guardbands than the guards used for BS in normal TDD operation are needed.

For WA BSs, wider guardbands than existing ones are needed for SBFD.

To set the guardbands for WA BS, a general calculation rule is proposed, i.e., the guardband size should be larger than the sum of the minimum guardbands (defined in TS 38.104 [5]) of the adjacent DL subband and the adjacent UL subband.

The calculation approach for SBFD transmission bandwidth configuration for FR1 is demonstrated as below.

- Step 1 – Define NRB,SBFD,UL. Reuse the existing number defined in Table 5.3.2-1 in TS 38.104 [5] corresponding to 20% - 25% of the considered channel bandwidth.

- Step 2 – Define NRB,SBFD,GB. Sum up the minimum guardbands (defined in Table 5.3.3-1 in TS 38.104 [5]) of the adjacent DL subband and the adjacent UL subband. Convert the number in kHz to RB and round up the decimal. The result obtained is the needed NRB,SBFD,GB.

NOTE: When the adjacent DL subband is not standard channel bandwidth, use the larger minimum guardband of the two closest channel bandwidths. E.g., for 70 MHz DUD (27.5 MHz - 15 MHz - 27.5 MHz), 30 kHz SCS, the minimum guardband for DL subband should use 945 kHz (corresponding to 30 MHz) instead of 785 kHz (corresponding to 25 MHz).

- Step 3 – Define NRB,SBFD,DL.

  - For DUD, NRB,SBFD,DL = (NRB – NRB,SBFD,UL – 2 * NRB,SBFD,GB)/2.

  - For DU/UD, NRB,SBFD,DL = NRB – NRB,SBFD,UL – NRB,SBFD,GB.

NOTE: When NRB is an odd number and NRB,SBFD,UL is an even number, e.g., calculation for 50 MHz channel bandwidth, the obtained NRB,SBFD,DL is a decimal of 0.5. In this case, NRB,SBFD,DL for two DL subbands should be 49 and 50.

It is also worth noting that, the NRB,SBFD,DL may not be the existing number defined in TS 38.104 [5], and the corresponding test models need FFS.

To have a straightforward view, the proposed supported BS channel bandwidth and possible subband configurations for WA BS are listed in Table 2 for FR1.

RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1.

Table 2: Transmission bandwidth configuration NRB for SBFD for FR1

|  |  | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
|  | D | 20 | 22.5 | 27.5 | 30 | 35 | 40 |
| DUD (SCS 30 kHz) | NRB,SBFD,DL | 49/50 | 5857 | 70/71 | 78 | 92 | 106 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 45 | 5 | 5 | 5 | 5 |
|  | D | 40 | 45 | 55 | 60 | 70 | 80 |
|  | U | 10 | 15 | 15 | 20 | 20 | 20 |
| DU/UD (SCS 30 kHz) | NRB,SBFD,DL | 104 | 119 | 146 | 161 | 189 | 217 |
|  | NRB,SBFD,UL | 24 | 38 | 38 | 51 | 51 | 51 |
|  | NRB,SBFD,GB | 5 | 5 | 5 | 5 | 5 | 5 |

NOTE: the subband bandwidths of DUD and DU/UD in Table 2 with strikethrough are in MHz and just for understanding, which will be removed when implemented in spec.

In addition, Table 2 shows that the numbers of NRB,SBFD,GB for different configurations are all 5, except for the case of DUD 60 MHz, which is 4. To maintain uniformity across the table, we recommend using 5 RBs as the guardbands for DUD 60 MHz, and the corresponding NRB,SBFD,DL is reduced to 57, instead of 58.

2.2.3	Figures for BS transmission bandwidth configuration for SBFD

#### Issue 1-2-3: Transmission bandwidth configuration for SBFD - Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed - The existing definition of BS transmission bandwidth configuration (for information only): - BS transmission bandwidth configuration: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or receiving by the BS. #### Issue 1-2-4: DL/UL subband configurations support for SBFD - Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

In the WF [4] approved in RAN4#112, under Issue 1-2-3: Transmission bandwidth configuration for SBFD, RAN4 is required to further study whether a corresponding figure for SBFD is needed. We believe that Figure 5.3.1-1 in TS 38.104 doesn’t reflect transmission bandwidth configuration for SBFD DUD and DU/UD patterns. Therefore, we propose to introduce Figure 1-3 as shown below, where the agreed notations for transmission bandwidth configuration for SBFD UL/DL subband: NRB,SBFD,DL and NRB,SBFD,UL, as well as guardband between SBFD UL and DL subband: NRB,SBFD,GB are included, to either TS 38.104 or a SBFD specific spec, which is another topic still under discussion.

Introduce Transmission bandwidth configuration for SBFD DUD, DU and UD patterns, as shown in Figure 1-3 below.

Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel

Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel

Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel

2.3	Specification structure

#### Issue 1-4-2: BS RF Specification structure for SBFD requirements - Way forward: - RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: - Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) - Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. - Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements - Option 3: A new specification for SBFD BS RF requirements

In the WF [4] approved during RAN4#112, there are three options captured on how to introduce BS RF new requirements for SBFD-capable BS. We prefer to create new specifications for SBFD-capable BS, rather than merging it into TS 38.104. The reasons are as follows.

On one hand, a dedicated SBFD spec ensures clarity. It includes only relevant information related to bands, bandwidth, and SBFD configurations, etc., and avoids unnecessary complexity and provides a concise, focused reference for SBFD-related details, which makes it easier to understand and maintain. Creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 for SBFD-specific existing or new gNB RF requirements could reduce the number of 3GPP specifications, but increase the complexity and reduce the readability of TS 38.104, as what we have on UE side. On the other hand, SBFD-capable BS may require new hardware components, which makes it different from legacy base stations.

RAN4 create new specifications for SBFD-capable BS.

# Conclusion

In this contribution, views, and proposals on general aspects for SBFD WI are presented.

Observation 1	Wide guardband can consume valuable bandwidth. When the channel bandwidth is low, after excluding the guardband, the remaining PRBs for both DL and UL subbands may become insufficient. Such a situation is not practical in real-world deployments.

Observation 2	SBFD configurations shall be captured in Rel-19 RAN4 specification and restricted to a limited set.

Observation 3	15 kHz SCS is only supported by channel bandwidth no larger than 50 MHz, which are too narrow to be considered for SBFD.

Proposal 1	A general principle should be applied when specifying SBFD: The channel bandwidth should be no less than 50 MHz.

Proposal 2	To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1: n38, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104 for FR1.

Proposal 3	The SBFD capable BS product shall remain fixed in its configuration during real operation, i.e., it shall not be reconfigurable to a different configuration/DL/UL/guardband setting other than those that have been declared and tested.

Proposal 4	For one supported channel bandwidth, only one fixed UL subband size is specified, which is around 20% of the supported channel bandwidth.

Proposal 5	Only symmetric DUD configurations are specified, as there is no foreseeable benefit from flexible placement of the UL subband.

Proposal 6	Select Option 1 to specify DL/UL subband configuration in BS RF requirements, due to interoperability issues between different vendors.

Proposal 7	Reuse the existing channel bandwidths for SBFD UL subbands.

Proposal 8	SBFD-capable BS channel bandwidths are down selected to 50 MHz, 60 MHz, 70 MHz, 80 MHz, 90 MHz and 100 MHz for FR1.

Proposal 9	For FR1, use 30 kHz as the SCS for SBFD slots.

Proposal 10	For WA BSs, wider guardbands than existing ones are needed for SBFD.

Proposal 11	RAN4 define SBFD subband configurations for WA BS as shown in Table 2 for FR1.

Proposal 12	Introduce Transmission bandwidth configuration for SBFD DUD, DU and UD patterns, as shown in Figure 1-3 below.

Figure 1: Transmission bandwidth configuration for SBFD DUD pattern for one NR channel

Figure 2: Transmission bandwidth configuration for SBFD DU pattern for one NR channel

Figure 3: Transmission bandwidth configuration for SBFD UD pattern for one NR channel

Proposal 13	RAN4 create new specifications for SBFD-capable BS.

# References

R4-2416136, Impact on SBFD BS RF TX requirements, Ericsson

R4-2416137, Impact on SBFD BS RF RX requirements, Ericsson

R4-2416135, Potentially new SBFD BS RF requirements, Ericsson

R4-2413514, Way Forward for [112][307] NR_duplex_evo_General

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP