3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416135

Hefei, China, October 14th – 18th 2024

Agenda Item:	6.20.2.1

Source:	Ericsson

Title:	Potentially new SBFD BS RF requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of Potentially new BS RF requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on impact on existing BS RF TX and RX requirements for SBFD slots and SBFD general aspects are presented in separate submissions [2-4].

# 2	Discussion

Transient period

#### Issue 2-1-3: Location of transient period between SBFD and non-SBFD - Agreement: - Location of transient period between SBFD and non-SBFD in RAN4 RF requirement: - FFS the location of transient period between SBFD and non-SBFD shall be located within the SBFD slot/symbol if it needs to be decided.

In the WF [5] approved during RAN4#112, one issue remains regarding the transient period, i.e., the location of the transient period between SBFD and non-SBFD in RAN4 RF requirement. In legacy TDD, the transient period is considered part of the guard period, as illustrated in Figure 1 below, where TGUARD represents the total allocated guard period (including both DL to UL and UL to DL guard periods); TAoffset is the guard period allocated for the UL to DL switch; TDL_UL is the guard period allocated for the DL to UL switch. Our understanding is the transient period is located outside of both the DL slot and the UL slot in legacy TDD. Therefore, we believe that the location of the transient period between SBFD and non-SBFD should be consistent with its placement in legacy TDD.

Figure 1: Illustration of guard period for legacy TDD

Place the transient period outside of both SBFD and non-SBFD slots.

In-channel adjacent sub-band leakage ratio

For the potential new requirement of in-channel adjacent subband leakage ratio, it is concluded that further study is needed on the necessity of this requirement in normative phase.

In TR 38.858 [1], the necessity of In-channel adjacent sub-band leakage ratio requires further study.

Even though conformance testing does not include other sites, a new requirement needs to be set to enable dimension site design in real networks. While self-interference is captured in the OTA sensitivity, interference from other sectors and other sites need to be considered, i.e., requirements on ACLR and ACS between the sub-bands are needed, which is the basis for dimensioning site design. Moreover, in the feasibility analysis, ACLR and ACS suppression between sub-bands were assumed by all companies.

Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

In-channel adjacent sub-band Blocking and adjacent sub-band selectivity

For the potential new requirements of in-channel adjacent subband blocking and selectivity, it is concluded that further study is needed on the necessity of this requirement in normative phase.

In TR 38.858 [1], the necessity of In-channel adjacent sub-band Blocking and adjacent sub-band selectivity requires further study.

The definition of Adjacent channel selectivity (ACS) in TS 38.104 [6] is,

a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system.

The definition of in-band blocking in TS 38.104 [6] is,

a measure of the receiver's ability to receive a wanted signal at its assigned channel at the antenna connector for BS type 1-C or TAB connector for BS type 1-H in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

From the definition, we know that ACS is the requirement to consider an unwanted interferer (e.g., -52 dBm for WA) from the adjacent channel, while in-band blocking is the requirement to consider an unwanted interferer at a higher level comparing to ACS (e.g., -43 dBm for WA) within the same frequency band. In SBFD, there is only one adjacent DL sub-band, which makes adjacent sub-band Blocking and adjacent sub-band selectivity somewhat the same requirements with two different interferer levels. Therefore, we propose to define a new requirement on RX sub-band ACS similar to the ACS requirement only and use existing ACS requirement as baseline.

Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

# Conclusion

In this contribution, overview of impact on SBFD BS RF requirements is presented with the following observations and proposals.

No table of figures entries found.

Proposal 1	Place the transient period outside of both SBFD and non-SBFD slots.

Proposal 2	Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

Proposal 3	Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

# References

TR 38.858, Study on evolution of NR duplex operation

R4-2416134, Discussion on SBFD general aspects, Ericsson

R4-2416136, Impact on SBFD BS RF TX requirements, Ericsson

R4-2416137, Impact on SBFD BS RF RX requirements, Ericsson

R4-2413514, Way Forward for [112][307] NR_duplex_evo_General

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP