3GPP TSG-RAN WG4 Meeting #112-bis	R4-2416136

Hefei, China, October 14th – 18th 2024

Agenda Item:	6.20.2.2

Source:	Ericsson

Title:	Impact on SBFD BS RF TX requirements

Document for:	Approval

# 1	Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for SBFD slots. For convenience, the content in Clause 10.1 in TR 38.858 (V18.0.0) [1] was copied to the tables under each requirement for the purpose of making observations and proposals.

Discussion on existing BS RF RX requirements for SBFD slots is presented in R4-2416137 [2], potentially new requirements for SBFD slot is presented in R4-2416135 [3], and SBFD general aspects is presented in R4-2416134 [4].

# 2	Discussion

Transmitted signal quality

## 1.4. Joint measurement for normal DL symbols/slots and SBFD symbols/slots WF: - FFS on the followings: - Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately. - Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Calculation of averaged EVM is defined in Annex B.7 for FR1 in TS 38.104 [5]:

EVM is averaged over all allocated downlink resource blocks with the considered modulation scheme in the frequency domain, and a minimum of Ndl slots where Ndl is the number of slots in a 10 ms measurement interval.

For TDD, let NdlTDD be the number of slots with downlink symbols within a 10 ms measurement interval, the averaging in the time domain can be calculated from  NdlTDD slots of different 10 ms measurement intervals and should have a minimum of Ndl slots averaging length where Ndl is the number of slots in a 10 ms measurement interval.

-	EVMframe is derived by: Square the EVM results in each 10 ms measurement interval. Sum the squares, divide the sum by the number of EVM relevant locations, square-root the quotient (RMS).

EVMframe=1i=1NdlTDDNii=1NdlTDDj=1NiEVMi,j2

-	Where Ni is the number of resource blocks with the considered modulation scheme in slot i.

-	The EVMframe is calculated, using the maximum of EVMframe at the window W extremities. Thus EVMframe,l is calculated using t=∆tl and EVMframe,h is calculated using t=∆th (l and h, low and high; where low is the timing ∆c-W/2 and and high is the timing ∆c+W/2).

EVMframe=maxEVMframe,l,EVMframe,h

-	In order to unite at least Ndl slots, consider the minimum integer number of 10 ms measurement intervals, where Nframe is determined by.

Nframe=10×NslotNdlTDD

and Nslot=1 for 15 kHz SCS, Nslot=2 for 30 kHz SCS and Nslot=4 for 60 kHz SCS normal CP.

-	Unite by RMS.

EVM=1Nframek=1NframeEVMframe,k2

Following the definition above, averaged EVM is over all allocated downlink resource blocks. Therefore, regarding the calculation of averaged EVM for SBFD slots, the data samples where ULs are allocated within the SBFD slots shall be removed from the calculation, and only PRBs carrying DL within SBFD slots shall be calculated.

Considering the averaged EVM for SBFD slots may be different from the averaged EVM for non-SBFD slots, in our opinion, they should be measured separately.

Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Unwanted emissions

## 1.3. ACLR and OBUE Agreement: - When coexisting with legacy TDD system in adjacent channel, RAN4 shall apply the legacy ACLR and OBUE requirement for SBFD-capable BS. WF: - FFS on the followings: - For FR1 macro deployments, when coexisting with new SBFD system in adjacent channel, RAN4 to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB. The assumption for the victim BS DESENS and CL of BS2BS is FFS.

In the approved WF [6] in RAN4#112, the open issue related to unwanted emissions requires further study. The proposals involve a new co-existence scenario: a SBFD-capable BS co-exists with another operator’s SBFD-capable BS. The proponents recommend that RAN4 consider more stringent ACLR and OBUE requirements to ensure the co-existence of these two SBFD-capable BSs. However, since the co-existence of two SBFD-capable BSs is a corner case, and the new scenario was not considered in the co-existence study performed in the SI phase, we believe it is unnecessary to impose tougher ACLR and OBUE requirements beyond the existing ones.

RAN4 do not impose tougher ACLR and OBUE requirements beyond the existing ones.

# Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Averaged EVMs for SBFD slots and non-SBFD slots shall be measured separately.

Proposal 2	Averaged EVM for SBFD slots shall be calculated only based on the data samples where DLs are allocated within the SBFD slots. The data samples where ULs are allocated within the SBFD slots shall be removed from the calculation of averaged EVM.

Proposal 3	RAN4 do not impose tougher ACLR and OBUE requirements beyond the existing ones.

# References

TR 38.858, Study on evolution of NR duplex operation

R4-2416137, Impact on SBFD BS RF RX requirements, Ericsson

R4-2416135, Potentially new SBFD BS RF requirements, Ericsson

R4-2416134, Discussion on SBFD general aspects, Ericsson

TS 38.104, “NR; Base Station (BS) radio transmission and reception”, 3GPP

R4-2413515, Way Forward for [112][308] NR_duplex_evo_BSRF