3GPP TSG RAN WG4 Meeting #112bis		 R4- 2416138

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.6.2

Source: 	MediaTek Inc.

Title: 	Discussion on UE Rx chain architecture and network assumption

Document for:	Approval

1. Introduction

It had been discussed in previous RAN4 meeting that considering the spectrum hold be North America operators are usually non-contiguous blocks as stated in the reference [1][2][3]. In the RAN4#112, there was extensive discussion on the scope and assumptions of the study item. Issues were recorded in the appendix of WF [4]. In this contribution, we continue discussion and provide our proposals on the open issues.

2. Discussion

The agreement in the WF[1] Rel-19 NR FR1 DL fragmented carriers were copied below:

The goal of the DL fragmented carrier study is to study the method to enable support of DL non-contiguous 2CC by using a single Rx RF chain

    - The two non-contiguous CCs is still under the current DL non-contiguous CA framework

      - Each fragment is treated as an individual CC

    - The study should be future-proof for the higher order inter/intra-band combinations or additional CC(s) within the same band with fragments within 100MHz.

    - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

      - Consider n2/n25, n3, n7, n66, n41, n39 as the example bands.

        - Provide the details related to gap between fragmented DL blocks]

2.1 Methods for reducing the number of UE Rx chains

Issue 2-1-1: Reference architecture [and baseband assumptions]

There are different UE implementation for DL NCCA configuration. Some typical methods are listed below:

- Separated Rx chain architecture

- Partially-shared Rx chain architecture

- Fully-shared Rx chain architecture

It is foreseen fully separated Rx chain architecture may have best performance with less degradation on REFSENS when there is blocking noise toward the wanted carriers. Such architecture can even handle asynchronous DL intra-band carriers as discussed in Rel-18 non-collocated WI. A fully-shared Rx chain may have difficulty to achieve comparable performance as those implemented with separated Rx chain. And performance of partially shared Rx chain is in between the above-mentioned two architectures.

Figure 1. Receiver architectures for single band DL NCCA configuration

The single band DL NCCA had been discussed and evaluated in TR36.823 and requirements had been specified in the TS36.101. In the previous evaluation, though is was said separated Rx architecture, the UE architecture was actually type 2 partially-shared Rx chain.

Observation 1: The existing DL NCCA requirements are based on partially-shared Rx architecture

The architecture assumption were still left open for discussion. There are two options:

- Option 1: Only the “fully shared Rx chain architecture” needs to be studied

- Option 2: RAN4 also studies the possibility of supporting only 2Rx in the mandatory 4Rx band below 2700MHz

Other options are not precluded

In our view, option 2 can be an alternative aspect that may not need further study on Rx chain requirements. At this stage we can focus on the requirements impact on option 1 : Only the “fully shared Rx chain architecture” needs to be studied.

Proposal 1: We propose focus on option 1 Only the “fully shared Rx chain architecture” needs to be studied for RF requirements evaluation. The option 2 (2Rx relaxation in mandatory 4Rx band below 2700MHz) can be discussed at later stage when signalling aspect starts to be discussed.

There are also discussion on baseband assumptions i.e., shared/separated BB filter that may relate to rejection ability toward unwanted signal for the study as well as “single FFT” or “separate FFT” assumption evaluation. Here we also provide our view as below.

For the baseband signal processing, separate FFT is legacy implement to handle DL NCCA carriers since it provides better rejection on interference and unwanted signal. Single FFT that can receive both NCCA carriers including in-gap interference (with poor rejection) at a time. There are some constraint for single FFT. The channel raster of PCC and SCC shall align to orthogonal grid that can avoid unnecessary ICI after FFT. This would need to be guaranteed by network scheduling. And due to the max FFT size is 4096, for the fragment carriers that span larger than 50MHz, only 30KHz or 60KHz SCS would be applicable for the case.

Observation 2: For DL fragmented carriers received by a single baseband with single FFT, channel raster of PCC and SCC shall align to orthogonal grid and this would need to be guaranteed by network scheduling.

Observation 3: For DL fragment carriers that span larger than 50MHz, single FFT window at 122.88MHz, only SCS≧30KHz can be supported.

With above assumptions in observations 2 & 3, we try to evaluate RF performance when there exists in-gap interference applying single FFT together. The fc of interference was assumed in the middle of two FFT grid as worst-case simulation.

Figure 2. SCC: 5M, PCC: 5M + ACS single FFT simulated spectrum plot

Figure 3. SCC: 5M, PCC: 5M + IBB1 single FFT simulated spectrum plot

Figure 4. SCC: 5M, PCC: 5M + IBB2 single FFT simulated spectrum plot

It can be seen from simulated results, the spectrum of interference was spread out due to allocated on non-orthogonal FFT grid. UE would be able to pass ACS1/2 and IBB1 requirements without relaxation but would fail on IBB2. And due to wide frequency span for a single FFT, the baseband power consumption could be higher. It is expected single FFT may have worse rejection ratio to interference signal and this is not common baseband implementation for DL NCCA carriers.

Observation 4: For a single FFT baseband, UE would be able to pass ACS1/2 and IBB1 requirements without relaxation but would fail on IBB2 due to worse digital filter rejection on IBB2.

If UE apply separate FFT baseband, it is foreseen the digital filter can provide better rejection on the in-gap interference signal so that performance of wanted CCs would not be degraded except when the interference falls in the Rx image range of wanted signal. Separate FFT for DL NCCA carriers has been widely used for modern modem design.

Proposal 2: Separate FFT for DL NCCA carriers is assumed for the DL fragmented carrier performance evaluation. RAN4 further discuss RF requirements of the case in-gap interference falls in the Rx image range of wanted carrier

Issue 2-1-2: Study on power spectral density difference between carriers of co-located adjacent channel operators

In the scope of the SI, the power spectral density difference between carriers of co-located adjacent channel operators also need to be studied. Here we provide some system level simulation which follows coexistence frame work considering large-scale parameters only. The environment and configuration assumptions are as following bullets:

- Co-site case

- Same BW/Power/ANT # of 2 operators at n25 (1.9GHz)

    - 10 MHz(wanted)+10 MHz(unwanted) BW

- Care interference from the same site.

- Log RSRP difference between 2 operators à CDF

The detailed parameters are listed in the table below:

| Parameter | setting |
| --- | --- |
| Scenario | Dense Urban |
| Channel model | UMa |
| ISD | 200m |
| Number of operators | 2 |
| Number of sites | 7/operator, each with 3 sectors |
| Number of UEs | 10/sectors |
| Carrier frequency | 1.9GHz |
| Channel BW | 10MHz |
| Antenna pattern | AAS is assumed (same as that in coexistence study) |
| BS antenna configuration | 32 ANT# : (M,N,P) = (4,4,2) 128 ANT# : (M,N,P) = (8,8,2) |
| Note: The simulation follows coexistence framework, which considers large-scale parameters only. |  |

Table 1. Simulation parameters of PSD difference from UE reception point of view

Figure 5. Antenna patterns for the simulation

Figure 6. Simulated PSD difference at UE receiving antenna

From simulation result, we can see there’s no “interference level larger than wanted CC case” if the BW of both wanted CCs and unwanted CC(=Wgap) are same. The maximum ratio of unwanted CC level to wanted CC level is 0dB in this case. Other different BW/Wgap configurations can be scalable estimated that existing requirements at in-gap ACS1/ACS2/IBB1/IBB2/NBB can fulfil the need for coexistence. On the other hand, collocation inter-operator assumption might be too much simplified and different with real deployment. Some company propose to keep same ACS/IBB requirements and the proposal can be further considered for non-collocated BS cases. And even applying the new UE capability that RF parts can support higher order CA configurations with more CCs, the baseband processing capability may need to extend to handle higher order CA combos including more DL carriers as well as more MIMO-layer processing capability.

Observation 5: Collocation inter-operator assumption might be too much simplified and different with real deployment. Some company propose to keep same ACS/IBB requirements and the proposal can be further considered for non-collocated BS cases

Observation 6: Enabling DL fragmented carrier capability makes UE possible to support higher order configurations/MIMO layers from RF perspective. It does not guarantee baseband/modem has enough resources to support higher order configurations accordingly.

Proposal 3: existing requirements with interference at in-gap ACS1/ACS2/IBB1/IBB2/NBB can fulfil the need for coexistence for co-located adjacent channel operators

3. Conclusion

Issue 2-1-1: Reference architecture [and baseband assumptions]

- Reference architecture:

Observation 1: The existing DL NCCA requirements are based on partially-shared Rx architecture

Proposal 1: We propose focus on option 1 Only the “fully shared Rx chain architecture” needs to be studied for RF requirements evaluation. The option 2 (2Rx relaxation in mandatory 4Rx band below 2700MHz) can be discussed at later stage when signalling aspect starts to be discussed.

- Baseband assumptions

Observation 2: For DL fragmented carriers received by a single baseband with single FFT, channel raster of PCC and SCC shall align to orthogonal grid and this would need to be guaranteed by network scheduling.

Observation 3: For DL fragment carriers that span larger than 50MHz, single FFT window at 122.88MHz, only SCS≧30KHz can be supported.

Observation 4: For a single FFT baseband, UE would be able to pass ACS1/2 and IBB1 requirements without relaxation but would fail on IBB2 due to worse digital filter rejection on IBB2.

Proposal 2: Separate FFT for DL NCCA carriers is assumed for the DL fragmented carrier performance evaluation. RAN4 further discuss RF requirements of the case in-gap interference falls in the Rx image range of wanted carrier

Issue 2-1-2: Study on power spectral density difference between carriers of co-located adjacent channel operators

Observation 5: Collocation inter-operator assumption might be too much simplified and different with real deployment. Some company propose to keep same ACS/IBB requirements and the proposal can be further considered for non-collocated BS cases

Observation 6: Enabling DL fragmented carrier capability makes UE possible to support higher order configurations/MIMO layers from RF perspective. It does not guarantee baseband/modem has enough resources to support higher order configurations accordingly.

Proposal 3: existing requirements with interference at in-gap ACS1/ACS2/IBB1/IBB2/NBB can fulfil the need for coexistence for co-located adjacent channel operators

Reference:

- RP-242404 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#105

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2414457 WF on fragmented DL carriers study, RAN4#112