# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416139

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:			Ericsson

Title:			NR channel BW less than 5 MHz for FR1 Phase 2

Agenda item:		6.4.2

Document for:		Discussion

# Introduction

In the scope of the Rel-19 WI ([1]), CA/DC operation shall be supported for 3 MHz channel BW. RAN4 had discussion on this topic and a Way Forward ([2]) was agreed in last RAN4#112 meeting.

This contribution is discussing the remaining open issue and proposes a LS to be sent to RAN2.

# Discussion

## UE capability signaling

From the agreed WF ([2]) in last RAN4#112 meeting, the remaining open issue is related to the UE capability signaling, as captured below:

The main issue with the existing UE capabilities for 3MHz is the applicability restriction to single carrier operation. This one was added in the description of each new capability as highlighted in below figure, extract from TS 38.306:

This restriction was triggered by RAN4’s request based on the RAN LS Reply ([3]) which itself was answering RAN4’s request for clarification related to the scope of the WI on spectrum less than 5 MHz.

Still, it should be noted that RAN stated that the Rel-18 WI “should consider single-carrier operation”, and not “shall”. Removing this single carrier restriction to those UE capabilities would then not be against RAN response.

Based on the previous observations and further analysis, our understanding is that the current CA/DC procedures and signaling as specified today for NR would be sufficient to support CA/DC operation with less than 5 MHz channel BW carrier(s). CA/DC operation with 3 MHz channel BW will be specified by introducing a new BCS that includes 3 MHz channel BW in the list of channel BW for a band, as it’s usually done today for any other channel BW.  It’s not needed to introduce (nor extend the existing capabilities and signaling mechanism) any new capability for less than 5 MHz to enable CA/DC operation. We are then supporting option 2 of issue 2-2 from the agreed WF ([2]).

In our view, it would then be enough requesting RAN2 to remove this single carrier restriction from existing UE capabilities for less than 5 MHz, after RAN2 confirmed there is no backward compatibility issue.

On top of this, 3 MHz channel BW should be added as a possible value of the FeatureSetUplinkPerCC parameter supportedMinBandwidthUL-r17 for BCS5.

Proposal1: RAN4 should not suggest RAN2 to specify new nor extend existing UE capabilities and signaling mechanism to support for CA/DC operation with less than 5 MHz channel BW.

Proposal2: Send the draft LS proposed in Annex to RAN2 requesting to remove the single carrier restriction in existing UE capabilities for less than 5 MHz channel BW after RAN2 confirms there is no issue.

# Conclusion

In this contribution, we analyzed the impacts of considering the NTN Ku-band(s) as part of FR1-NTN or FR2-NTN. We made the following proposals:

Proposal1: RAN4 should not suggest RAN2 to specify new nor extend existing UE capabilities and signaling mechanism to support for CA/DC operation with less than 5 MHz channel BW.

Proposal2: Send the draft LS proposed in Annex to RAN2 requesting to remove the single carrier restriction in existing UE capabilities for less than 5 MHz channel BW after RAN2 confirms there is no issue.

# References

- RP-241622, WID on NR channel BW less than 5 MHz for FR1 Phase 2, Intel Corporation

- R4-2414451, WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2, Intel Corporation

- RP-230781, Response to LS from RAN4 on spectrum less than 5 MHz, RAN

# Draft LS to RAN2

# 3GPP TSG-RAN WG4 Meeting #112bis	R4-24xxxxx

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:	LS to RAN2 on UE capabilities for less than 5 MHz channel bandwidth

Response to:

Release:	Release 19

Work Item:	NR_FR1_lessthan_5MHz_BW_Ph2

Source:	TSG RAN WG4

To:	TSG RAN WG2

Cc:	TSG RAN WG1

Contact Person:

#### Name:		Dominique Everaere

- E-mail Address:	dominique.everaere@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

Under the scope of the Rel-19 “NR channel BW less than 5MHz for FR1 Phase 2” work item RAN4 has discussed how to enable CA/DC operation for carrier(s) with less than 5 MHz channel BW.

To RAN4 understanding, no new UE capability nor new signaling would be needed to enable such operation, the UE capabilities introduced in Rel-18 and the existing signaling would be sufficient without adding any extension.

Nevertheless, the following UE capabilties for less than 5 MHz channel BW would need to be updated, removing their applicability restriction to single carrier operation:

- support12PRB-CORESET0-r18,

- support3MHz-ChannelBW-Asymmetric-r18,

- support3MHz-ChannelBW-Symmetric-r18,

- support5MHz-ChannelBW-20PRB-CORESET0-r18,

- support12PRB-CORESET0-GSCN-41637-r18.

Also, 3 MHz channel BW should be added to the list of possible value for the FeatureSetUplinkPerCC parameter supportedMinBandwidthUL-r17 for BCS5

RAN4 kindly requests RAN2 to check that RAN4 understanding is correct and then udpate accordingly the UE capabilities listed above.

2. Actions:

To RAN WG2.

ACTION: 	RAN4 respectfully requests RAN2 to take into consideration the RAN4’s conclusion.

3. Date of Next TSG-RAN WG4 Meeting:

RAN4#113	from 2024-11-18	to 2024-11-22		Orlando, USA

RAN4#114	from 2025-02-17	to 2025-02-21		Athens, Greece