# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416140

# Hefei, Anhui, China, 14th – 18th October, 2024

Title:			NTN Ku-band – coexistence study

Agenda item:		6.9.2

Source:			Ericsson

Document for:		Discussion

# Introduction

In RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s). One of the WI objectives is to conduct a coexistence study, reusing as much as possible the assumptions of the coexistence study for the NTN Ka-band.

This contribution is presenting early results to progress on this topic based on assumptions in annex.

# Discussion

## WI objectives

The NTN Ku band WI lists the following objectives which are major input for this coexistence study:

When specifying new RF requirements for SAN/UE in a new frequency range, as it has been done for NTN and TN, RAN4 determines the SAN/UE ACLR/ACS requirements based on the outcomes of a coexistence study. Such study is assuming adjacent channel networks. RAN4 shall run such study in the context of this WI as it’s a new frequency range for which no NTN band has been specified so far. This is aligned with the second bullet of WI objectives.

Moreover, as USA should also be considered for the priority one of this WI, we should also take into account the potential US band in the 13GHz (12.7-13.25 GHz). This would be an additional reason to motivate coexistence study between adjacent network to determine ACLR/ACS requirements.

Proposal1: As done for any TN/NTN band previously, and as also mentioned in the WI objectives, RAN4 shall conduct a coexistence study assuming adjacent TN/NTN networks.

## Assumptions

The following assumptions were agreed ([4]) in last RAN4#112 meeting:

On top of those agreements, to run some preliminary coexistence simulations, we used the following assumptions based on offline discussion with Thales:

- Reuse Ka-band assumptions for shadow fading and clutter loss tables, and for the tropospheric scintillation.

- Use the following satellite parameters:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 31.4 dB K-1 | 11.4 dB K-1 | 11.4 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

- Use the following VSAT parameters:

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 10 W (40 dBm) |
| Tx antenna gain | 36.2 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

The complete list of assumptions is captures in Annex.

Proposal2: Adopt the simulation assumptions listed in Annex to perform the Ku-band coexistence study.

Note that the TN antenna parameters for 11/14 GHz should still be updated and aligned with the study on IMT parameters for ITU-R on 14.8-15.35 GHz.

## Preliminary results

Based on the assumptions listed in annex, we run some preliminary simulations summarized for urban macro only. The results are shown in below table.

| Case | Aggressor | Victim | Frequency | LEO600 | GEO | Resulting ACLR/ACS |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | VSAT UL | UE UL | 14 GHz |  | 11 (90o elev. only) | VSAT ACLR <15 |
| 2 | UE UL | VSAT UL | 14 GHz |  | 6 | SAN ACS <10 |
| 3 | VSAT UL | gNodeB DL | 14 GHz | <5 |  | VSAT ACLR <15 |
| 4 | gNodeB DL | VSAT UL | 14 GHz | TBD | TBD | SAN ACS |
| 5 | gNodeB DL | Satellite DL | 11 GHz | 56 | 53 | VSAT ACS |
| 6 | Satellite DL | gNodeB DL | 11 GHz | <5 | <5 | SAN ACLR ~12 |
| 7 | Satellite DL | UE UL | 11 GHz | <5 |  | SAN ACLR ~12 |
| 8 | UE UL | Satellite DL | 11 GHz | 12 |  | VSAT ACS |

Note that the values for 14 GHz are very rough estimation as the IMT study on 15GHz is not yet concluded.

As expected, the case 5 is the problematic one where the value for VSAT ACS should be high to enable coexistence with an adjacent TN. RAN4 should further discuss on how to handle this case, e.g. reconsidering the ACIR model when TN is not adjacent to NTN, …

To not delay the WI, we would make the following proposals.

Proposal3: Perform and conclude on coexistence simulations for all cases except case 5 and further discuss how to handle case 5 in parallel.

# Conclusion

In this contribution, we had further discussion on NTN Ku-band coexistence study and shared some very preliminary results. We made the following proposals and observations:

Proposal1: As done for any TN/NTN band previously, and as also mentioned in the WI objectives, RAN4 shall conduct a coexistence study assuming adjacent TN/NTN networks.

Proposal2: Adopt the simulation assumptions listed in Annex to perform the Ku-band coexistence study.

Proposal3: Perform and conclude on coexistence simulations for all cases except case 5 and further discuss how to handle case 5 in parallel.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- TR 38.921, Study on International Mobile Telecommunications (IMT) parameters for 6.425 - 7.025 GHz, 7.025 - 7.125 GHz and 10.0 - 10.5 GHz

- RP-240787, New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz, Ericsson.

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General

# Annex : Simulation assumptions

## Co-existence simulation scenario for 11/14GHz

Scenarios for the coexistence study for 11/14GHz are listed in Table 6a.1-1

Table 6a.1-1: Scenarios for NTN-NTN/TN co-existence

| 11/14GHz |  | NTN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Set 1 |  |  | Set 2 |  |  |
|  |  | GEO3 | LEO 600km | LEO 1200km | GEO | LEO 600km | LEO 1200km |
| NR | Rural | N/A | N/A | N/A | N/A | N/A | N/A |
|  | Urban macro | X | X | X | X | X | X |
|  | Dense Urban6 | N/A | N/A | N/A | N/A | N/A | N/A |

The aggressor and victim combinations for 11/14GHz cases are listed in Table 6a.1-2.

Table 6a.1-2: Aggressor and victim

| No | Combination | Aggressor | Victim | Frequency Band | Notes on the Ku Band | Scope of Coexistence Simulation |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | TN with NTN | VSAT UL | UE UL | 14 GHz | Ku Band VSAT uplink interfering with gNodeB receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the gNodeB is fixed. |
| 2 | TN with NTN | UE UL | VSAT UL | 14 GHz | UE uplink interfering with Ku Band Satellite receiver. | The ACLR of the UE is fixed. The ACS of the satellite to be varied/defined. |
| 3 | TN with NTN | VSAT UL | gNodeB DL | 14 GHz | Ku Band VSAT uplink interfering with UE receiver. | The ACLR of the VSAT to be varied/defined. The ACS of the UE is fixed. |
| 4 | TN with NTN | gNodeB DL | VSAT UL | 14 GHz | gNodeB downlink interfering with Ku Band Satellite receiver. | The ACLR of the gNodeB is fixed. The ACS of the Satellite to be varied/defined. |
| 5 | TN with NTN | gNodeB DL | Satellite DL | 11 GHz | gNodeB downlink interfering with Ku Band VSAT receiver. | The ACLR of the gNodeB is fixed. The ACS of the VSAT to be varied/defined. |
| 6 | TN with NTN | Satellite DL | gNodeB DL | 11 GHz | Ku Band Satellite downlink interfering with UE receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the UE is fixed. |
| 7 | TN with NTN | Satellite DL | UE UL | 11 GHz | Ku Band Satellite downlink interfering with gNodeB receiver. | The ACLR of the Satellite to be varied/defined. The ACS of the gNodeB is fixed. |
| 8 | TN with NTN | UE UL | Satellite DL | 11 GHz | UE uplink interfering with Ku Band VSAT receiver. | The ACLR of the UE is fixed. The ACS of the VSAT to be varied/defined. |
| NOTE 1: For coexistence between Ku Band DL and adjacent TN bands, there are no 3GPP defined/specified TN bands. |  |  |  |  |  |  |

The frequency and bandwidth are listed in Table 6a.1-3.

Table 6a.1-3: Proposed frequency and bandwidth for co-existence study

|  | Frequency | Bandwidth | Duplex mode | Frequency reuse factor |
| --- | --- | --- | --- | --- |
| TN Urban macro | 11/14 GHz | 100MHz | TDD | 1 |
| GEO | 11GHz (DL)/14GHz(UL) | 100MHz per beam | FDD | 2 |
| LEO | 11GHz (DL)/14GHz(UL) | 100MHz per beam | FDD | 2 |

## 6a.2	Co-existence simulation assumption for 11/14GHz

### 6a.2.1	Network layout model

#### 6a.2.1.1	Co-existence between NTN satellite and TN

Cellular cell structure is considered for both NTN and TN network layout.

Referring to TR 38.811 Section 6.3 and Annex A, a 3D global coordinate system is considered (Earth-Centred Earth Fixed) for simulating NTN beams direction and location on the earth surface. It means the NTN beam location, TN randomly dropping location are generated with a set of three parameters (x,y,z)

Deployment of NTN satellite and TN cells and UEs for co-existence study is listed in Table 6a.2.1.1-1.

Table 6a.2.1.1-1: Network and UE deployment

| No. | Combination | Aggressor | Victim | Which NTN cell/UE to observe? | Which TN/UE to observe? | Which TN cells in a TN to observe? |
| --- | --- | --- | --- | --- | --- | --- |
| 1a | TN with NTN | NTN UL | TN UL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 2a | TN with NTN | TN UL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider an active rate of 20% for TN | Only the active TN cells in central NTN beam |
| 3a | TN with NTN | NTN UL | TN DL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 4a | TN with NTN | TN DL | NTN UL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN. | All active TN cells in central NTN beam |
| 5a | TN with NTN | TN DL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters. | One cluster with 19 TN cells (57 sectors) randomly placed in the central NTN beam | Only the active TN clusters which contain the NTN UE(s) |
| 6a | TN with NTN | NTN DL | TN DL | NTN cell: Nadir point. NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | All in central NTN beam |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO, interested companies can bring analysis and results for other values) NTN UE: NTN UEs randomly dropped in TN clusters | TN clusters randomly placed in this NTN beam |  |
| 7a | TN with NTN | NTN DL | TN UL | NTN cell: Nadir point NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
|  |  |  |  | NTN cell: NTN cell with satellite at low elevation (to be further discussed for GEO and LEO，interested companies can bring analysis and results for other values). NTN UE: NTN UEs randomly dropped in TN clusters. | TN clusters randomly placed in this NTN beam | Only the active TN clusters which contain NTN UE(s) |
| 8a | TN with NTN | TN UL | NTN DL | NTN cell: Observe NTN central beam for SINR, 6 adjacent beams for inter-beam interference. NTN UE: NTN UEs randomly dropped in TN clusters | Consider the active rate of 20% for TN | All active TN cells in central NTN beam |

### 6a.2.2	System parameters

#### 6a.2.2.1	Satellite parameters

Set-1 satellite parameters are listed in Table 6a.2.2.1-2 according to TR 38.821[6].

The satellite max Tx power can be calculated by the equation as below:

maxTx powerdBm=EIRP densitydBWMHz+30+10 log10NRB*SCSMHz*12-Max GaindBi

Table 6a.2.2.1-1: NRB configuration per BandWidth size and SCS

| Configuration Ku-band | NRB (100MHz BW) |
| --- | --- |
| SCS 30 kHz | 273 |

Table 6a.2.2.1-2: Set-1 satellite parameters for co-existence study

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in [2] | Section 6.4.1 in [2] | Section 6.4.1 in [2] |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ku-band (i.e. 11 GHz for DL) | 9.1 m | 0.91 m | 0.91 m |
| Satellite EIRP density |  | 43 dBW/MHz | 13 dBW/MHz | 7 dBW/MHz |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.177 deg | 1.768 deg | 1.768 deg |
| Satellite beam diameter (Note 2) |  | 110 km | 37 km | 19 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 31.4 dB K-1 | 11.4 dB K-1 | 11.4 dB K-1 |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB. |  |  |  |  |

Table 6a.2.2.1-3: Other parameters for NTN satellite

| Parameters | NTN satellite | Remark |
| --- | --- | --- |
| Carrier frequency | 14GHz(UL) / 11GHz(DL) |  |
| The number of active UE (UL) | 10 UEs and 27RBs per UE for GEO and LEO |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | See Session 6a.2.6.2 |  |
| NTN satellite Noise Figure in dB | See Table 6a.2.2.1-4 |  |
| Handover margin | 3dB |  |

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 31.4 | 11.4 | 11.4 |
| NF (dB) | 3.5 | 3.5 | 3.5 |

#### 6a.2.2.2	NTN UE parameters

NTN UE parameters are given in Table 6a.2.2.2-1.

Table 6a.2.2.2-1: NTN UE characteristics for system level simulations

| Characteristics | VSAT (Note 2) |
| --- | --- |
| Frequency band | Ku-band (i.e. 11 GHz DL and 14 GHz UL) |
| Antenna type and configuration | Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter |
| Polarisation | Circular |
| Rx Antenna gain | 34.3 dBi |
| Antenna temperature | 150 K |
| Noise figure | 1.2 dB |
| Tx transmit power | 10 W (40 dBm) |
| Tx antenna gain | 36.2 dBi |
| NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information. NOTE 2: VSAT terminal characteristics could be implemented with phased array antenna |  |

The altitudes of NTN UE for different use case are defined in Table 6a.2.2.2-2.

Table 6a.2.2.2-3: NTN UE altitude

| Use case | Fixed VSAT (on roof) | M-ESIM | A-ESIM | L-ESIM |
| --- | --- | --- | --- | --- |
| Altitude | 22.5m | 22.5m | 3-14km | 1.5m |

#### 6a.2.2.3	Void

#### 6a.2.2.4	TN parameters

TN parameters for co-existence study are given in Table 6a.2.2.4-1 and Table 6a.2.2.4-2.

Table 6a.2.2.4-1: Simulation assumptions of TN NR

| Carrier frequency in GHz | 14 |
| --- | --- |
| Size of each nominal channel BW in MHz | 100 |
| Transmission bandwidth in MHz | 98.28 |
| Environment | Urban macro |
| Network layout | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around |
| Inter-site distance in meter | 300m |
| UE number per cell | Same as the number of BS beam |
| System loading and activity | 20% |
| Network location | See Table 6a.2.1.1-2 |
| DL subcarrier spacing | 120kHz |
| UL | OFDMA |
| DL power control | No |
| UL power control | See Section 6a.2.6.2 |
| Frequency reuse | 1 |
| Number of scheduled UE per cell (DL) | Same as the number of BS beam |
| Number of scheduled UE per cell (UL) | Same as the number of BS beam |
| BS height | 25 |
| UE antenna height in meter | 1.5 |
| UE TX power in dBm | -40 to 23 |
| Building penetration loss | In path loss model, TR 38.901 |
| Cell selection margin in dB | 3 |
| BS-MS min distance in meters | 35 |
| BS noise figure in dB | 10 |
| UE noise figure in dB | 10 |
| BS-UE path-loss model | TR 38.901 |
| Standard deviation of BS-UE log-normal shadow fading in dB | Deployment scenario related, referring to TR 38.901. |
| Shadowing correlation | Between cells: 1.0 Between sites: 0.5 |
| Link-level performance model | See Section 6a.2.9 |
| UE distribution | Uniform |
| Evaluation metrics | See Section 6a.2.8 |

Table 6a.2.2.4-2: ACLR/ACS for TN (11 GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | 37 dB |
|  | ACS | 40 dB |
| UE | ACLR | 24 dB |
|  | ACS | 31 dB |

Note: the following values should be aligned with the values agreed for 15GHz in the scope of the LS Reply to ITU-R SI FS_NR_IMT_4400_7125_14800MHz.

Table 6a.2.2.4-2a: ACLR/ACS for TN (14 GHz)

|  |  | NR |
| --- | --- | --- |
| BS | ACLR | TBD dB |
|  | ACS | TBD dB |
| UE | ACLR | TBD dB |
|  | ACS | TBD dB |

### 6a.2.3	Antenna and beam forming pattern modelling

#### 6a.2.3.1	Satellite parameters

In Rel-18, antenna pattern in section 6.4.1 of TR38.811 is reused for NTN SAN and NTN UE. Use of phased array antenna model needs further discussion.

The following normalized antenna gain pattern, corresponding to a typical reflector antenna with a circular aperture, is considered.

1                for θ=0

4J1kasinθπ180kasinθπ1802        for 0<θ≤90°

For UE, 4J1kasinπ2kasinπ22          for θ>90°

where:

-	J1(x) is the Bessel function of the first kind and first order with argument ‘x’;

-	ais the radius of the antenna's circular aperture;

-	k = 2f/c is the wave number;

-	f is the frequency of operation;

-	c is the speed of light in a vacuum and  is the angle measured from the bore sight of the antenna's main beam.

Note that ka equals to the number of wavelengths on the circumference of the aperture and is independent of the operating frequency. And the sin() function is in radian.

The beam layout definition for a single satellite simulation in Ka-Band is defined in Table 6a.2.3.1-1.

Table 6a.2.3.1-1: Beam layout definition for single satellite simulation

| Beam layout definition | Same with Table 6.2.3.1-1 |
| --- | --- |
| Number of beams | Same with Table 6.2.3.1-1 |
| UV plane illustration (extracted from [19]) | Same with Table 6.2.3.1-1 |
| UV plane convention | Same with Table 6.2.3.1-1 |
| Adjacent beam spacing on UV plane | Same with Table 6.2.3.1-1 |
| Central beam bore sight direction definition | Baseline: Case 1: Central beam center is considered at nadir point Case 2: 25° for GEO and LEO |
| Frequency-reuse factor | FRF=2 Do not consider interference leakage from adjacent beams as starting point for co-ex study. For example for the figure above, do not consider the interference leakage from green, blue and purple beams when study SINR for the red beam. |
| Polarization re-use | Enable Note: Polarization re-use should apply only if circular polarization for terminal antenna is considered |
| UEs outdoor/indoor distribution | Same with Table 6.2.3.1-1 |
| UE distribution | Same with Table 6.2.3.1-1 |
| UE configuration | Fixed and mobile VSAT for GSO Fixed VSAT for NGSO |
| UE orientation | Random Ideal Tracking serving beam |
| UE attachment | RSRP |

#### 6a.2.3.2	TN BS and UE antenna and beam forming pattern modelling

The parameters in Table 6a.2.3.2-1 are used for BS antenna pattern in the coexistence study.

Table 6a.2.3.2-1: TN BS antenna modelling for Urban macro scenario

Note that, to be consistent, the following parameters shall be aligned with the ones used for the study on IMT parameters for 15 GHz in the scope of the LS Reply to ITU-R SI FS_NR_IMT_4400_7125_14800MHz.

| Parameters | Values |
| --- | --- |
| Scenario | Urban macro |
| Antenna pattern | TR 38.803 |
| Element gain GE,max (dBi) | 5.5 |
| Horizontal 3dB /vertical 3dB beam width of single element | 90º for H 90º for V |
| Horizontal/vertical front-to-back ratio Am (dB) | 30 for both H/V |
| Side lobe suppression SLAv (dB) | 30 |
| Antenna polarization | Linear ±45º |
| Antenna array configuration (Row × Column) | 16 × 8 elements |
| Horizontal/Vertical radiating element spacing | dh = 0.5 λ dv = 0.5λ |
| Array Ohmic loss LE (dB) | 2 |
| Conducted power (before Ohmic loss) per antenna element (dBm) | 22 |
| Mechanical downtilt (degrees) | 10 |
| Antenna element vertical radiation pattern (dB) | AE,Vθ"=-min12θ"-90°θ3dB2, SLAV, θ3dB=90°， SLAV=30dB |
| Antenna element horizontal radiation pattern (dB) | AE,Hφ"=-min12φ"φ3dB2, Am, φ3dB=90°， Am=30dB |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5.5 dBi |
| (Mg, Ng, M, N, P) note | (1, 1, 8, 16, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| Note: An additional 3dB gain is added to the total beamforming gain to account for the two polarization directions. Boresight direction is horizontal. |  |

For TN UE, the antenna parameters refer to TR 38.803 and are defined in Table 6a.2.3.2-2.

Table 6a.2.3.2-2: TN UE antenna parameters

| Parameter | Values |
| --- | --- |
| Antenna element vertical radiation pattern (dB) |  |
| Antenna element horizontal radiation pattern (dB) |  |
| Combining method for 3D antenna element pattern (dB) |  |
| Maximum directional gain of an antenna element GE,max | 5 dBi |
| (Mg, Ng, M, N, P) | (1, 1, 2, 2, 2) |
| (dv, dh) | (0.5λ, 0.5λ) |
| UE orientation | Random orientation in the azimuth domain: uniformly distributed between -90 and 90 degrees* Fixed elevation: 90 degrees |
| NOTE: This is done to emulate two panels: the configuration is equivalent to 2 panels with 180 shift in horizontal orientation and UE orientation uniformly distributed in the azimuth domain between -180 and 180 degrees. |  |

### 6a.2.4	ACIR model

ACLR modelling for TN and NTN co-existence study referring to clause 5.1.1.4.1 and 5.1.1.4.2 in TR 36.942[8] is used as baseline. The number of RBs refers to Table 6a.2.2.1-3 and Table 6a.2.2.4-1 respectively.

### 6a.2.5	Propagation model

Propagation models between NTN and TN components used for the coexistence study are listed in Table 6a.2.5-1.

Table 6a.2.5-1: Propogation models between NTN and TN

| Link | Propagation model |
| --- | --- |
| TN BS to Fixed VSAT on roof | Free space path loss |
| TN BS to L-ESIM at 1.5 m | UMa as in 3GPP TR 38.803 |
| TN BS to TN UE | UMa as in 3GPP TR 38.803 |
| TN UE to Fixed VSAT on roof | UMa as in 3GPP TR 38.803 (BS is to be replaced with VSAT) |
| TN UE to L-ESIM | Umi |
| Satellite to TN BS/UE | 3GPP TR 38.821 |
| Satellite to VSAT/ESIM | 3GPP TR 38.821 |
| TN BS to Satellite | 3GPP TR 38.821 |
| Note1: For the propagation models which use the 3GPP TR 38.821 [3], to use same assumptions as in [3] and consider the atmospheric losses and the scintillation losses. Note2: TN BS height is 25m |  |

### 6a.2.6	Transmission power control model

#### 6a.2.6.1	UL TPC

Following UL power control model for both NTN and TN are used as starting point.

Where,

-	Pmax = maximum UE Tx power, i.e. 23 dBm for TN UE, and 33 dBm for fixed VSAT NTN UE;

-	Rmin = minimum power reduction ratio, i.e. -63 dB for TN UE, and [-63 dB by assuming NTN UE min Tx power as -30dBm as starting point] for NTN UE;

-	CLx-ile and γ are set as following:

-	γ = 1;

-	CLx-ile = –SNR_target + Pmax – Thermal Noise – BS/SAN_NoiseFigure - 10*log10(BW), considering SNR_target is 15dB and BW is actual UL BW.

#### 6a.2.6.2	DL TPC

For downlink scenario, no power control scheme is applied.

### 6a.2.7	Received power model

See Section 6.2.7.

### 6a.2.8	Performance metric

See Section 6.2.8.

### 6a.2.9	Throughput ~ SNR mapping

See Section 6.2.9. [Further discuss the applicability of the mapping for NTN link.]

## 6a.3	Co-existence simulation methodology for 11/14GHz

### 6a.3.1	Simulation procedure

See Section 6.3.1, noting the FRF and network deployment for NTN and TN in above 10GHz are specified accordingly.

Following approaches for the deployment of NTN UE and satellite are used:

-	The satellite should be generated in the visible area/sky of NTN UE;

-	NTN UEs point to the satellite accurately;

-	The position of the satellite should guarantee that NTN UE vertical angle towards the satellite;

-	For calibration, use 30 degree and 90 degree instead of the range;

-	SAN elevation angle value, 25 & 90 degree (computed from the centre of the beam), for co-existence -simulation;

-	Horizontal angle of NTN UEs should be calculated based on the satellite position;

-	Use 35m as minimum distance assumed between VSAT and TN BS for co-ex study.