# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2416141

# Hefei, Anhui, China, 14th – 18th October, 2024

Source:			Ericsson

Title:			NTN Ku-band – FR1-TN vs FR2-TN

Agenda item:		6.9.3

Document for:		Discussion

# Introduction

In RAN#104, a new WI ([1]) has been approved to specify new NTN Ku band(s). One of the WI objectives is to enable 125 MHz in UL and 250 MHz, considering either FR1-TN range or FR2-TN range.

This contribution is discussing the different approaches, based on the agreed WF ([2]) from last RAN4#112 meeting.

# Discussion

## WI objectives

In last RAN4#112 meeting, the following agreements ([2]) related to the FR choice for the new NTN Ku-band(s) were made:

First, it should be noted that 60kHz SCS was specified in RF specifications but, so far, it’s not supported in RRM, some requirements are missing for this SCS (e.g. timing error limit). If the NTN Ku-band(s) should be part of FR1-NTN, 60 kHz might be an interesting SCS value for such frequency range but this would require some RRM specifications update. The same comment is valid for demodulation requirements.

Observation1: 60kHz SCS is not supported for FR1-NTN in RRM specifications, neither in demodulation requirements.

From the above listed criteria to investigate, “UL timing synchronization” and “beam hopping granularity” should better be discussed in the RRM session, not in RF, with the relevant experts.

Observation2: To determine if the NTN Ku-band(s) should belong to FR1-NTN or FR2-NTN, some criteria (e.g. UL timing synchronization”) should better be discussed in RRM session, noting that RRM discussion should only start in RAN4#114 (Feb. 2025).

Phase noise increases with LO frequency but decreases with wider sub-carrier spacing. However, the subcarrier spacing’s choice has direct impact on CP length and Doppler shift management. RAN4 developed phase noise models for various frequency and for BS and UE ([3]). They could be used as starting point, but it would be better to get updates from satellite companies, also more adapted to the satellite industry. Nevertheless, phase noise should not have any major impact on the choice of FR1-NTN vs FR2-NTN.

Observation3: RAN4 developed phase noise models for TN BS and UE in the 10 and 15 GHz, updates from the satellite industry would be expected. Nevertheless, phase noise should not have any major impact on the FR1-NTN vs FR2-NTN choice.

The following table compares the spectrum utilization for the targeted aggregated bandwidth, using FR1 or FR2 channel bandwidth.

| Targeted aggregated channel BW | FR1 channel BW |  | FR2 channel BW |  |
| --- | --- | --- | --- | --- |
|  | Spectrum utilization (MHz) | BW Combination / SCS | Spectrum utilization (MHz) | BW Combination / SCS |
| 36 MHz | 33.84 | 35 MHz / 15kHz | NA | NA |
| 72 MHz | 68.04 | 70 MHz / 30kHz | 47.52 | 50 MHz / 60kHz |
| 250 MHz DL | 244.44 | 2*100 + 50 MHz / 30 kHz | 237.6 | 200 + 50 MHz / 60 kHz |
| 125 MHz UL | 121.68 | 100 + 25 MHz / 30 kHz | 95.172 | 100 MHz / 60 kHz |

Observation4: For the targeted aggregated channel BW in the Ku-band, the spectrum utilization would be higher combining FR1 channel BWs than combining FR2 channel BWs.

Observation5: If RAN4 decides that NTN KU-band(s) should be part of FR1-NTN, the following channel BWs should be added to TS 38.108: 25, 35, 50, 70 and 100 MHz.

Also, if NTN KU-band(s) should be part of FR1-NTN, it should be noted that, currently, the RF and demodulation requirements for VSAT type of UE are only specified for bands belonging to FR2-NTN. This would have some impacts on the effort to specify VSAT requirement for the Ku-band(s) then.

Observation6: RF and demodulation requirements are specified for NTN UE types 1-5 when operating in bands belonging to FR2-NTN.

# Conclusion

In this contribution, we analyzed the impacts of considering the NTN Ku-band(s) as part of FR1-NTN or FR2-NTN. We made the following proposals and observations:

Observation1: 60kHz SCS is not supported for FR1-NTN in RRM specifications, neither in demodulation requirements.

Observation2: To determine if the NTN Ku-band(s) should belong to FR1-NTN or FR2-NTN, some criteria (e.g. UL timing synchronization”) should better be discussed in RRM session, noting that RRM discussion should only start in RAN4#114 (Feb. 2025).

Observation3: RAN4 developed phase noise models for TN BS and UE in the 10 and 15 GHz, updates from the satellite industry would be expected. Nevertheless, phase noise should not have any major impact on the FR1-NTN vs FR2-NTN choice.

Observation4: For the targeted aggregated channel BW in the Ku-band, the spectrum utilization would be higher combining FR1 channel BWs than combining FR2 channel BWs.

Observation5: If RAN4 decides that NTN KU-band(s) should be part of FR1-NTN, the following channel BWs should be added to TS 38.108: 25, 35, 50, 70 and 100 MHz.

Observation6: RF and demodulation requirements are specified for NTN UE types 1-5 when operating in bands belonging to FR2-NTN.

# References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General

- R4-1911831, Discussion on parameterized phase noise model for example frequencies in 7-24 GHz, Ericsson