# 3GPP TSG-RAN WG4 Meeting # 112bis      													R4-2416150

# Hefei, China, 14-18 October, 2024

Title:					NR band n87-n88- Regulatory aspects

Agenda item:		5.15.1

Source:				Ericsson

Document for:		Discussion

# 1	Background

In RAN#104, a new WI has been approved to specify the new NR bands n87 and n88, re-farmed from LTE bands 87 and 88.

One open issue captured in the Way Forward ([2]) from last RAN4#112 meeting is related to concerns on the regulatory aspects for those 2 bands. This contribution is discussing this aspect.

# 2 	Discussion

## 2.1	Background

In last RAN4#112 meeting, RAN4 agreed to check that the regulatory requirements from ECC Decisions and corresponding ECC Report were properly captured when LTE bands 87 and 88 were specified. The following open issue was then captured in the agreed WF:

## 2.2 	TR 36.762

TR 36.762 captured the technical analysis made by RAN4 when introducing bands in the 410-430 MHz frequency range for LTE PPDR and PAMR in Europe.

In the references of this TR, the following ECC Decisions are listed:

- ECC Decision (16)02 with the version amended on 8 March 2019.

- ECC Decision (19)02 which was approved 8 March 2019.

Those 2 ECC Decision versions are the last one published by ECO. The analysis done by RAN4 was then made on the latest version of ECC Decision(16)02 and ECC Decision(19)02.

## 2.3 	ECC Decision(16)02

According to ECC Decision(16)02, decides 3 c) and 3 e), the LRTCs specify in Annex 3 shall apply, those being derived from ECC Report 283.

In Annex 3, UE requirements are specified in tables 6 and 7, which are also captured in TR 36.762, tables 4.2-1 and 4.2-2. Those corresponds to the UE Maximum output power and UE SEM as specified in TS 36.101 tables 6.2.2-1 and 6.6.2.1.1-1.

In Annex 3, BS requirements are specified in table 8 which corresponds to OBUE requirements specified in TS 38.104 for 1.4, 3 and 5 MHz channel BW.

## 2.3 	ECC Decision 19(02)

The same technical conditions as ECC Decision(16)02 for BS and UE are specified in Annex 2.

The spurious emission limits specified in A2.3.1 are those specified in TS 38.104 for any FR1 band in CEPT region.

The major difference (related to 3GPP RATs) with ECC Decision (16)02 is the consideration of NB-IoT:

- The Foffset definition (200kHz) is aligned with TS 36.104.

- The out of band emissions limits (table 18) for NB-IoT standalone are those specified in TS 36.104.

The only difference is how NB-IoT in guard band operation is specified. According to A4.2, NB-IoT operating in guard band should be compliant with the NB-IoT standalone requirements (table 16 and 18) while, like when operating in band, the requirements for the “hosting” LTE/NR carrier should apply.

Anyway, this has no impact on n87 and n88 as the wider channel BW supported in those is 5 MHz, and NB-IoT operating in guard band is not supported in a 5 MHz channel BW signal.

To conclude:

Observation: The Technical Conditions specified in ECC Decision(16) and ECC Decision(19)02 have been taken into account when LTE bands 87 and 88 were specified. No additional requirement is expected for bands n87 and n88.

# 3	Conclusion

In this contribution, we reviewed ECC Decision(16)02 and ECC Decision(19)02 and made the following observation:

Observation: The Technical Conditions specified in ECC Decision(16) and ECC Decision(19)02 have been taken into account when LTE bands 87 and 88 were specified. No additional requirement is expected for bands n87 and n88.

# 4	References

- RP-241663, New WID on introduction of NR bands n87 and n88, Nokia

- R4-2414272, WF on introduction of NR bands n87 and n88, Nokia