3GPP TSG RAN WG4 Meeting #112bis		 R4-2416164

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.6.3

Source: 	MediaTek Inc., Telstra

Title: 	Discussion of impacts on UE RF requirements and DL performance impacts

Document for:	Approval

1. Introduction

It had been discussed in previous RAN4 meeting and WF[4] was agreed. Open issues were also captured in the appendix of the WF. In this meeting, we continue the discussion and provide our views and discussion points on UE RF requirements and DL performance impacts.

2. Discussion

2.1 Discussion on assumptions on test configurations for evaluation

In the scope of the study, it was assumed “up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs”. We have been provided some observation on existing REFSENS requirements[5] and had following observations:

Observation 1: For FDD bands, the REFSENS level over different DL CBWs are usually not equal PSD duel to interference from self-band uplink transmission

Observation 2: Equal PSD may apply for BS Tx perspective for single band DL transmission non-contiguous carriers

In last RAN4 meeting, companies also provide their views and a recommended WF was come out. We propose to agree on the recommended WF proposal for the assumptions of test configurations:

Proposal 1: We propose evaluation configuration start from those defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 for performance evaluation, for ΔRIBNC, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example bands

- Assumptions for UL transmission: for NR PC3 PA i.e., ACPR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

- Reuse the Rel-15 assumption of Rx chain image rejection, which is 25dB

- At this stage RAN4 does not take DL MIMO into account

- Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc.

Other configurations are not precluded if identified as having performance impact

2.2 Example band consideration for evaluation

The example bands for the SI were still put in square bracket and meanwhile we still receive more requirements on adding example bands to be supported for the study item. One of the required bands is band n26 for the reason of frequency range coverage. There are mid-bands and high-bands included but there’s no sub-1GHz band in the example band list. We agree the sub-1GHz band shall also be considered and would propose to include band n26 with above justification as well as required by operator.

Proposal 2: We propose to include band n26 to be one of example bands

    - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

      - Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example bands.

        - Provide the details related to gap between fragmented DL blocks]

Proposal 3: In order to evaluate impacts on UE DL performance for the study, company may select preferred example band to evaluate impacts on UE RF requirements and DL performance

2.3 Inspection of impacts on DL Rx requirements

In our view, n25 is a good example band that we would like to start the study with. With above proposal 1 & 2, let’s look at the three test configurations in existing requirements. The Rx architecture assumption of existing requirements is based on partially-shared Rx chains. It is foreseen both CCs may have good performance w.r.t ACS/IBB/NBB as well as rejection to self-band UL transmission interference. If the Rx chain is shared by the two non-contiguous CCs, (fully-shared Rx chain), the channel selection filter would need to be adjusted to wider bandwidth to accommodate the two DL carriers including Wgap between the two CCs. The Tx rejection of shared Rx chain would be much worse. Not only SCC would still impacted by self-band UL Tx, PCC may also be impacted due to worse filter rejection ratio toward UL Tx transmission. It is foreseen the DL AGC would need to be adjusted to tolerate larger Tx leakage portion thus the gain setting would become lower that brings noise level raised up for both PCC and SCC. Another potential degradation may happen on in-gap ACS/IBB/NBB performance, these requirements would also need to be evaluated.

Figure 1. Rx requirements to be evaluated

Figure 2. Illustration on the two BCS for DL NCCA test

Observation 3: Not only SCC is impacted by self-band UL Tx, PCC may also be impacted by it due to channel selection filter would be adjusted to wider total bandwidth in fully-shared Rx chain architecture and RX AGC would be adjusted for larger Tx leakage portion. The Rx noise level would be raised up for both PCC and SCC

Proposal 4: Impacts on DL performance shall also consider degradation on PCC due to wider channel selection filter configured under fully-shared Rx chain architecture

Let’s take an initial inspection on the potential performance impact with the assumptions in proposal 1. It can be seen for example, a DL NCCA composed of 5+20MHz bandwidth combination, when testing in-gap ACS/IBB1/IBB2 on CC0, if BW of CC0 is larger than CC1, the image of in-gap interference may also fall into co-channel of CC0 that result in performance degradation as illustrated in Figure 3. RAN4 need further discuss on how to handle the performance degradation.

Proposal 5: RAN4 discuss on the in-gap ACS/IBB performance impact due to Rx image assumption

On the other hand, when measuring ACS/IBB1/IBB2 of CC0, the image of interference may fall into the co-channel range of CC1. Though this may not be an issue for the conformance test since the conformance test does measure REFSENS of CC1 when testing ACS/IBB1/IBB2 of CC0, this may be an implementation issue in the field as illustrated in Figure 4.

Figure 3. Illustration on impacts of the unwanted/interference image fall into co-channel CC

Figure 4. Illustration on impacts of the unwanted/interference image fall into the other CC

3. Conclusion

2.1 Discussion on assumptions on test configurations for evaluation

Observation 1: For FDD bands, the REFSENS level over different DL CBWs are usually not equal PSD duel to interference from self-band uplink transmission

Observation 2: Equal PSD may apply for BS Tx perspective for single band DL transmission non-contiguous carriers

Proposal 1: We propose evaluation configuration start from those defined in Table 7.3A.2.2-1 and Table 7.3A.2.2-2 of TS 38.101-1 for performance evaluation, for ΔRIBNC, both out-of-gap and in-gap ACS, IBB and NBB requirements for selected example bands

- Assumptions for UL transmission: for NR PC3 PA i.e., ACPR=30dB, with MPR=1dB, Full RB allocation. Tx LO leakage and image rejection ratio are 28dB

- Reuse the Rel-15 assumption of Rx chain image rejection, which is 25dB

- At this stage RAN4 does not take DL MIMO into account

- Evaluation may consider impairment factors such as ACLR, phase noise or IMDx of aggressor ability, duplexer isolation…etc.

Other configurations are not precluded if identified as having performance impact

2.2 Example band consideration for evaluation

Proposal 2: We propose to include band n26 to be one of example bands

    - [The scope should be those bands, where the fragments are fully confined within 100MHz, which includes all FDD/SDL bands and TDD bands

      - Consider n2/n25, n3, n7, n66, n41, n39, n26 as the example bands.

        - Provide the details related to gap between fragmented DL blocks]

Proposal 3: In order to evaluate impacts on UE DL performance for the study, company may select preferred example band to evaluate impacts on UE RF requirements and DL performance

2.3 Inspection of impacts on DL Rx requirements

Observation 3: Not only SCC is impacted by self-band UL Tx, PCC may also be impacted by it due to channel selection filter would be adjusted to wider total bandwidth in fully-shared Rx chain architecture and RX AGC would be adjusted for larger Tx leakage portion. The Rx noise level would be raised up for both PCC and SCC

Proposal 4: Impacts on DL performance shall also consider degradation on PCC due to wider channel selection filter configured under fully-shared Rx chain architecture

Proposal 5: RAN4 discuss on the in-gap ACS/IBB performance impact due to Rx image assumption

Reference:

- RP-242404 New SID: Study on NR FR1 DL Fragmented Carriers, RANP#105

- RP-233374, “Fragmented carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2402309: “Study on effective utilization of fragmented FR1 carriers in the DL”, TELUS, Bell Mobility, Telstra, Nokia, Nokia Shanghai Bell, AT&T, US Cellular

- R4-2414457 WF on fragmented DL carriers study, RAN4#112

- R4-2411556 Discussion on impact on requirements, RAN4#112