3GPP TSG-RAN WG4 Meeting #112 bis	R4-2416166

# Hefei, China, 14th – 18th October 2024

Agenda Item:	6.24.2.2

Source: 	Ericsson

Title:	On general RRM aspects for Rel-19 conditional Intra-CU LTM

Document for:	Discussion

# Introduction

After RAN plenary in September , the following objective has been agreed in the WID [1] for Conditional LTM:

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

  - Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios and if so, to which cases. RAN WG work to not start before this checkpoint

In this contribution, we will address several aspects which are related for RRM requirements for conditional LTM

# Discussion

## Scope of the RRM requirement for conditional LTM

Intra-CU Conditional LTM is an extension of the LTM feature specified in Rel-18. The Rel-19 enhancement aiming at specify the UE evaluated conditions for triggering LTM. The Rel-19 scope is also limited in specifying the conditional intra-CU LTM to the scenario where the UE is in non-DC scenario.

Rel-18 LTM is specified for both NR SA and NR-DC scenario. In Rel-18, RAN4 defined following requirements for LTM.

- PDCCH order based RACH requirements

- TCI state activation requirements

- Cell switch delay requirements

- L1-RSRP measurement requirements for neighbouring cell

RAN2 is the leading WG CLTM and RAN2 starts discussing CLTM in October meeting. Based on the conditional LTM framework designed by RAN2, RAN4 needs to study and specify the requirements for the procedures introduced by RAN2. As per the WID currently only NR SA is considered for the CLTM. Checkpoint at RAN#107 to review the objective on whether Intra-CU conditional LTM can be specified to DC scenarios. Hence, Rel-19 CLTM can use the Rel-18 LTM NR SA scenario as starting point.

The scenarios specified for Rel-18 SA LTM are listed based on TS 38.133 clause 6.3.1

The requirements in this clause are applicable to SA for the following scenarios: PCell switch to a neighboring LTM candidate cell - FR1 cell to FR1 cell - FR1 cell to FR2 cell - FR2 cell to FR2 cell - FR2 cell to FR1 cell PCell switch to an LTM candidate cell that is a serving SCell in MCG - FR1 cell to FR1 cell - FR2 cell to FR2 cell

- Proposal 1 : Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM.

## Conditions for triggering and executing the LTM

For Rel-18 LTM, the processing time has relation to the number of maximum candidates that can be supported for early ASN.1 decoding based on UE capability.

For conditional LTM, the maximum number of candidates that can be configured will be discussed in RAN2. In Rel-18, RAN4 introduced a new UE capability for early ASN.1 decoding or fast RRC processing. If UE supports this capability, upon TCI state activation or PDCCH order based RACH trigger, UE can decode the ASN.1 for those cells and the RRC processing time for early ASN.1 is 0ms for these cells. The early decoding capability shall be supported for Rel-19 conditional LTM, the detail numbers can be further discussed upon RAN1/RAN2 agreements.

- Proposal 2: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

Another issue is about the early DL and UL synchronization procedure for conditional LTM. The Rel-18 LTM procedure follows the network controlled early DL and UL sync procedure.

Regarding how the framework for early DL/UL sync will be specified for Rel-19 conditional LTM, RAN4 can wait for RAN2 further agreements.

- Proposal 3: Rel-19 conditional LTM early DL/UL sync can wait for RAN2 further agreement on the framework.

## Conditional LTM measurement requirements

The Rel-18 LTM, L1-RSRP measurements requirements on neighbouring cell was introduced. Further, L1 measurement-based event-triggered reporting is currently under discussion for another objective within the Rel-19 Mobility enhancements. As the conditional LTM is expected to execute the cell switch upon an event entering condition, RAN4 needs to define the measurement delay for the event triggered measurements which execute the conditional LTM. Since this is the first meeting for the CLTM in RAN2 also, we suggest waiting for further RAN2 progress.

For the event triggered measurement reporting, there are currently 4 events for LTM that have been agreed, however which events will be applicable for conditional LTM has to be discussed and decided in RAN2.

- Proposal 4:  RAN4 to wait for RAN2 progress for defining the CLTM measurement requirements.

## Conditional LTM cell switch delay requirements

Though RAN2 did not start the discussion of the framework for CLTM configuration and execution, based on the CHO and LTM framework, we expect that CLTM procedure might include:

- UE receive of the conditional LTM configuration

- UE measure and evaluate the conditions

- UE receive TCI state activation

- UE establishes DL/UL early sync with the Network

- UE execute LTM and cell switch.

As RAN2 did not agree on the flow of the CLTM execution, based on the flow of the steps, cell switch delay consists of different delay components. Hence, we think it is too early to discuss the cell switch delay requirements for CLTM.

- Proposal 5: RAN4 to wait for RAN2 progress for defining cell switch delay requirements for conditional LTM.

# Summary

The following have been proposed in the current contribution:

	Proposal 1 : Rel-19 CLTM requirements are applicable for NR SA scenarios defined in Rel-18 LTM.

	Proposal 2: Rel-18 UE capability for early ASN.1 decoding or fast RRC processing shall be supported for Rel-19 Conditional LTM.

	Proposal 3: Rel-19 conditional LTM early DL/UL sync can wait for RAN2 further agreement on the framework.

	Proposal 4:  RAN4 to wait for RAN2 progress for defining the CLTM measurement requirements.

	Proposal 5: RAN4 to wait for RAN2 progress for defining cell switch delay requirements for conditional LTM.

# References

- [1] RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom, 3GPP TSG RAN #105, Melbourne, Australia, Sep 9-12, 2024.