3GPP TSG-RAN WG4 Meeting #112 bis	R4-2416167

Hefei, China, 14th – 18th, October 2024

Agenda Item:	6.20.3.

Source: 	Ericsson

Title:	Discussion on RRM core requirements and simulation assumptions for Rel-19 Sub-band full duplex (SBFD)

Document for:	Discussion

# Introduction

The agreements on the SBFD CLI handling and SBFD operation during RAN4#112 meeting are captured in the approved WF [1].

In this paper we provide our view on the listed open issues on RRM core requirement within related WID [2] objectives:

- Specify applicable RRM core requirements for CLI handling mechanisms [RAN4]

- Specify other RRM core requirements for SBFD operation, if identified [RAN4]

# Discussion

## RRM impacts of UE-to-UE CLI handling

### Measurement quantities and Requirement baseline

#### Issue 2-1-2: Measurement quantities Recommended WF - RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. This can be revisited based on further RAN1 agreement. - FFS if other measurement quantities to be considered in the core requirements based on further RAN1 agreement. #### Issue 2-1-3: Baseline for defining requirements Recommended WF - Both R16 CLI measurement requirements and L1-RSRP measurement requirements are to be considered for defining requirements for L1 based UE-to-UE CLI measurement. - RAN4 can directly discuss the requirements for L1 based UE-to-UE CLI measurement in next meeting. Issue 2-1-4: Measurement methods Recommended WF • RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement. • Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements.

Current RAN1 agreement and working assumption is that two new measurement quantities will be specified for Rel-19 SBFD CLI handling mechanism as follows:

- L1-CLI-RSSI

- L1-SRS-RSRP

The RSSI is defined as the linear average of the total received power observed in the configured OFDM symbols and measurement bandwidth from all sources. These sources including co-channel serving and non-serving cells, adjacent channel interferences and thermal noises.

The main difference between Rel-16 CLI-RSSI measurements and Rel-19 L1-CLI-RSSI measurements are

- Whether there is L3 filtering involved

- Where the RSSI RS is located

Here the L3 filtering might not have strong impact on the measurement granularity of the RSSI since it is total received linear average power from all sources existed, where the reference symbol is located may have higher impact.

Based on latest RAN1 agreement, the Rel-19 RSSI RS location can be within DL subband, or UL subband due to the redefinition of active DL BWP.

RAN1#118 agreement: For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - FFS: Method#3: UE measures RSSI within UL subband

For Rel-16 CLI measurements the RS location as defined in TS 38.133

TS 38.133 Clause 9.7.1 CLI measurements are only applicable for RRC_CONNECTED intra-frequency: - when SRS-RSRP measurement resource is fully confined within BW of DL active BWP - when CLI-RSSI measurement resource is configured within active BWP

To our interpretation for the legacy definition, the RSSI-RS location within the active BWP can be regarded as the combination of the DL subband and UL subband.

For Rel-19, currently RAN1 discussion on how the measurement resource shall be configured and where the RSSI measurement resource can be configured is still on-going. If the RS is being configured with DL subband, the dominant interference may not only be UE-to-UE CLI.

- Observation 1: For RSSI measurement Method#1 and Method # 3 may result in different measurement accuracy.

- Proposal 1: For L1-CLI-RSSI the Rel-16 CLI-RSSI requirement framework can be used as a baseline. Further updates can be made when RAN1 finalized the measurement RS configuration and method.

#### Issue 2-1-6: Measurement resources Recommended WF - RAN4 to wait for RAN1 conclusion on measurement resources to discuss whether is an impact on the requirements. #### Issue 2-1-7: Side condition Options for further discussion and down-selection (for information purpose): - Time offset between DL timing and SRS arrival timing. - Option 1: Rel-16 CLI SRS RSRP assumption - Option 2: Remove cell phase error from option 1 - Other options are not precluded, pending on RAN1 progress. - SRS Es/Iot, SRS configuration and maximum/minimum SRS-RSRP - Option 1: Rel-16 CLI SRS RSRP assumption - Other options are not precluded.

As defined in TS38.215, the RSSI measurement is the linear average of total received power (in [W]) over the configured OFDM symbols, measurement time resources, and bandwidth, including all sources such as co-channel, non-serving cells, adjacent interference, and thermal noise. However, simulations are limited in modeling interference sources compared to real deployments, reducing their relevance for discussion.

In contrast, L1-SRS-RSRP can be effectively evaluated in simulations by modeling both victim and aggressor UEs. Technical parameters such as timing error, SRS Es/Ioc, and measurement samples can be assessed during these simulations.

- Proposal 2: Rel-19 SBFD L1-CLI measurement requirement simulation shall focus on L1-SRS-RSRP measurement accuracy evaluation.

For L1-SRS-RSRP, there is only method#2 being agreed in RAN1. The measurement requirement needs to consider several main attributors:

- SRS configuration

- Measurement timing error

- Measurement period

- Measurement samples for L1 averaging

- SRS Es/Ioc

We have provided our detailed view on above aspects in Appendix in chapter 4 to propose how to move forward with simulation evaluation for setting L1-SRS-RSRP measurement requirements.

We would like to emphasize the Rel-19 SBFD UE-to-UE CLI RRM requirement scenario based on the revised WID [1]

- Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

- Proposal 3: For L1-SRS-RSRP, new measurement requirements shall focus on Intra-cell UE-to-UE CLI scenario only.

### Measurement Reporting

#### Issue 2-1-9: Measurement reporting Recommended WF: - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement - FFS whether L1-RSRP measurement reporting requirement can be re-used. #### Issue 2-1-13: Report mapping Recommended WF - FFS whether RAN4 needs to discuss report mapping for L1 based UE-to-UE CLI measurement. - FFS whether R16 report mapping can be re-used.

The RAN1 #117 agreement narrows down measurement reporting to aperiodic, while considering periodic and semi-persistent reporting. It's important to note that the previous RAN1 #116 bis agreement aimed to integrate CLI measurement into the CSI measurement framework.

This differs from the Rel-16 CLI reporting, which supports periodic, event-triggered, and event-triggered periodic reporting. Therefore, new requirements must be specified by RAN4 to support at least aperiodic reporting.

We believe the existing L1-RSRP aperiodic measurement reporting requirements can serve as a baseline.

RAN1#116 bis agreement on Measurement reporting - CSI measurement procedure integrating CLI measurement. RAN1#117 agreement on Measurement reporting - Aperiodic reporting Note: Periodic and semi-persistent reporting can be considered

- Proposal 4: For L1 CLI measurement reporting, the existing L1-RSRP aperiodic reporting requirement can be re-used as a baseline.

The L1-RSRP measurement reporting requirements are defined alongside L3-RSRP, with a resolution of 1 dB. Additionally, the reporting range for SS-RSRP and CSI-RSRP differentiates L1 reporting from L3 reporting, defined from 0 dB to -30 dB with a resolution of 2 dB.

The existing Rel-16 L3 CLI SRS-RSRP is specified within the range of -140 dBm to -44 dBm, with a resolution of 1 dB, accompanied by a specific mapping table that mirrors L1-RSRP, with a different range as outlined in TS 38.133, clause 10.1.22.1.2.

Furthermore, Rel-16 SRS-RSSI is defined within the range of -100 dBm to -25 dBm, with a resolution of 1 dB, as specified in TS 38.133, clause 10.1.22.2.2.

- Proposal 5: For measurement report mapping the Rel-19 CLI measurement reporting should be based on the simulation outcome.

- Proposal 6: No differential SS-RSRP for L1 reporting and L3 reporting is needed.

### Scheduling availability

#### Issue 2-1-11: Scheduling and measurement restriction Recommended WF - RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement if it is needed. - FFS the impact of measurement methods and Rx beam assumption.

Our working assumption is that the victim UE may utilize a different timing than the DL reference timing of the serving cell while the aggressor UE transmits the SRS within the timing advance (TA) when both UEs are operating within the same cell.

Given that the SBFD UE supports only half-duplex operation, it is not feasible to transmit UL during CLI measurements.

Since the SBFD UE only support half-duplex, it is not possible to transmit UL during the CLI measurement.

For L1-SRS-RSRP, currently only measurement methods#2 is supported from RAN1, the victim UE will measure the aggressor UE within the UL subband.

Based on the latest RAN1 #117 agreements:

- Measurement resources - Periodic, semi-persistent, or aperiodic measurement resource (set) i.e., SRS-RSRP resource or CLI-RSSI resource - Note: The measurement resources for SRS-RSRP are based on the existing legacy RS patterns - Rx beams configuration for UE-to-UE CLI measurement (if spatial domain coordination is supported)

The existing legacy RS patterns are expected for Rel-19, the L1-SRS-RSRP will have same impact to the DL reception in comparing with the Rel-16 for at least FR1.

Regarding L1-CLI-RSSI, with two measurement methods available from RAN1, the location of the RS may affect scheduling availability differently. Therefore, we recommend awaiting RAN1's progress on the conclusion of the measurement methods.

Based on the RAN1 #117 agreement for FR2, the RX beam configuration may be specified, also potentially UE capability will be introduced, the scheduling restriction requirements can wait for RAN1 progress.

If spatial domain-based schemes are supported for UE-to-UE CLI handling for FR2, the following are recommended to be specified - Rx beams configuration for UE-to-UE CLI measurement - Note: The above is subject to a separate optional UE capability.

- Proposal 7: For L1-SRS-RSRP, the Rel-16 scheduling availability for FR1 can be reused as a baseline.

- Proposal 8: The FR2 L1-SRS-RSRP and FR1 and FR2 L1-CLI-RSSI scheduling availability requirements can wait for RAN1 progress.

### RX beam configuration impact

#### Issue 2-1-5: Rx beam Recommended WF - RAN4 to wait for RAN1 conclusion on the Rx beam for L1 based UE-to-UE CLI measurement. Meanwhile, RAN4 to discuss the impacts of Rx beam configuration/determination on the requirements.

The current Rel-16 CLI assumes CLI measurement resources are QCL-ed with Type D to the latest PDSCH or CORESET, meaning the UE uses the same RX beam as the latest PDSCH or PDCCH. One can view this as an implicit indication of QCL Type D.

RAN1 is discussing whether to explicitly configure a CLI measurement resource with a TCI state (with QCL Type-D) to indicate the RX beam that the UE can use for CLI measurement. With this explicit configuration, some companies have suggested that the network can configure multiple such resources, each with a different TCI state. Then, if the network triggers a measurement on each resource at a different time, the collection of measurement reports from all resources can provide the network a view of which direction is the source of potential CLI for the UE.

We doubt that such “multiple direction probing” is actually useful, since by the time the network receives all CLI reports, the CLI situation may have changed, and it is difficult for the network to take actions to mitigate the CLI. In other words, measurement and reporting delays associated with such an approach undermines the original intention of achieving shorter delays by introduction of L1 UE-UE CLI measurement/reporting compared to existing L3 measurement/reporting. For this reason, we think the current approach of Rx beam indication based on the latest PDSCH or CORESET is sufficient.

For RAN4, it is reasonable to start with the Rel-16 assumption for L1-SRS-RSRP and L1-CLI-RSSI measurement requirement, as the RX beam configuration is tied to separate UE capability agreements in RAN1.

- Proposal 9: RAN4 can initiate discussions on the measurement requirements for L1-SRS-RSRP and L1-CLI-RSSI based on the Rel-16 legacy QCL assumption.

- Proposal 10: Further requirements can be introduced if RAN1 agrees on further UE capability for spatial domain CLI mechanism and RX beam configuration in FR2.

## Other RRM core requirements for SBFD operation

### Impact on SSB based measurements

#### Issue 2-3-2: Requirements for SSB based measurement Recommended WF RAN4 further discuss impact on SSB based neighbor cell measurement in RAN4. Issue 2-3-4: Requirements for scheduling restriction Recommended WF • RAN4 to discuss whether scheduling restriction requirements are impacted due to SBFD operation based on RAN1 agreements.

Based on the RAN1#117 agreements, SSB reception from the serving cell will be prioritized over any UL transmission when there is a collision. However, based on the RAN1#117 agreement, it is unclear whether this agreement applies only to SSB reception on the serving cell or whether it also applies to SSB reception of the neighboring cell(s). Thus, we suggest waiting for further RAN1 conclusion.

Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission. - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs

- Proposal 11: For SSB based L3 measurement on neighbor cell collision with UL transmission, RAN4 to wait for RAN1 progress.

For SSB-based L1 measurements, we recognize a potential issue arising from the absence of a guard period during the DL to UL transition for UL transmissions within the SBFD symbol. Specifically, if the SSB is configured in the DL symbol immediately before or after the UL transmission within the SBFD symbol, the UL transmission may partially collide with the SSB due to DL propagation delay and the UE switching delay.

The RAN1 #116 agreement specifically stated a note to cover this scenario.

Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Our interpretation of the RAN1 agreement is that SSB (DL reception) is prioritized over UL transmission, both within the same symbol and across different symbols. If a symbol containing the SSB collides with a UL transmission, all such symbols should be prioritized accordingly.

- Proposal 12: For SSB based L1 measurement of the serving cell collision with UL transmission, the scenario is covered by RAN1 agreement, no RAN4 scheduling restriction requirement is needed.

### Impact on CSI-RS based measurements

#### Issue 2-3-3: Requirements for CSI-RS based measurement Recommended WF - RAN4 to discuss the impact of SBFD operation on the CSI-RS measurement requirements.

For CSI-RS based L1 measurement, we will go through several RAN1 agreements and share our thoughts on the RRM requirement impact.

At RAN1#117 several agreements were reached for CSI-RS:

For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

There appears to be a risk of CSI-RS being punctured if it falls outside the usable DL PRBs, which may impact the CSI-RS-based measurement requirements. However, our understanding is that the existing CSI-RS measurement accuracy requirements stipulate that the CSI-RS resource must be transmitted over a bandwidth greater than a specified threshold, such as the bandwidth of CSI-RS is 48 PRBs and the density is 3. Even if some PRBs containing the CSI-RS are punctured, if the total configured CSI-RS resources exceed the bandwidth threshold after punctuation, measurement accuracy should be maintained. This can be ensured through appropriate network configuration. By comparing the RAN4 minimum requirement bandwidth thresholds to the maximum number of NZP-CSI-RS resources that can be configured, we believe this issue can be effectively addressed with proper network configuration.

- Observation 2: For CSI-RS based L1 measurement, the transmitted bandwidth can be guaranteed by network configuration to avoid impact on the measurement accuracy.

- Proposal 13: For aperiodic CSI-RS based L1 measurement, RAN4 can further discuss whether the legacy requirement can be reused.

For CSI-RS colliding with the UL transmission (case 1 and case 2), RAN1 reached agreement with the priority rules at RAN1#116.

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met. - The above does not imply link direction indication is supported - FFS on dynamically scheduled DL reception with repetition

If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported - FFS on dynamically scheduled UL transmission with repetition

In our view, there are two alternatives for defining CSI-RS-based and L3 neighboring cell measurements requirements:

- Alternative 1: Reassess the RAN1 priority rules for serving cells and evaluate their applicability to neighboring cell measurements.

- Alternative 2: RAN4 specify the scenarios for collision handling and specify rules accordingly.

We believe both alternatives should be considered, as the RAN1 rules are currently based on serving cell conditions. RAN4 requirements need to assess these rules on a case-by-case basis while also identifying scenarios that could benefit from new priority rules.

- Proposal 14: For CSI-RS-based L3 measurements, in cases where collisions with uplink transmissions occur, RAN4 should further discuss the handling of such collisions while taking consideration of the RAN1 priority rules.

### Impact on other RRM requirements

#### Issue 2-3-5: Requirements for RACH requirements Recommended WF - RAN4 to discuss whether RACH requirements are impacted due to SBFD operation based on RAN1 agreements. #### Issue 2-3-6: Requirements for UL PC and or spatial relation update Recommended WF - RAN4 to discuss whether requirements for UL PC and or spatial relation switch are impacted due to SBFD operation based on RAN1 agreements. #### Issue 2-3-7: Requirements for MG and BWP switch Recommended WF - RAN4 to discuss whether to clarify the term “UL slot” in MG and BWP switch requirements when SBFD operation is enabled. #### Issue 2-3-8: Requirements for UL resource muting Recommended WF - RAN4 further to discuss whether additional RRM requirements would be defined for UL resource muting for PUSCH feature. #### Issue 2-3-9: Requirements for generic SBFD operation Recommended WF - RAN4 to discuss whether there is any RRM impact e.g. to scheduling restrictions/interruptions requirements due to SBFD operation.

Many of the listing issues are still waiting for RAN1 progress for example the RACH RO validation, the UL resources muting for PUSCH etc. We suggest RAN4 to wait for the further conclusion from RAN1 and decide whether there will further RRM requirement impacts.

For the UL slot definition, we do see there is a need to clarify the UL slot and the UL transmission within the SBFD slot. However, this wording update can wait until the CR phase.

- Proposal 15:For RACH impact, UL resources muting and spatial relation update, RAN4 can wait for RAN1 further conclusion.

- Proposal 16: For UL slot definition and wording update within the TS 38.133, the issue can be addressed during the CR phase.

#### Issue 2-3-10: Limits on the maximum number of DL/UL switching Recommended WF RAN4 to discuss whether to put limits on the maximum number of DL/UL switching during SBFD slots within the SBFD periodicity

According to the RAN1#117 agreement regarding TDD configuration, a maximum of two types of TDD patterns can be configured. Each TDD pattern defines the maximum number of DL/UL configurations. Based on our understanding, the maximum number of DL/UL switching has already been addressed.

For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

- Proposal 17: Maximum number of DL/UL switching shall follow the RAN1 agreement on TDD pattern configuration, there is no need to be discussed in RAN4.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: For L1-CLI-RSSI the Rel-16 CLI-RSSI requirement framework can be used as a baseline. Further updates can be made when RAN1 finalized the measurement RS configuration and method.

	Proposal 2: Rel-19 SBFD L1-CLI measurement requirement simulation shall focus on L1-SRS-RSRP measurement accuracy evaluation.

	Proposal 3: For L1-SRS-RSRP, new measurement requirements shall focus on Intra-cell UE-to-UE CLI scenario only.

	Proposal 4: For L1 CLI measurement reporting, the existing L1-RSRP aperiodic reporting requirement can be re-used as a baseline.

	Proposal 5: For measurement report mapping the Rel-19 CLI measurement reporting should be based on the simulation outcome.

	Proposal 6: No differential SS-RSRP for L1 reporting and L3 reporting is needed.

	Proposal 7: For L1-SRS-RSRP, the Rel-16 scheduling availability for FR1 can be reused as a baseline.

	Proposal 8: The FR2 L1-SRS-RSRP and FR1 and FR2 L1-CLI-RSSI scheduling availability requirements can wait for RAN1 progress.

	Proposal 9: RAN4 can initiate discussions on the measurement requirements for L1-SRS-RSRP and L1-CLI-RSSI based on the Rel-16 legacy QCL assumption.

	Proposal 10: Further requirements can be introduced if RAN1 agrees on further UE capability for spatial domain CLI mechanism and RX beam configuration in FR2.

	Proposal 11: For SSB based L3 measurement on neighbor cell collision with UL transmission, RAN4 to wait for RAN1 progress.

	Proposal 12: For SSB based L1 measurement of the serving cell collision with UL transmission, the scenario is covered by RAN1 agreement, no RAN4 scheduling restriction requirement is needed.

	Proposal 13: For aperiodic CSI-RS based L1 measurement, RAN4 can further discuss whether the legacy requirement can be reused.

	Proposal 14: For CSI-RS-based L3 measurements, in cases where collisions with uplink transmissions occur, RAN4 should further discuss the handling of such collisions while taking consideration of the RAN1 priority rules.

	Proposal 15:For RACH impact, UL resources muting and spatial relation update, RAN4 can wait for RAN1 further conclusion.

	Proposal 16: For UL slot definition and wording update within the TS 38.133, the issue can be addressed during the CR phase.

	Proposal 17: Maximum number of DL/UL switching shall follow the RAN1 agreement on TDD pattern configuration, there is no need to be discussed in RAN4.

# Appendix

## L1-SRS-RSRP simulation assumption

The performance evaluation for SRS-RSRP measurement accuracy according to DL measurement timing error and cyclic shift of SRS is based on the link level simulation, and simulation assumptions are summarized in Table 4 -1.

UE0 is the measurement UE connecting to cell 0 and UE1 is the SRS transmission UE connecting to cell0. UE 0 measures SRS-RSRP based on SRS transmitted by UE1 of cell 0.

The simulation shall assume that the UE is not receiving PDSCH on the symbols where it is performing SRS-RSRP measurement. At least UE0 and UE1 are visible in the simulation.

The simulation should consider noise-only scenario first and further discuss the need for interference scenario.

Table 2-1 L1-SRS-RSRP simulation assumptions:

| Parameters |  | Value |  |
| --- | --- | --- | --- |
|  |  | UE0 (cell 0) | UE1 (cell 0) |
| Carrier frequency |  | 4GHz for FR1, 30GHz for FR2 |  |
| Subcarrier spacing |  | 15kHz, 30kHz,[60kHz] for FR1 60kHz, 120kHz for FR2 | same as UE0 |
| SRS configuration | Number of port | N/A | 1 |
|  | TransmissionComb |  | 2 |
|  | Cyclic shift |  | 0 |
|  | StartPosition (SRS symbol start position) |  | 0 |
|  | Number of symbols |  | [1,2,3,4] |
|  | Number of RB Note1 |  | [20, 40] |
|  | Frequency domain position and shift |  | 0 |
|  | Periodicity [slot] |  | 5 |
| DL measurement timing errorNote2 |  | Depending on UE1 | [0, 1, 2, 10]usec for FR1 [0, 0.5, 1, 5]usec for FR2 |
| CP Length |  | Normal |  |
| Number of measurement samples in L1 averaging |  | [1, 3, 5] | N/A |
| Number of UE receive antennas |  | 2 Rx | N/A |
| Propagation conditions |  | CDL-C TDL-A10, [TDL-B400], TDL-C100 for FR1, TDL-A75, [TDL-C300] for FR2 |  |
| SRS Es/Ioc |  | [-3~0] dB | N/A |
| Note 1: Allocated RBs are within active BWP for measurement UE Note 2: Companies are encouraged to provide proper DL measurement timing error. Serving cell DL timing to the Arrival time of SRS Note 3: No frequency hopping is assumed |  |  |  |

### 4.2 L1-SRS-RSRP performance metric

The CDF curves (including the 95% and 5% CDF points) for delta L1-SRS-RSRP.

For performance metric comparison:

- Delta SRS-RSRP = [estimated SRS-RSRP – ideal SRS-RSRP] dB

No impairment margin should be included for provided results.

Note 1: The definition of L1-SRS-RSRP is the same as CSI-RSRP defined in TS38.215.

# References

- R4-2413900 “WF on SBFD RRM”, Huawei, June ,2024

- RP-241614 “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, June ,2024