3GPP TSG-RAN WG4 Meeting #112 bis	R4-2416168

Hefei, China, October 14th to October 18th, 2024

Agenda Item:	6.15.3

Source: 	Ericsson

Title:	On Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

# Introduction

From RAN #103 meeting the WID for RRM enhancement phase 5 is listed in [1]. The RAN4#112 WF is listed in [2] In this paper we provide our view for the remaining open issues.

Background: WID objective #2

- Fast SCell activation for UE supporting Rel-18 EMR

- Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting

- Apply fast scell activation in FR1 and FR2-1

- Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

# Discussion

## Scope of fast Scell activation

Scell activation is a feature that has long evolution with several variations since LTE times. EMR was introduced during Rel-16 to facilitate an early configuration for CA from network to the UE.

Currently there are 3 options to be used as the starting point of Scell activation procedures for fast Scell activation for UE supporting Rel-18 EMR.

- RRC signaling based Direct Scell activation at Scell addition

- Single MAC CE based DL Scell activation

- Single MAC CE based PUCCH Scell addition.

Issue 1-1-3: Scope of fast SCell activation for UE supporting Rel-18 eEMR

Agreement: • Start from the following scope of fast SCell activation for UE supporting Rel-18 EMR: o Direct SCell Activation at SCell addition o FFS: Single MAC CE based DL SCell Activation o FFS: Single MAC CE based PUCCH SCell Activation • RAN4 to discuss whether and how to apply the solutions to other SCell activation procedures after RAN4 has conclusions for the above cases.

EMR measurements and reporting are supported for UE in RRC_Idle or RRC_Inactive mode. From activation timeline aspect, the difference between the Direct Scell activation at Scell addition and the MAC CE based DL Scell activation have different time. This can be seen from Figure 1.

Figure 1 EMR report Idle/Inactive and Scell activation timeline RRC and MAC-CE

The figure illustrates the time window from the EMR report to the Scell activation command. The EMR report can be sent from msg3(Inactive) or UE information Response (Idle). The Scell activation command can be sent from network from 1st RRC_ reconfiguration message (Direct Scell activaiton) to MAC CE command (MAC-CE based DL Scell activation/PUCCH Scell activation).

One main aspect to define the PUCCH Scell activation requirement scenario is whether UE has a valid TA upon receiving the activation command.

If UE has a valid TA, UE shall perform the DL Scell activation based on the Rel-18 Scell activation with the L3 reporting specified TS38.133 clause 8.3. with update of the Tuncertainty_MAC. Then UE needs to estimate the Pathloss and report the CSI.

If UE does not have a valid TA, UE shall first perform the MAC CE based DL Scell activation based on requirements from TS 38.133 clause 8.3.2 with update of the Tuncertainty_MAC, then UE needs extra time to obtain a valid TA, estimate pathloss and report CSI.

To consider the situation Scell activation via EMR where UE has just transit from Idle/Inactive mode, the UE almost always has a valid TA, in this case the PUCCH Scell activation delay can be seen from clause 8.3.12

If the UE has a valid TA for transmitting on an SCell then the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated on the PUCCH SCell no later than in slot n+THARQ+Tactivation_time+max ((TFirst_available_CSI +TCSI_processing), 3*Ttarget_PL-RS)+TCSI_Reporting_afterNR slot length, Where: - A TA is considered to be valid provided that the TimeAlignmentTimer [2] associated with the TAG containing the PUCCH SCell is running. - THARQ (in ms) is the timing between DL data transmission and acknowledgement as specified in TS 38.213 [3]. - Tactivation_time is the SCell activation delay in millisecond. For the UE capable of l3-MeasUnknownSCellActivation, if the UE is provided with ReportOnScellActivation and it reports valid L3 measurement results after receiving the SCell activation command for unknown SCell, the Tactivation_time is the SCell activation delay with L3 reporting in millisecond as specified in section 8.3.17 except the definition of Tuncertainty_MAC is replaced with: - Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to - SCell activation command for known case; - First valid L3-RSRP reporting for unknown case, when UE reports valid L3-RSRP; - First valid L1-RSRP reporting for unknown case, when UE does not report L3-RSRP results; Otherwise, Tactivation_time is the SCell activation delay in millisecond as specified in section 8.3.2 except the definition of Tuncertainty_MAC is replaced with: - Tuncertainty_MAC is the time period between reception of the last activation command for PDCCH TCI, PDSCH TCI (when applicable), UL spatial relation (for FR2) relative to - SCell activation command for known case; - First valid L1-RSRP reporting for unknown case. - Ttarget_PL-RS is the periodicity of the target pathloss reference signal determined during PUCCH SCell activation. - TFirst_available_CSI is the delay uncertainty (in ms) in acquiring the first available downlink CSI reference resource. - TCSI_processing is the UE processing time for CSI reporting. - TCSI_reporting_after is the delay uncertainty (in ms) in acquiring the first available CSI reporting resources after end of max ((TFirst_available_CSI + TCSI_processing), 3*Ttarget_PL-RS)

Procedure wise, PUCCH Scell activation will always need to activate the DL Scell first, then estimate the pathloss and report CSI.

- Observation 1: MAC CE based PUCCH Scell activation always need to go through DL Scell activation first.

How the Rel-18 EMR can enhance the single MAC CE based DL Scell activation will impact PUCCH Scell activation in very similar way.

- Proposal 1: Both Direct Scell activation at Scell addition and Single MAC CE based DL Scell activation can be considered as starting point.

#### Issue 1-1-4: Target scenarios of fast SCell activation

Proposals - Proposal 1: - RAN4 shall specify scenarios where the delay is less than 100 ms, even close to 20 ms (RRC setup/resume delay). - Fast SCell activation for UE supporting Rel-18 EMR should aim to achieve less than 100ms activation delay. RAN4 to discuss the options how this is achieved. - Proposal 2: - Suggest to focus on the following scenario for fast SCell activation for UE supporting Rel-18 EMR: The SCell being activated belongs to FR2 and there is no active serving cell on that FR2 band provided that PCell or PSCell is FR1. - Proposal 3: - it is proposed to reduce SCell activation delay for unknown case in FR1 and FR2-1.

To our observation, the WID clearly stated the target scenarios of fast Scell activation, we don’t see the motivation to further discuss this issue.

- Proposal 2: Fast scell activation for UE supporting Rel-18 EMR shall include scenarios both in FR1 and FR2-1.

## Aspects on Scell activation delay

In regarding how the delay requirements can be updated for UE supporting Rel-18 EMR report, we would like to discuss the 3 main aspects that we think may impact the Scell activation delay.

- SNR condition

- Known condition window

- UE indication to the network

## 2.2.1 SNR condition

Issue 1-2-2: Consideration on SINR condition during the whole procedure for known SCell activation

Proposals - Option 1: - RAN4 to discuss whether it’s necessary to define a consistent SINR condition during the whole procedure for known SCell activation requirement (including EMR measurement in IDLE/Inactive and known SCell activation in RRC connected mode), e.g., SINR>=-2dB

Firstly, we recognize the issue that the SSB Ês/Iot for SS-RSRP and SS-RSRQ of the EMR measurement is different from many side conditions defined for legacy Scell activation from Rel-15 to Rel-18 where the side condition is Ês/Iot ≥ -2dB.

To our understanding this side condition for Scell activation Ês/Iot ≥ -2dB is a generic term that not specifically related with SSB.

This can be reflected from the test case for Scell activation for example A4.5.3 table A4.5.3.1.1-4 cell specific test parameters. The Ês/Iot is set to 17dB to test the Scell activation delay performance. To our understanding, the Ês/Iot ≥ -2dB is a side condition from UE point of view to guarantee the target Scell radio condition to specify the minimum requirement. It is different from the SSB Ês/Iot for RSRP/RSRQ EMR measurement.

The EMR SSB Ês/Iot side condition defined for measurement shall not impact or interfere with the cell specific SNR condition for Scell activation.

Typically, there is a SSB related side condition on top of the cell specific side condition for Scell activation. This can also be seen based on the known condition of the Scell activation for example in TS 38.133 Clause 8.3.2.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3.

Also to notice the SSB remains detectable specified in clause 9.2 and 9.3 is generally based on a lower side condition in comparing with the EMR measurement SSB side condition.

### 9.2.2 Requirements applicability The requirements in clause 9.2 apply, provided: - The cell being identified or measured is detectable. An intra-frequency cell shall be considered detectable when for each relevant SSB: - SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band, - SS-RSRQ related side conditions given in clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band, - SS-SINR related side conditions given in clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band, - SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

This can be seen from the table 1 below summarized all the SSB Ês/Iot side condition specified in TS 38.133 clause 10.1.

| Measurements | FR group | Legacy SSB detectable Ês/Iot side condition | EMR measurement SSB Ês/Iot side conditions |
| --- | --- | --- | --- |
| RSRP Intra-frequency | FR1 | -6 dB | -4 dB |
| RSRP Intra-frequency | FR2 | -6 dB | -4 dB |
| RSRP Inter-frequency | FR1 | -6 dB | -4 dB |
| RSRP Inter-frequency | FR2 | -4 dB | -4 dB |
| RSRQ Intra-frequency | FR1 higher accuracy | -3 dB | 3 dB |
|  | FR1 relaxed accuracy | -6 dB | -4 dB |
| RSRQ Intra-frequency | FR2 higher accuracy | -3 dB | 3 dB |
|  | FR2 relaxed accuracy | -6 dB | -4 dB |
| RSRQ Inter-frequency | FR1 higher accuracy | -3 dB | 3 dB |
|  | FR1 relaxed accuracy | -6 dB | -4 dB |
| RSRQ Inter-frequency | FR2 higher accuracy | -3 dB | 3 dB |
|  | FR2 relaxed accuracy | -4 dB | -4 dB |

Table 1 summary of the SSB Ês/Iot side condition comparison between legacy and EMR

- Observation 2: Scell activation SNR condition Ês/Iot≥ -2dB is cell specific.

- Observation 3: The EMR SSB Ês/Iot can always guarantee the legacy Scell activation SSB remain detectable Ês/Iot side condition defined in TS 38.133.

- Proposal 2: Scell activation SNR Ês/Iot≥ -2dB can be kept unchanged.

## 2.2.2 Known condition window

Issue 1-2-3: Consideration on measurement period condition of FR1 known SCell activation

Proposals - Option 1: - RAN4 to discuss whether and how to change the measurement period condition of FR1 known SCell activation (i.e., 2400ms in current requirement) for fast SCell activation with EMR.

To our understanding, the side condition for measurement period is defined based on the connected mode deactivated scell measurement cycle measCycleScell.

When UE in connected mode has been activated then being deactivated, the measurement cycle is used for deactivated Scell measurement for power saving purpose. When this measurement cycle is larger than 2400ms, the UE cannot maintain the timing and needs an extra sample for fine time tracking if UE is being activated again.

This can be reflected from TS38.133 clause 8.3.2 legacy requirements as well.

If the SCell is known and belongs to FR1, Tactivation_time is: - TFirstSSB+ 5ms, if the measurement period of the SCell being activated is equal to or smaller than 2400ms. - TFirstSSB_MAX + Trs + 5ms, if the measurement period of the SCell being activated is larger than 2400ms.

Fast Scell activation using EMR main targeting scenario is the first-time activation for UE which switching from the Idle/Inactive mode to Connected mode.

The EMR measurements will use its own measurement configuration, not the measurement cycle configured for connected mode measurements.

In this case, UE will always go through the random-access procedure before the Scell activation command. The most critical issue here is not how long the previous measurement cycle, but how long the TA timer that guarantee the UE can maintain the TA value.

- Observation 4:  The legacy FR1 known condition window max (5*measCycleSCell, 5*DRX cycles) cannot be used for UE supporting EMR.

In regarding whether the EMR report can be used as a valid measurement report, the time window for MAC CE based Scell activation and Direct Scell activation will be different. This can be seen from figure 2 and figure 3, from setting requirement point of view, the window to define the known condition also needs to be different.

Figure 2 MAC CE based 1st time Scell activation

- Proposal 3:New known condition window can be introduced for MAC-CE based fast Scell activation via EMR.

For FR1 and FR2 MAC CE based Scell activation, we think the known condition to support EMR report shall be considered to cover the time duration from msg1 to the time UE receives the activation command. The Rel-18 validity duration window ends when the UE transmit the RACH preamble. The validity timer defines the UE report behavior. By introducing a new duration can clarifies the UE behavior and requirement applicability.

Before msg 1 the measurement report behavior is controlled by the validity timer.

After msg1 whether the EMR measurement report is valid as a normal L3 report is governed by the known side condition that will be introduced in Rel-19.

The potential specification change can be reflected as highlighted in yellow.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. For UE supporting fast Scell activation via EMR, During the period from UE transmit the RACH preamble to the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated - The measurement report considered being valid if following conditions are met: - …… - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown.

- Proposal 4: Known condition time window for MAC CE based DL Scell activation using EMR can start from the point UE transmits RACH preamble to the time UE receives the MAC CE activation command.

For RRC based direct Scell activation, we can see, the measCycleScell is not used to define the known cell conditions, see TS 38.133 clause 8.3.4.

The SCell in FR1 is known provided the following conditions are met for the SCell: - During the last 5 seconds before the reception of the direct SCell configuration command: - the UE has sent a valid measurement report for the SCell being directly activated, and - the SSB measured remains detectable according to the cell identification conditions specified in sections 9.2 and 9.3, - the SSB measured during the period equal to [5] seconds also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise, the SCell is unknown.

- Figure 3 RRC based Direct Scell activation at Scell addition

Here the known condition window will most likely be overlapping with the EMR validity timer since the time from UE transmits RACH preamble to the time of first RRC reconfiguration shall always be shorter than 5s. We think the legacy 5s can be reused to define the known condition window since the report behaviour will not be controlled by the 5s condition. The known cell requirements apply when UE does report. Otherwise, the known conditions do not apply.

- Proposal 5:For RRC based direct Scell activation the legacy 5 seconds known condition window can be re-used.

## 2.2.3 UE indication to the network

#### Issue 1-2-4: Whether the indication to network is needed

Proposals - Option 1: - The Rel-19 RRM enhancement in fast scell activation for UE support Rel-18 EMR shall not only be RAN4 side condition update, certain activation fast or slow indication to the network is needed.

As we have pointed out in the SNR condition discussion, the side condition that widely used in Scell activation Ês/Iot ≥ -2dB is not always SSB related. To successfully activate a candidate Scell fast, the UE must rely on the real time measurements. The measurement report and the side condition are to guarantee the minimum requirements.

At the time when UE sends the EMR report either in msg3 or UE information response, the UE shall have very good knowledge about the target Scell based on the latest measurement of the target cell and UE shall have very good sync with the target Scell after receiving the msg2 for RAR.

The situation is very different for the network. When network has configured several candidate EMR carriers for UE to measure, network has no knowledge whether this UE will have the Scell activated successfully or not until the network receives the CSI report.

This issue can become more complex in real time network since network needs to serve many UEs with different capabilities. The UE who can indicate a fast traffic flow can be easily prioritized by the network. This will benefit both the network and the UE.

A simple indication to the network whether this candidate cell can be activated fast or not can be beneficial for the UE as well since the UE can be prioritized by the network. The network doesn’t have to blindly activate the UE for scell without knowing whether it will be successful or whether it will take very long time especially in FR2.

Since RAN4 specification known and unknown condition is for UE requirement, this is transparent to the network. This fast and not fast definition can be reflected from the known and unknown condition specified in fast Scell activation via EMR. This can be a very good alignment between the RAN4 minimum requirement and the RAN2 signaling and procedure.

- Observation 5: For fast scell activation using EMR a simple activation to the network can be beneficial to both the UE and the network.

- Proposal 6: RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: Both Direct Scell activation at Scell addition and Single MAC CE based DL Scell activation can be considered as starting point.

	Proposal 2: Fast scell activation for UE supporting Rel-18 EMR shall include scenarios both in FR1 and FR2-1.

	Proposal 3:New known condition window can be introduced for MAC-CE based fast Scell activation via EMR.

	Proposal 4: Known condition time window for MAC CE based DL Scell activation using EMR can start from the point UE transmits RACH preamble to the time UE receives the MAC CE activation command.

	Proposal 5:For RRC based direct Scell activation the legacy 5 seconds known condition window can be re-used.

	Proposal 6: RAN4 to introduce a UE indication to the network. The fast or not definition can refer to the known/unknown condition that will be defined for fast Scell activation via EMR.

# References

- RP-240830 “New WID: WI resulting from discussion on RRM enhancements”, VIVO, March 2024

- R4-2414041 “RRM pha5 part II WF”, CATT, August 2024