3GPP TSG-RAN WG4 Meeting #112bis							R4-2416174

Hefei, China, October 14th-18th 2024

Agenda Item:	6.8.2.1

Source:	Ericsson

Title:	On Co-existence simulation pre-liminary results for NR-NTN and NB-IoT-NTN HP UE

Document for:	Information

# 1	Introduction

In the last RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#112 meeting have been captured in WF [2].

This contribution provides co-existence simulation pre-liminary results for the two agreed cases 4 and 5 where TN UL and TN DL is the victim respectively.

# 2	Discussion

In the following sections, we present our pre-liminary results based on the agreed simulation parameters in WF [2].

## 2.1	Case 4 – TN UL as victim and NR NTN/ NB-IoT NTN UL as aggressor

In this sub-section, we share the pre-liminary simulation results between NR-NTN/ NB-IoT NTN HPUE interfering TN UL with Power class 2,1.5 and 1. In this scenario, NTN HPUE is transmitting as aggressor and TN gNB receiving as victim. The two NTN scenarios considered are GEO and LEO1200.

The isolation distance is assumed according to TR 38.863 between NTN UEs and the edge of TN cluster. The isolation distance is 1500m (2*ISD) and the ISD for Urban Macro is 750m. NTN UEs are placed at the NTN cluster edge. This section is further divided based on the assumption of with/ without isolation distance and deployment scenarios.

Some of the other assumptions are as following:

- Isolation distance is taken as 1.5 km

- With wrap around

- Statistics are based on Option 2: TN sectors which have NTN UEs at their edges

- NR NTN/ IoT-NTN HPUE ratio is 100%

- NTN UE NF is 9 dB

- NB-IoT NTN UL UEs and SCS – 9 UEs for 15 kHz

#### 2.1.1	With Isolation distance

##### 2.1.1.1	NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 | 12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 10,0 | 8,0 | 7,0 | 5,0 | 4,0 |  |  |
|  |  |  | 5%-tile | 11,0 | 8,0 | 3,0 | 2,0 | 2,0 |  |  |
|  | PC1.5 | Throughput Loss | Average | 14,00 | 12,00 | 9,00 | 8,00 | 6,00 | 4,00 |  |
|  |  |  | 5%-tile | 27,00 | 16,00 | 11,00 | 6,00 | 2,00 | 2,00 |  |
|  | PC1 | Throughput Loss | Average | 17,0 | 14,0 | 12,0 | 9,0 | 8,0 | 6,0 |  |
|  |  |  | 5%-tile | 41,0 | 27,0 | 16,0 | 11,0 | 6,0 | 2,0 |  |
| LEO1200 | PC2 | Throughput Loss | Average |  |  | 6,00 | 5,00 | 4,00 | 3,00 | 2,00 |
|  |  |  | 5%-tile |  |  | 9,0 | 4,0 | 1,0 | 0,0 | 0,0 |
|  | PC1.5 | Throughput Loss | Average |  |  | 9,00 | 7,00 | 6,00 | 4,00 | 3,00 |
|  |  |  | 5%-tile |  |  | 13,00 | 10,00 | 7,00 | 3,00 |  |
|  | PC1 | Throughput Loss | Average |  |  |  |  |  | 6,0 | 4,0 |
|  |  |  | 5%-tile |  |  |  |  |  | 7,0 | 3,0 |

Case 4 - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 8dB from 26 dBm to 31 dBm UE transmission power.

Case 4 - For LEO1200 in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 11dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.1.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 4,0 | 3,0 | 3,0 | 2,0 | 1,0 | 1,0 |
|  |  |  | 5%-tile | 4,0 | 2,0 | 1,0 | 1,0 | 1,0 | 1,0 |
|  | PC1.5 | Throughput Loss | Average | 8,00 | 6,00 | 4,00 | 3,00 | 2,00 | 1,00 |
|  |  |  | 5%-tile | 14,00 | 6,00 | 2,00 | 1,00 | 1,00 | 1,00 |
|  | PC1 | Throughput Loss | Average | 11,0 | 8,0 | 6,0 | 4,0 | 3,0 | 2,0 |
|  |  |  | 5%-tile | 18,0 | 14,0 | 6,0 | 2,0 | 1,0 | 1,0 |
| LEO1200 | PC2 | Throughput Loss | Average | 4.4 | 2,0 | 2,0 |  |  |  |
|  |  |  | 5%-tile | 3.1 | 3.1 | 1.1 |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 7,0 | 5,0 | 4,0 | 3,0 |  |  |
|  |  |  | 5%-tile | 6,0 | 4,0 | 3,0 | 2,0 |  |  |
|  | PC1 | Throughput Loss | Average | 10,00 | 7,00 | 5,00 | 4,00 |  |  |
|  |  |  | 5%-tile | 11,00 | 6,00 | 4,00 | 3,00 |  |  |

Case 4 - For GEO in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 5dB from 26 dBm to 31 dBm UE transmission power.

Case 4 - For LEO1200 in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 3dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.1.3	NR NTN; TN BS type – AAS; Deployment Scenario – Rural

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 0,0 | 0,0 | 0,0 |  |  |  |
|  |  |  | 5%-tile | 0,0 | 0,0 | 0,0 |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 0,0 | 0,0 | 0,0 |  |  |  |
|  |  |  | 5%-tile | 0,0 | 0,0 | 0,0 |  |  |  |
|  | PC1 | Throughput Loss | Average | 0,0 | 0,0 | 0,0 |  |  |  |
|  |  |  | 5%-tile | 0,0 | 0,0 | 0,0 |  |  |  |

Case 4 - For GEO in NR NTN Rural scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss cannot be derived.

##### 2.1.1.4	NR NTN; TN BS type – Non-AAS; Deployment Scenario – Rural

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 0,0 | 0,0 | 0,0 |  |  |  |
|  |  |  | 5%-tile |  |  |  |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 0,0 | 0,0 | 0,0 |  |  |  |
|  |  |  | 5%-tile |  |  |  |  |  |  |
|  | PC1 | Throughput Loss | Average | 0,0 | 0,0 | 0,0 |  |  |  |
|  |  |  | 5%-tile |  |  |  |  |  |  |

Case 4 - For GEO in NR NTN Rural scenario with isolation distance and Non-AAS TN BS, ACIR at 5%tile throughput loss cannot be derived.

##### 2.1.1.5	NB-IoT NTN; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | SNR Loss | Average | 1.5 | 1.2 | 0.9 |  |  |  |
|  |  |  | 5%-tile | 0.6 | 0.3 | 0.2 |  |  |  |
|  | PC1.5 | SNR Loss | Average | 2.2 | 1.7 | 1.3 | 1.04 | 0.8 |  |
|  |  |  | 5%-tile | 1.1 | 0.7 | 0.4 | 0.3 | 0.2 |  |
|  | PC1 | SNR Loss | Average | 2.7 | 2.2 | 1.7 | 1.4 | 1.05 | 0.8 |
|  |  |  | 5%-tile | 1.3 | 1.1 | 0.70 | 0.4 | 0.3 | 0.2 |
| LEO1200 | PC2 | SNR Loss | Average | 1.7 | 1.3 | 1,0 | 0.7 |  |  |
|  |  |  | 5%-tile | 0.4 | 0.3 | 0.2 | 0.1 |  |  |
|  | PC1.5 | SNR Loss | Average | 2.4 | 1.9 | 1.5 | 1.1 | 0.9 | 0.6 |
|  |  |  | 5%-tile | 0.8 | 0.5 | 0.3 | 0.2 | 0.1 | 0.1 |
|  | PC1 | SNR Loss | Average | 3,0 | 2.4 | 1.9 | 1.5 | 1.1 | 0.9 |
|  |  |  | 5%-tile | 1.5 | 0.8 | 0.5 | 0.3 | 0.2 | 0.1 |

Case 4 - For GEO in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss are close to 0 or cannot be derived.

Case 4 - For LEO1200 in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss are close to 0 or cannot be derived.

#### 2.1.2	Without Isolation distance

##### 2.1.2.1	NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 5,0 | 4,0 | 3,0 | 2,0 | 2,0 | 1,0 |  |
|  |  |  | 5%-tile | 6,0 | 6,0 | 6,0 | 4,0 | 2,0 | 2,0 |  |
|  | PC1.5 | Throughput Loss | Average | 6,0 | 5,0 | 4,0 | 3,0 | 2,0 | 1,0 |  |
|  |  |  | 5%-tile | 10,0 | 9,0 | 7,0 | 6,0 | 6,0 | 3,0 |  |
|  | PC1 | Throughput Loss | Average |  |  | 5,00 | 4,00 | 3,00 | 2,00 | 2,00 |
|  |  |  | 5%-tile |  |  | 10,00 | 7,00 | 6,00 | 6,00 | 3,00 |
| LEO1200 | PC2 | Throughput Loss | Average |  |  | 3,0 | 3,0 | 2,0 | 1,0 |  |
|  |  |  | 5%-tile |  |  | 10,0 | 5,0 | 2,0 | 1,0 |  |
|  | PC1.5 | Throughput Loss | Average |  |  |  |  | 3,0 | 2,0 | 2,0 |
|  |  |  | 5%-tile |  |  |  |  | 9,0 | 3,0 | 2,0 |
|  | PC1 | Throughput Loss | Average |  |  |  |  | 4,00 | 3,00 | 2,00 |
|  |  |  | 5%-tile |  |  |  |  | 13,00 | 9,00 | 3,00 |

Case 4 - For GEO in NR NTN Urban scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 39dB from 26 dBm to 31 dBm UE transmission power.

Case 4 - For LEO1200 in NR NTN Urban scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 39dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.2.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 2,0 | 2,0 | 1,0 | 1,0 | 1,0 | 1,0 | 0,0 |
|  |  |  | 5%-tile | 8,0 | 7,0 | 4,0 | 1,0 | 1,0 | 1,0 | 0,0 |
|  | PC1.5 | Throughput Loss | Average | 3,0 | 2,0 | 2,0 | 1,0 | 1,0 | 1,0 | 1,0 |
|  |  |  | 5%-tile | 13,00 | 9,00 | 7,00 | 5,00 | 1,00 | 1,00 | 1,00 |
|  | PC1 | Throughput Loss | Average | 4,00 | 3,00 | 2,00 | 2,00 | 1,00 | 1,00 | 1,00 |
|  |  |  | 5%-tile | 15,00 | 13,00 | 9,00 | 7,00 | 5,00 | 1,00 | 1,00 |
| LEO1200 | PC2 | Throughput Loss | Average |  | 3,0 | 3,0 | 2,0 | 2,0 | 1,0 | 1,0 |
|  |  |  | 5%-tile |  | 8,0 | 5,0 | 3,0 | 3,0 | 3,0 | 3,0 |
|  | PC1.5 | Throughput Loss | Average |  | 5,0 | 4,0 | 3,0 | 2,0 | 2,0 | 1,0 |
|  |  |  | 5%-tile |  | 19,0 | 10,0 | 7,0 | 3,0 | 3,0 | 3,0 |
|  | PC1 | Throughput Loss | Average |  | 6,00 | 4,00 | 4,00 | 3,00 | 2,00 | 2,00 |
|  |  |  | 5%-tile |  | 33,00 | 19,00 | 9,00 | 7,00 | 3,00 | 3,00 |

Case 4 - For GEO in NR NTN Urban scenario without isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 36dB from 26 dBm to 31 dBm UE transmission power.

Case 4 - For LEO1200 in NR NTN Urban scenario without isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 36dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.2.3	NR NTN; TN BS type – AAS; Deployment Scenario – Rural

| GEO | Power Class |  | Required ACIR (dB) | 10 | 12 | 14 | 16 | 18 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 3 | 2 | 2 | 1 |  |  |  |
|  |  |  | 5%-tile | 6 | 4 | 3 | 3 |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 4 | 3 | 2 | 1 | 1 |  |  |
|  |  |  | 5%-tile | 13 | 9 | 5 | 3 | 3 |  |  |
|  | PC1 | Throughput Loss | Average |  |  |  | 2 | 2 |  |  |
|  |  |  | 5%-tile |  |  |  | 5 | 3 |  |  |

Case 4 - For GEO in NR NTN Rural scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 16dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.2.4	NR NTN; TN BS type – Non AAS; Deployment Scenario – rural

| GEO | Power Class |  | Required ACIR (dB) | 4 | 6 | 8 | 10 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 7,0 | 6,0 | 4,0 | 3,0 | 3,0 | 2,0 | 2,0 |
|  |  |  | 5%-tile |  |  |  |  |  |  |  |
|  | PC1.5 | Throughput Loss | Average |  | 8,0 | 6,0 | 5,0 | 3,0 | 3,0 | 2,0 |
|  |  |  | 5%-tile |  |  |  |  |  |  |  |
|  | PC1 | Throughput Loss | Average |  | 10,00 | 8,00 | 6,00 | 5,00 | 4,00 | 3,00 |
|  |  |  | 5%-tile |  |  |  |  |  |  |  |

Case 4 - For GEO in NR NTN Rural scenario without isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 12dB from 26 dBm to 31 dBm UE transmission power.

##### 2.1.2.5	NB-IoT NTN; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | SNR Loss | Average | 2.3 | 1.9 | 1.5 | 1.2 | 0.9 | 0.7 |  |  |
|  |  |  | 5%-tile | 2.5 | 2,0 | 1.3 | 1.1 | 0.7 | 0.5 |  |  |
|  | PC1.5 | SNR Loss | Average |  |  | 2.1 | 1.7 | 1.3 | 1.1 | 0.8 | 0.6 |
|  |  |  | 5%-tile |  |  | 2.1 | 1.5 | 1.1 | 0.8 | 0.6 | 0.5 |
|  | PC1 | SNR Loss | Average |  |  | 2.6 | 2.1 | 1.7 | 1.3 | 1.1 | 0.8 |
|  |  |  | 5%-tile |  |  | 3.2 | 2.2 | 1.5 | 1.1 | 0.9 | 0.6 |
| LEO1200 | PC2 | SNR Loss | Average |  |  | 1.3 | 1.1 | 0.9 | 0.7 | 0.6 | 0.5 |
|  |  |  | 5%-tile |  |  | 1.8 | 1.4 | 1,0 | 0.8 | 0.6 | 0.4 |
|  | PC1.5 | SNR Loss | Average |  |  | 1.8 | 1.5 | 1.2 | 1,00 | 0.8 | 0.6 |
|  |  |  | 5%-tile |  |  | 3.4 | 2.2 | 1.5 | 1.2 | 1,00 | 0.8 |
|  | PC1 | SNR Loss | Average |  |  | 1.8 | 1.5 | 1.2 | 1,00 | 0.8 | 0.6 |
|  |  |  | 5%-tile |  |  | 5.2 | 3.4 | 2.2 | 1.5 | 1.3 | 1,00 |

Case 4 - For GEO in NB-IoT NTN Urban scenario without isolation distance, ACIR at 5%tile SNR loss increases to 31dB from 26 dBm to 31 dBm UE transmission power.

Case 4 - For LEO1200 in NB-IoT NTN Urban scenario without isolation distance, ACIR requirements at 5%tile SNR loss increases to 34dB from 26 dBm to 31 dBm UE transmission power.

## 2.2	Case 5 – TN DL as victim and NTN UL as aggressor

#### 2.2.1	With Isolation distance

##### 2.2.1.1	NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |

Case 5 - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

##### 2.2.1.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |

Case 5 - For GEO in NR NTN Urban scenario with isolation distance and Non-AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

##### 2.2.1.3	NR NTN; TN BS type – AAS; Deployment Scenario – Rural

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |

Case 5 - For GEO in NR NTN Rural scenario with isolation distance and AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

##### 2.2.1.4	NR NTN; TN BS type – Non AAS; Deployment Scenario – Rural

| GEO | Power Class |  | Required ACIR (dB) | 0 | 2 | 4 | 6 | 8 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |
|  | PC1 | Throughput Loss | Average | 0,00% | 0,00% | 0,00% |  |  |  |
|  |  |  | 5%-tile | 0,00% | 0,00% | 0,00% |  |  |  |

Case 5 - For GEO in NR NTN Rural scenario with isolation distance and Non-AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

#### 2.2.2	Without Isolation distance

##### 2.2.2.1	NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 8 | 10 | 12 | 14 | 16 | 18 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 2,00% | 2,00% | 1,00% |  |  |  |  |
|  |  |  | 5%-tile | 8,00% | 4,00% | 4,00% |  |  |  |  |
|  | PC1.5 | Throughput Loss | Average |  | 2,00 | 2,00 | 1,00 | 1,00 |  |  |
|  |  |  | 5%-tile |  | 8,00 | 4,00 | 4,00 | 4,00 |  |  |
|  | PC1 | Throughput Loss | Average |  | 3,00% | 2,00% | 2,00% | 1,00% |  |  |
|  |  |  | 5%-tile |  | 12,00% | 8,00% | 4,00% | 4,00% |  |  |

Case 5 - For GEO in NR NTN Urban scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 14 dB from 26 dBm to 31 dBm UE transmission power.

##### 2.2.1.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 2 | 4 | 6 | 8 | 10 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 4,00% | 3,00% | 3,00% | 2,00% | 2,00% | 1,00% |  |  |
|  |  |  | 5%-tile | 12,00% | 12,00% | 5,00% | 3,00% | 3,00% | 1,00% |  |  |
|  | PC1.5 | Throughput Loss | Average | 6,00 | 5,00 | 4,00 | 3,00 | 2,00 | 2,00 |  |  |
|  |  |  | 5%-tile | 18,00 | 12,00 | 12,00 | 7,00 | 5,00 | 3,00 |  |  |
|  | PC1 | Throughput Loss | Average |  |  | 4,00% | 4,00% | 3,00% | 2,00% | 2,00% | 1,00% |
|  |  |  | 5%-tile |  |  | 12,00% | 12,00% | 7,00% | 5,00% | 3,00% | 2,00% |

Case 5 - For GEO in NR NTN Urban scenario without isolation distance and Non-AAS TN BS, ACIR at 5%tile throughput loss increases to 12 dB from 26 dBm to 31 dBm UE transmission power.

##### 2.2.1.3	NR NTN; TN BS type – AAS; Deployment Scenario – Rural

| GEO | Power Class |  | Required ACIR (dB) | 4 | 6 | 8 | 10 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 1,00% | 1,00% | 1,00% | 1,00% |  |  |  |
|  |  |  | 5%-tile | 6,00% | 6,00% | 4,00% | 3,00% |  |  |  |
|  | PC1.5 | Throughput Loss | Average |  | 1,00 | 1,00 | 1,00 | 1,00 | 1,00 |  |
|  |  |  | 5%-tile |  | 6,00 | 5,00 | 4,00 | 4,00 | 3,00 |  |
|  | PC1 | Throughput Loss | Average |  | 1,00% | 1,00% | 1,00% | 1,00% | 1,00% | 1,00% |
|  |  |  | 5%-tile |  | 6,00% | 4,00% | 4,00% | 4,00% | 4,00% | 3,00% |

Case 5 - For GEO in NR NTN Rural scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 8 dB from 26 dBm to 31 dBm UE transmission power.

##### 2.2.1.4	NR NTN; TN BS type – Non AAS; Deployment Scenario – Rural

| GEO | Power Class |  | Required ACIR (dB) | 6 | 8 | 10 | 12 | 14 | 16 | 18 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 1,00% | 1,00% | 1,00% | 1,00% |  |  |  |  |
|  |  |  | 5%-tile | 9,00% | 4,00% | 3,00% | 3,00% |  |  |  |  |
|  | PC1.5 | Throughput Loss | Average | 1,00 | 1,00 | 1,00 | 1,00 | 1,00 |  |  |  |
|  |  |  | 5%-tile | 9,00 | 8,00 | 5,00 | 4,00 | 3,00 |  |  |  |
|  | PC1 | Throughput Loss | Average | 1,00% | 1,00% | 1,00% | 1,00% | 1,00% | 1,00% |  |  |
|  |  |  | 5%-tile | 8,00% | 8,00% | 4,00% | 4,00% | 4,00% | 3,00% |  |  |

Case 5 - For GEO in NR NTN Rural scenario without isolation distance and Non-AAS TN BS, ACIR at 5%tile throughput loss increases to 8 dB from 26 dBm to 31 dBm UE transmission power.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Case 4 - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 8dB from 26 dBm to 31 dBm UE transmission power.

Observation 2	Case 4 - For LEO1200 in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 11dB from 26 dBm to 31 dBm UE transmission power.

Observation 3	Case 4 - For GEO in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 5dB from 26 dBm to 31 dBm UE transmission power.

Observation 4	Case 4 - For LEO1200 in NR NTN Urban scenario with isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 3dB from 26 dBm to 31 dBm UE transmission power.

Observation 5	Case 4 - For GEO in NR NTN Rural scenario with isolation distance and AAS TN BS, ACIR at 5%tile throughput loss cannot be derived.

Observation 6	Case 4 - For GEO in NR NTN Rural scenario with isolation distance and Non-AAS TN BS, ACIR at 5%tile throughput loss cannot be derived.

Observation 7	Case 4 - For GEO in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss are close to 0 or cannot be derived.

Observation 8	Case 4 - For LEO1200 in NB-IoT NTN Urban scenario with isolation distance, ACIR requirements at 5%tile SNR loss are close to 0 or cannot be derived.

Observation 9	Case 4 - For GEO in NR NTN Urban scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 39dB from 26 dBm to 31 dBm UE transmission power.

Observation 10	Case 4 - For LEO1200 in NR NTN Urban scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 39dB from 26 dBm to 31 dBm UE transmission power.

Observation 11	Case 4 - For GEO in NR NTN Urban scenario without isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 36dB from 26 dBm to 31 dBm UE transmission power.

Observation 12	Case 4 - For LEO1200 in NR NTN Urban scenario without isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 36dB from 26 dBm to 31 dBm UE transmission power.

Observation 13	Case 4 - For GEO in NR NTN Rural scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 16dB from 26 dBm to 31 dBm UE transmission power.

Observation 14	Case 4 - For GEO in NR NTN Rural scenario without isolation distance and Non AAS TN BS, ACIR at 5%tile throughput loss increases to 12dB from 26 dBm to 31 dBm UE transmission power.

Observation 15	Case 4 - For GEO in NB-IoT NTN Urban scenario without isolation distance, ACIR at 5%tile SNR loss increases to 31dB from 26 dBm to 31 dBm UE transmission power.

Observation 16	Case 4 - For LEO1200 in NB-IoT NTN Urban scenario without isolation distance, ACIR requirements at 5%tile SNR loss increases to 34dB from 26 dBm to 31 dBm UE transmission power.

Observation 17	Case 5 - For GEO in NR NTN Urban scenario with isolation distance and AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

Observation 18	Case 5 - For GEO in NR NTN Urban scenario with isolation distance and Non-AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

Observation 19	Case 5 - For GEO in NR NTN Rural scenario with isolation distance and AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

Observation 20	Case 5 - For GEO in NR NTN Rural scenario with isolation distance and Non-AAS TN BS, ACIR requirements at 5%tile throughput loss are close to 0 or cannot be derived.

Observation 21	Case 5 - For GEO in NR NTN Urban scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 14 dB from 26 dBm to 31 dBm UE transmission power.

Observation 22	Case 5 - For GEO in NR NTN Urban scenario without isolation distance and Non-AAS TN BS, ACIR at 5%tile throughput loss increases to 12 dB from 26 dBm to 31 dBm UE transmission power.

Observation 23	Case 5 - For GEO in NR NTN Rural scenario without isolation distance and AAS TN BS, ACIR at 5%tile throughput loss increases to 8 dB from 26 dBm to 31 dBm UE transmission power.

Observation 24	Case 5 - For GEO in NR NTN Rural scenario without isolation distance and Non-AAS TN BS, ACIR at 5%tile throughput loss increases to 8 dB from 26 dBm to 31 dBm UE transmission power.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4-2414452,”WF on UE RF requirements for NTN HPUE”, 3GPP RAN4#112, August 2024