3GPP TSG-RAN WG4 Meeting #112bis							R4-2416175

Hefei, China, October 14th-18th 2024

Agenda Item:	6.8.2.1

Source:	Ericsson

Title:	On Co-existence simulation assumption considerations for NR-NTN and NB-IoT-NTN HP UE

Document for:	Discussion

# 1	Introduction

In the last RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#112 meeting have been captured in WF [2].

This contribution intends to trigger further discussion on co-existence studies assumptions focusing on adjacent channel interference between NTN and TN networks based on the Release – 17 NTN co-existence studies [3].

# 2	Co-existence simulation assumptions

### 2.1	NTN Co-existence scenarios

Based on our pre-liminary results in [4], the ACIR requirements as worst case are derived when no isolation distance is considered in Case 4. The ACIR requirements however get relaxed with the introduction of isolation distance of 1.5 km, possibly due to reduced interference.

RAN4 to prioritize Case 4 when TN UL is victim for co-existence evaluation, considering it as the worst-case.

### 2.2	TN Co-existence scenarios

Based on our pre-liminary co-existence evaluation in [4], we observe that the ACIR requirements are derived majorly when TN UL is impacted in Case 4 Urban Macro scenario which can be considered as the worst case.

When no isolation distance is considered in Case 4 for both Urban Macro and Rural scenarios, both the scenarios show similar ACIR requirements.

In Case 4 when TN UL is the victim, both Urban Macro and Rural scenarios show similar ACIR requirements when no isolation distance is considered.

### 2.3	Isolation distance

In our companion contribution [4], we have presented our pre-liminary results with/ without isolation distance of 1.5 km.

Case 4 when TN UL is the victim and NR NTN/ NB-IoT NTN UL is the aggressor,

- In GEO and LEO1200 Urban Macro scenarios when no isolation distance is considered we see an ACIR upto 39 dB, whereas with isolation distance the ACIR reduces to 11 dB.

- In GEO Rural scenarios when no isolation distance is considered we see an ACIR upto 39 dB, whereas with isolation distance the ACIR reduces to 8 dB.

Case 5 when TN DL is the victim and NR NTN/ NB-IoT NTN UL is the aggressor,

- In GEO Urban Macro when no isolation distance is considered we see an ACIR upto 14 dB, whereas with isolation distance the ACIR is close to 0 or cannot be derived.

- In GEO Rural when no isolation distance is considered we see an ACIR upto 12 dB, whereas with isolation distance the ACIR is close to 0 or cannot be derived.

Based on the results, the ACIR requirements as worst case are derived when no isolation distance is considered in Case 4. The ACIR requirements however get relaxed with the introduction of isolation distance of 1.5 km, possibly due to reduced interference.

Further studies can be carried out to check the impact of reduced isolation distance on ACIR requirements other than 1.5km in Case 4.

With the assumption of isolation distance of 1.5km, pre-liminary co-existence evaluation shows relaxed ACIR requirements for both Case 4 and 5.

### 2.4	NTN HPUE ratio

It is important to consider the worst case when deciding on other ratio values to reduce simulation efforts and possible combinations. We believe that the worst case would be when all NTN UEs are HPUE with full buffer traffic model, causing maximum interference effect in the network.

RAN4 to consider all NR NTN/ NB-IoT NTN UEs as HPUEs, taking it as the worst-case scenario.

### 2.5	TN network topology – with/ without wrap around functionality

In our companion contribution [4], we have presented our pre-liminary results with wrap around functionality.

The wrap around of TN is used to derive the SINR of victim TN. When wrap around is not implemented in simulation studies, there is a possibility to see unrealistically higher coverage and better SINR pointing to over optimistic results. When wrap around is implemented, the effect of interference is statistically equivalent across the simulation area. It is viable to say that the effect of SINR may be more prominent for the cell edge UEs since they will operate in low SINR region.

If the other companies are certain to disable wrap around, one option is to only evaluate the TN BS in the “central 19 TN cells cluster”, and not evaluate the TN BS in “wrap around TN cells”. With this, the wrap around TN cells is only needed to calculate the TN SINRs and do not affect the NTN UE. Hence, they will not be treated as interfered TN BS.

RAN4 to prioritize wrap around functionality as it impacts the derivation of SINR for victim TN BS.

### 2.6	IoT NTN UE UL number and SCS

In our pre-liminary simulation results, we have considered 9 UEs as IoT-NTN UEs in UL with SCS as 15 KHz. Considering Case 4 when TN UL is victim, the worst ACIR requirements is 34 dBm when considering PC1 without isolation distance. However, the ACIR requirements cannot be derived or are close 0 when isolation distance is considered. 
On that basis, we don’t see big impact on ACIR requirements when considering more number of IoT NTN UP UEs or smaller SCS in these scenarios.

We don’t see big impact on ACIR requirements with more number of IoT NTN UL UEs and smaller SCS when isolation distance is considered.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In Case 4 when TN UL is the victim, both Urban Macro and Rural scenarios show similar ACIR requirements when no isolation distance is considered.

Observation 2	Further studies can be carried out to check the impact of reduced isolation distance on ACIR requirements other than 1.5km in Case 4.

Observation 3	We don’t see big impact on ACIR requirements with more number of IoT NTN UL UEs and smaller SCS when isolation distance is considered.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN4 should prioritize Case 4 when TN UL is victim for co-existence evaluation, considering it as the worst-case.

Proposal 2	With the assumption of isolation distance of 1.5km, pre-liminary co-existence evaluation shows relaxed ACIR requirements for both Case 4 and 5.

Proposal 3	RAN4 to consider all NR NTN/ NB-IoT NTN UEs as HPUEs, taking it as the worst-case scenario.

Proposal 4	RAN4 to prioritize wrap around functionality as it impacts the derivation of SINR for victim TN BS.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4-2414452,”WF on UE RF requirements for NTN HPUE”, 3GPP RAN4#112, August 2024

- 3GPP TR 38.863 v18.3.0: “Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects”

- R4-2416174, “On Co-existence simulation assumption considerations for NR-NTN and NB-IoT-NTN HP UE”, 3GPP RAN4#112bis, Ericsson