3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416177

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon, China Unicom, Orange

Title: 	On power domain enhancements for single carrier

Agenda Item:	6.1.1.2.1

Document for:	Approval

Introduction

During RAN4#112, power domain enhancement for single carrier has been discussed. Following conclusions can be found in [1]. In this contribution, we would like to share our views on this topic concentrating on following aspects:

- Scenario classification based on the worldwide spectrum allocation

- Consideration on the MPR reduction solution

Discussion

- Scenario classification based on the worldwide spectrum allocation

As RAN4 agreed to study below three scenarios, it would be beneficial to look closely at each one and take the worldwide spectrum allocation and deployment into account, so that the corresponding solution can be more targeted.

In the following part, we will try to enumerate the suitable examples under each scenario. What needs to be noted is that the spectrum allocation/deployment may not be static by the time passage where spectrum policy and business decision could weight in, so we only explicitly mark the region and waive the operator instead.

The distinction between Scenario 1-1 and Scenario 1-2

For these two scenarios, we understand the key criteria to distinguish Scenario 1-1 from Scenario 1-2 is whether there is any other operator(s) close to the holding spectrum. Specifically, the Scenario 1-1 stands for the case where a sufficient gap (at each side) exists between two allocations of spectrum (not shorter than the half of the indicated maximum UE channel bandwidth or the indicated OOB boundary shifting offset for UE to apply inner MPR to out RB allocations). Otherwise, it should be Scenario 1-2. Note that the case where the holding spectrum is larger than 100MHz should be included as well, because besides the gap there are operator’s own frequencies that can be used as offset for shifting the OOB boundary.

Observation 1: The key criteria to distinguish Scenario 1-1 from Scenario 1-2 is whether there is sufficient gap between operator holding spectrums.

- The gap (at each side) is sufficient If it is larger than the half of the indicated maximum UE channel bandwidth or the indicated OOB boundary shifting offset for UE to apply inner MPR to out RB allocations.

- The case where the concerned holding spectrum is larger than 100MHz should be included as well.

Accordingly, we can screen out the matched ones based on the current spectrum allocation and list them in Table 2. One interesting finding is that for a same band e.g. n78, it can belong to different scenario due to certain spectrum allocation and deployment.

Table 1. On how the scenario belonging would change for band n78 in different region

| Region | Spectrum | Matched Scenario |
| --- | --- | --- |
| China |  | 1-1 (300MHz) |
| Romania |  | 1-2 (115MHz, at least 15MHz at each side can be used as offset) |

Table 2. On the operator holding spectrum that fits Scenario 1-1 or Scenario 1-2

| Region | NR band | Holding spectrum for UL | Matched Scenario | Note |
| --- | --- | --- | --- | --- |
| Australia | n7 | 2510MHz ~ 2550MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>2550MHz is occupied by adjacent operator) |
| Britain | n40 | 2350MHz ~ 2390MHz 40MHz | 1-1 | Single operator |
| China | n28 | 703MHz ~ 733MHz 30MHz | 1-1 | Co-constructed and shared by operators |
|  | n5 | 825MHz ~ 835MHz 10MHz | 1-1 | Single operator |
|  | n39 | 1885MHz ~ 1915MHz 30MHz | 1-2 | Insufficient gap (5MHz) adjacent to this CBW |
|  | n1 | 1920MHz ~ 1965MHz 45MHz | 1-2 | Co-constructed and shared by operators Insufficient gap (15MHz) adjacent to this CBW |
|  | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
|  | n40 | 2300MHz ~ 2370MHz 70MHz | 1-1 | Single operator |
|  | n41 | 2515~2675MHz 160MHz | 1-1 | Single operator |
|  | n78 | 3300~3600MHz 300MHz | 1-1 | Co-constructed and shared by operators |
|  | n79 | 4800~4960MHz 160MHz | 1-1 | Co-constructed and shared by operators |
| Croatia | n78 | 3480MHz ~ 3600MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (>3600MHz is occupied by adjacent operator) |
| Denmark | n1 | 1920MHz ~ 1940MHz 20MHz | 1-2 | Insufficient gap adjacent to this CBW (>1940MHz is occupied by adjacent operator) |
|  | n40 | 2300MHz ~ 2400MHz 100MHz | 1-1 | Single operator |
|  | n78 | 3410MHz ~ 3540MHz 130MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
|  | n78 | 3540MHz ~ 3660MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (<3540MHz is occupied by adjacent operator) |
| Finland | n78 | 3410MHz ~ 3540MHz 130MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
| Greek | n78 | 3410MHz ~ 3450MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>3450MHz is occupied by adjacent operator) |
| Hungary | n78 | 3410MHz ~ 3470MHz 60MHz | 1-2 | Insufficient gap adjacent to this CBW (>3470MHz is occupied by adjacent operator) |
| Ireland | n40 | 2300MHz ~ 2360MHz 60MHz | 1-1 | Single operator |
| Lithuania | n40 | 2310MHz ~ 2390MHz 80MHz | 1-1 | Single operator |
| Luxembourg | n78 | 3420MHz ~ 3530MHz 110MHz | 1-2 | Insufficient gap adjacent to this CBW (>3540MHz is occupied by adjacent operator) |
|  | n78 | 3540MHz ~ 3650MHz 110MHz | 1-2 | Insufficient gap adjacent to this CBW (<3530MHz and >3650MHz are occupied by adjacent operator, respectively) |
|  | n78 | 3650MHz ~ 3750MHz 100MHz | 1-2 | Insufficient gap adjacent to this CBW (<3650MHz is occupied by adjacent operator and 3800MHz is the edge of n78) |
| Netherland | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
| Norway | n78 | 3400MHz ~ 3480MHz 80MHz | 1-1 | Sufficient gap (100MHz) adjacent to this CBW |
|  | n78 | 3580MHz ~ 3700MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (>3700MHz is occupied by adjacent operator) |
| Romania | n78 | 3700MHz ~ 3750MHz 50MHz | 1-1 | Sufficient gap (>=50MHz) adjacent to this CBW |
|  | n78 | 3490MHz ~ 3530MHz 40MHz | 1-2 | Insufficient gap adjacent to this CBW (>3530MHz is occupied by adjacent operator) |
|  | n78 | 3530MHz ~ 3645MHz 115MHz | 1-2 | Insufficient gap adjacent to this CBW (<3530MHz and >3700MHz are occupied by adjacent operator, respectively) |
| Slovenia | n34 | 2010MHz ~ 2025MHz 15MHz | 1-1 | Single operator |
| Spain | n28 | 703MHz ~ 733MHz 30MHz | 1-1 | Co-constructed and shared by operators |
|  | n78 | 3420MHz ~ 3500MHz 80MHz | 1-2 | Insufficient gap adjacent to this CBW (<3420MHz is occupied by adjacent operator) |
| Sweden | n78 | 3400MHz ~ 3500MHz 100MHz | 1-2 | Insufficient gap adjacent to this CBW (>3500MHz is occupied by adjacent operator) |
|  | n78 | 3500MHz ~ 3620MHz 120MHz | 1-2 | Insufficient gap adjacent to this CBW (<3500MHz is occupied by adjacent operator) |
| Switzerland | n78 | 3500MHz ~ 3580MHz 80MHz | 1-2 | Insufficient gap adjacent to this CBW (>3580MHz is occupied by adjacent operator) |

On Scenario 2

For this scenario, there is a previous agreement that the shifted boundary of where ACLR and SEM applied cannot exceed the BS CBW in order to avoid additional interference to adjacent operators. Hence, any spectrum allocation with no gap to other operator in adjacent channels can be incorporated to Scenario 2. But one should focus on the case where the operator holding spectrum is larger than 100MHz instead and it doesn’t need to confine to single carrier operation at BS side because:

- The multi-carriers deployment could have a disposition to configure 100MHz to one CC as it could be a practical decision after considering achievable peak throughputs and overall coverage performance along with the maximum CBW capability normally indicated by an eMBB UE i.e. 100MHz.

- But if it is arbitrarily confined to single carrier, to accommodate the required offset e.g. 1/2 UE CBW into holding spectrum, the gNB may have to configure the UE with a smaller CBW if it wants to enable this feature.

- The reduced MPR is still the same as the original one for inner region, which can be achieved under inner RB allocations scheduling.

- Since a smaller UE CBW is configured, the original outer RB allocation cannot be scheduled anymore.

Figure 1. Illustration on the drawbacks when confining to single carrier at BS side

Proposal 1: As depicted in Figure 1, for Scenario 2, one should focus on the case where the operator holding spectrum is larger than 100MHz and it is multi-carriers operation at BS side. Because:

- For >100MHz contiguous spectrum, it could have a disposition to configure 100MHz to one CC as it could be a practical decision after considering achievable peak throughputs and overall coverage performance along with the maximum CBW capability normally indicated by an eMBB UE i.e. 100MHz.

- But if it is arbitrarily confined to single carrier, to accommodate the required offset e.g. 1/2 UE CBW into holding spectrum, the gNB may have to configure the UE with a smaller CBW rather than the supported maximum UE CBW, if it wants to enable this feature.

- The reduced MPR is still the same as the one that can be achieved on the very same RB allocations, which are identical to the inner RB allocations within the original maximum UE CBW.

- Since a smaller UE CBW is configured, the original outer RB allocation cannot be scheduled anymore.

- Last but not the least, the motivation of this feature is to unleash the UE MPR potential but not power saving.

Then we can list the qualified cases in below table.

Table 3. On the operator holding spectrum that fits Scenario 2

| Region | NR band | Holding spectrum for UL | Note |
| --- | --- | --- | --- |
| Any kind of spectrum allocation that the BS sees fit for extended UE CBW |  |  | Under a certain spectrum allocation, the BS may need to reconfigure the UE CBW in order to accommodate the extended UE CBW. |
| Finland | n78 | 3540MHz ~ 3670MHz 130MHz | No gap adjacent to this CBW (<3540MHz and >3670MHz are occupied by adjacent operator, respectively) |
| Greek | n78 | 3450MHz ~ 3600MHz 150MHz | No gap adjacent to this CBW (<3450MHz and >3600MHz are occupied by adjacent operator, respectively) |
| Hungary | n78 | 3540MHz ~ 3660MHz 120MHz | No gap adjacent to this CBW (<3540MHz and >3660MHz are occupied by adjacent operator, respectively) |
| Slovenia | n78 | 3560MHz ~ 3700MHz 140MHz | No gap adjacent to this CBW (<3560MHz and >3700MHz are occupied by adjacent operator, respectively) |
| Spain | n78 | 3600MHz ~ 3710MHz 110MHz | No gap adjacent to this CBW (<3600MHz and >3710MHz are occupied by adjacent operator, respectively) |
| Switzerland | n78 | 3580MHz ~ 3700MHz 120MHz | No gap adjacent to this CBW (<3580MHz and >3700MHz are occupied by adjacent operator, respectively) |

- Consideration on the MPR reduction solution

In the light of our above analysis, we provide a general solution in the following part, while a supplementary solution is specially considered for Scenario 2.

General solution

- How to relax the out-of-band requirements

As the expected MPR reduction is essentially achieved by proper relaxation on OOB requirements, which can be accomplished through relaxed value and/or shifted applicable range, we prefer to shift the boundary to apply ACLR and SEM instead of considering relaxed ACLR value or lifted SEM mask. Further, the integral region of OOBE should be based on the UE CBW still. Subsequently, the boundary to apply SE needs to be shifted accordingly. Otherwise the UE wouldn’t obtain a true relaxation. Simple and intuitive, the reduced MPR can be achieved by this way on the outer RB allocation considering different adjacent channel situation.

Proposal 2: The integral region and the boundary of ACLR/SEM should still be based on UE CBW.

Proposal 3: Confirm that the application range of SE should be altered with the extended UE CBW.

- Necessary information exchange between the UE and NW to enable MPR reduction

The required offset for UE to serve inner region MPR in all outer RB allocations of the UE CBW should be known by the NW. Firstly, it is symmetrically extended from the edge of UE CBW. Secondly, to consider 1/2 UE CBW as default value can be a start point, while UE indication on such capability can be further discussed.

Proposal 4: For the required offset for UE to serve inner region MPR in all outer RB allocations of the UE CBW:

- Symmetrically extended from the edge of UE CBW, it is the start point where ACLR/SEM requirements apply.

- The default ratio between it and the UE CBW can be 1/2, while it can be up to UE capability for other values which shall be smaller than the default one.

Then from NW perspective, it can determine whether and how to enable this feature based on the holding spectrum and adjacent channel situation. When the gap can be enough for the extended UE CBW e.g. two times UE CBW, the NW can directly enable this feature.

Proposal 5: For the NW to determine whether to enable Rel-19 MPR reduction when there is non-zero gap at least on one side of the concerning operator holding spectrum:

- The frequency span composed by the concerning operator holding spectrum and the gap can accommodate the extended UE CBW without overlapping with the adjacent operator holding spectrum (if any).

A supplementary solution for Scenario 2

Targeting Scenario 2, the gNB can have a choice other than reconfiguring a smaller UE CBW, then a broader “new inner region” can be obtained since the shifted boundary can reach to the edge of BS RF CBW at most. In this sense, more RB allocations can enjoy the MPR gain. Following figure can better explain it.

Figure 2. Illustration on the supplementary solution for Scenario 2

(taking 110MHz holding spectrum as an example)

As depicted in Figure 2, additional ~7.5MHz frequency resources can be expected with inner MPR for UL transmission, while the remaining outer RB allocations can still be scheduled.

Figure 3. Illustration on the supplementary solution for Scenario 2

(taking 160MHz holding spectrum as an example)

Another example in Figure 3 is just to show that such new inner shall not exceed the indicated maximum UE CBW even the BS RF CBW is larger enough. In conclusion, we have the following proposal.

Proposal 6: For the cases where there is no gap in the adjacent channels, use the edge of BS RF CBW as the shifted boundary of ACLR/SEM. The new inner region is defined as the union of region A and B, where:

- A is the legacy inner region calculated under the indicated maximum UE CBW.

- B is the inner region calculated by using the total RB number derived from BS RF CBW.

- The new inner shall not exceed the indicated maximum UE CBW.

System level simulation on including BS side multi-carriers operation for MPR reduction

In TR 36.942, the assumption for determining co-existence requirements i.e. UE ACLR who dominates the ACIR is captured as a 3%~5% throughput loss tolerance. By taking this experience value as metric, we evaluate the co-existence performance on applying MPR reduction to BS side multi-carriers operation with following considerations:

- For the BS side multi-carriers operation, it is two contiguous CCs (same RB length) in the same band.

- For a victim PC3 UE on CC1, the aggressor is another PC3 UE on CC2 that will apply MPR reduction by shifting the ACLR boundary overlapping with CC1.

- As the ACLR boundary shifting away, the interference in the first adjacent channel (the overlapping part in CC1) is increased. Following figure can better explain it.

Figure 4. Illustration on why the interference in the 1st adjacent channel is increased

- As depicted in Figure 4, since the PSD would be lower in the “new” 1st adjacent channel and the same ACLR value applies in that integral region, there must be a room for the aggressor to deliver higher output power by utilizing reduced MPR. Thus the interference landed in CC1 will increase.

- By observing the throughput loss on CC1 with different shifted ACLR boundary, we can identify the offset when the loss hits e.g. 3%.

|  | Throughput loss. ACLR boundary; @50% CDF; @90% CDF. Shift to 1/5 UE CBW; 0.88%; 1.43%. Shift with 1/2 UE CBW; 2.26%; 2.78%. |
| --- | --- |

Figure 5. Comparison on the CDF curve of throughput when different ACLR boundary is applied

From the above results, we can find that the throughput loss due to reduced MPR is acceptable when the ACLR integral region is overlapping with the adjacent CC.

Observation 2: Taking 3% throughput loss as the metric, the interference to the 1st adjacent channel (overlapping with another CC right next to the concerned CC) caused by a UE enables MPR reduction is acceptable.

Conclusion

In this contribution we discussed on the Rel-19 MPR reduction for single CC. We have the following proposals:

Observation 1: The key criteria to distinguish Scenario 1-1 from Scenario 1-2 is whether there is sufficient gap between operator holding spectrums.

- The gap (at each side) is sufficient If it is larger than the half of the indicated maximum UE channel bandwidth or the indicated OOB boundary shifting offset for UE to apply inner MPR to out RB allocations.

- The case where the concerned holding spectrum is larger than 100MHz should be included as well.

Proposal 1: As depicted in Figure 1, for Scenario 2, one should focus on the case where the operator holding spectrum is larger than 100MHz and it is multi-carriers operation at BS side. Because:

- For >100MHz contiguous spectrum, it could have a disposition to configure 100MHz to one CC as it could be a practical decision after considering achievable peak throughputs and overall coverage performance along with the maximum CBW capability normally indicated by an eMBB UE i.e. 100MHz.

- But if it is arbitrarily confined to single carrier, to accommodate the required offset e.g. 1/2 UE CBW into holding spectrum, the gNB may have to configure the UE with a smaller CBW rather than the supported maximum UE CBW, if it wants to enable this feature.

- The reduced MPR is still the same as the one that can be achieved on the very same RB allocations, which are identical to the inner RB allocations within the original maximum UE CBW.

- Since a smaller UE CBW is configured, the original outer RB allocation cannot be scheduled anymore.

- Last but not the least, the motivation of this feature is to unleash the UE MPR potential but not power saving.

Proposal 2: The integral region and the boundary of ACLR/SEM should still be based on UE CBW.

Proposal 3: Confirm that the application range of SE should be altered with the extended UE CBW.

Proposal 4: For the required offset for UE to serve inner region MPR in all outer RB allocations of the UE CBW:

- Symmetrically extended from the edge of UE CBW, it is the start point where ACLR/SEM requirements apply.

- The default ratio between it and the UE CBW can be 1/2, while it can be up to UE capability for other values which shall be smaller than the default one.

Proposal 5: For the NW to determine whether to enable Rel-19 MPR reduction when there is non-zero gap at least on one side of the concerning operator holding spectrum:

- The frequency span composed by the concerning operator holding spectrum and the gap can accommodate the extended UE CBW without overlapping with the adjacent operator holding spectrum (if any).

Proposal 6: For the cases where there is no gap in the adjacent channels, use the edge of BS RF CBW as the shifted boundary of ACLR/SEM. The new inner region is defined as the union of region A and B, where:

- A is the legacy inner region calculated under the indicated maximum UE CBW.

- B is the inner region calculated by using the total RB number derived from BS RF CBW.

- The new inner shall not exceed the indicated maximum UE CBW.

Observation 2: Taking 3% throughput loss as the metric, the interference to the 1st adjacent channel (overlapping with another CC right next to the concerned CC) caused by a UE enables MPR reduction is acceptable.

- References

- R4-2414441, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23, 2024.