3GPP TSG-RAN WG4 Meeting #112bis                                      R4-2416178

Hefei, Anhui, China, October 14 - 18, 2024

Source: 	Huawei, HiSilicon

Title: 	On MPR applicability for FR1 intra-band UL CA

Agenda Item:	6.1.1.2.2

Document for:	Approval

Introduction

During RAN4#112, MPR applicability for FR1 intra-band contiguous/non-contiguous UL CA has been discussed. Following conclusions can be found in [1].

- MPR applicability for FR1 intra-band contiguous UL CA - Applicable MPR for intra-band contiguous CA with single activated cell - Proposals - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung) - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE) - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C; - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated; - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent. - Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm). - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4 - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4 - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4 - WF - FFS in next meeting - Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE - Proposals - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung) - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei) - WF - FFS in next meeting - Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell - Proposals - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks) - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo) - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson) - WF - FFS in next meeting - MPR applicability for FR1 intra-band UL non-contiguous CA - General considerations - Proposals - Proposal 1: The work scope for FR1 NC CA should be limited to only study MPR applicability. (Samsung) - WF - Agree with the proposal as it is aligned with the WID - Applicable MPR for FR1 intra-band non-contiguous UL CA - Proposals - Proposal 1: for PC3 and PC2 intra-band non-contiguous CA as the standard already accounts for the use of the single CC MPR tables when only 1 CC is scheduled no further changes to the standard are required. (Qualcomm, ZTE, Huawei) - Proposal 2: When DualPA is not signalled or TxD or UL MIMO is signalled, transmit interruption may be needed to allow LO switching and the single carrier MPR can be sued when transmitted RBs are allocated in only one of the CC. (Skyworks) - Proposal 3: for non-contiguous UL CA configurations with sub-blocks consisting of one cell and supported by dual PA architecture, the non-CA (single CC) MPR applies for one cell active among the configured uplink serving cells, the other cell deactivated. (Ericsson) - Proposal 4: for non-contiguous UL CA configurations with sub-blocks consisting of one cell not supported by a dual PA architecture, applicability of the non-CA (single CC) MPR for one active cell among the configured uplink serving cells, the other cell deactivated, is subject to UE capability, e.g. indication of [mpr-singleCC-activated-FR1] for the band combination. (Ericsson) - WF - FFS in next meeting

In this contribution, we would like to share our views regarding this topic.

Discussion

- FR1 intra-band contiguous UL CA

Reviewing the discussion in last meeting, agreement on applying single CC MPR to intra-band contiguous UL CA when SCell is deactivated was almost achieved given that UE vendors can be confident in meeting NW scheduling preference when UL traffic is low. Actually, MAC CE based mechanism could provide sufficient time for UE to adjust RF resources assignment then utilizing smaller MPR can be possible.

In addition to the MPR applicability, one FFS point is whether to explicitly refer to the corresponding Tx power requirement when only 1 CC is activated. From our understanding, such change could be interpreted as introducing Rel-18 power boosting to intra-band contiguous UL CA, which is not agreed in any previous discussions (whether Rel-18 further coverage enhancement or TEI).

Proposal 1: RAN4 specifies the following MPR applicability:

- For PC3, the single CC MPR requirements (Table 6.2.2-1) apply to FR1 intra-band contiguous UL CA with only one UL CC activated.

- For PC2, the single CC MPR requirements apply to FR1 intra-band contiguous UL CA with only one UL CC activated:

  - If TxD is indicated for this intra-band contiguous UL CA, single CC with TxD MPR (Table 6.2D.2-1) should apply.

  - If TxD is not indicated for this intra-band contiguous UL CA, single CC without TxD MPR (Table 6.2.2-2) should apply.

Proposal 2: Do not refer to the corresponding configured Tx power requirements for PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

Another issue is how to handle the spurious emissions/ACLR/SEM requirements for the new MPR applicability. Intuitively, reuse the emission requirements defined for intra-band contiguous UL CA would be the most UE implementation friendly way. Otherwise, the UE that indicates dual PA capability to support intra-band contiguous UL CA could be risky to violate the SEM requirement defined for single carrier as identified in [2].

Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, reuse the spurious emissions/ACLR/SEM defined for FR1 intra-band contiguous UL CA even when only one CC is activated.

Conclusion

In this contribution we discussed on the remaining issue for Rel-19 MPR applicability for FR1 intra-band UL CA. We have the following proposals:

Proposal 1: RAN4 specifies the following MPR applicability:

- For PC3, the single CC MPR requirements (Table 6.2.2-1) apply to FR1 intra-band contiguous UL CA with only one UL CC activated.

- For PC2, the single CC MPR requirements apply to FR1 intra-band contiguous UL CA with only one UL CC activated:

  - If TxD is indicated for this intra-band contiguous UL CA, single CC with TxD MPR (Table 6.2D.2-1) should apply.

  - If TxD is not indicated for this intra-band contiguous UL CA, single CC without TxD MPR (Table 6.2.2-2) should apply.

Proposal 2: Do not refer to the corresponding configured Tx power requirements for PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated.

Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, reuse the spurious emissions/ACLR/SEM defined for FR1 intra-band contiguous UL CA even when only one CC is activated.

- References

- R4-2414441, “WF on power domain enhancements”, Huawei, HiSilicon, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23.

- R4-2411049, “On improved MPR for intra-band ULCA when only one CC has RBs allocated”, Skyworks, 3GPP TSG-RAN4 Meeting #112, Maastricht, Netherlands, August 19-23.