3GPP TSG-RAN WG4 Meeting # 112bis	R4-2416181

# Hefei, Anhui, China, October 14 - 18, 2024

Title: 	On MIMO layer evaluation for 6Rx UE

Source: 	Huawei, HiSilicon

Agenda item:	6.1.1.3.2

Document for:	Approval

Background

In last meeting, WF R4-2414285 [1] was approved, one objective is related to the feasibility of 6 layers for 6Rx UE. This contribution provides our views.

- Evaluation

In last meeting, Table 5-1 in clause 5 was agreed as simulation assumptions table for MIMO layers evaluation.

In last meeting, some companies proposed to take BS antenna correlation into account, based on our understanding, the alpha values defining in 38.101-4 is too pessimistic causing rank deficiency, even for 4layers, the target throughput is unachievable. It's hard to find a typical configuration for BS antenna correlation. Hence, we used low correlation for simulation in this meeting. Under the assumption of low correlation, the channel beam direction is omni, which means channel has sufficient space to accommodate any number of beams, so we don’t think there is performance gap between 8Tx and 16Tx. Hence, we use 8Tx for simulation.

Considering the metric is absolute throughput, which is different for 4layers and 6layers, it’s more reasonable to use adaptive MCS for comparison. However, we understand that CQI reporting implementation could be different for 4 layers and 6 layers. We also provide simulation with fixed MCS to avoid the potential CQI implementation impact. But for fairness, we tried serval MCSs for 4Layer and 6Layer respectively at some given SNR points considering 4Layers and 6Layers could have different optimal MCS to achieve high throughout.

DMRS overhead is one key issue if additional DMRS symbol is applied. Some companies proposed DMRS 2+0 for 6 layers to align the DMRS overhead with 4 layers and proposed 5Hz doppler to minimum the channel estimation performance degradation. To check the performance for such proposed case, we simulated two scenarios: Scenario 1: DMRS 1+1 for 4 layers and DMRS 2+2 for 6layers, TDLA30-10 is applied. Scenario 2: DMRS 1+1 for 4 layers and DMRS 2+0 for 6layers, TDLA30-5 is applied Scenario 3: DMRS 1+1 for 4Layers and DMRS 2+0 for 6Layers and TDLC300-100 is applied.

It is noted that Rank 5 is one candidate option, however, according to the WI, rank5 is outside the scope, we propose to only focus on 6layers and 4layers.

Other simulation parameters are reused from Table 5-1.

Tx EVM is not modelled in the simulation with adaptive MCS but modelled in the simulation with fix MCS.

Proposal 1: RAN4 should consider following assumptions for evaluation:

Low correlation at BS side

8Tx

Only Rank4 and 6

Adaptive MCS

The absolute throughput- SNR curve is shown in Figure 2-1: The upper bound SNR point for the simulation is 32.5dB corresponding to 8%EVM.

Figure 2-1: Simulation results for adaptive MCS

It is observed that there is neigligible performnace difference between two UE correlation matrixes. For both scenarios(Aligning DMRS overhead and Non-aligning DMRS overhead), 4 layers outperfomrs 6layers at SNR range the upbound of which is limited  by 8%EVM.

Observation 1: The performance difference between two UE correlation matrixes is negligible.

Observation 2: For both scenarios(Aligning DMRS overhead and Non-aligning DMRS overhead), 4 layers outperfomrs 6layers

Fixed MCS

Considering the 6layers is applicable for high SNR range, we simulated PDSCH requirements at some given high SNR points with serval MCSs for both scenario:

Scenario 1: 4 Layers: DMRS 1+1, 6Layers: DMRS 2+2, TDLA30-10. SNR: 30dB, 40dB

Scenario 2: 4 Layers: DMRS 1+1, 6Layers: DMRS 2+0, TDLA30-5. SNR: 20dB, 25dB, 30dB, 35dB

Scenario 3: 4 Layers: DMRS 1+1, 6Layers: DMRS 2+0, TDLC300-100. SNR: 35dB

At given SNR point, the absolute throughput increases first and then decreases with the increase of the MCS, so there is always optimal MCS for the given SNR. We try to find optimal MCS for 4layers and 6layers at given SNR respectively and compare the absolute throughput of 4Layers with that of 6 layers with optimal MCS for 4Layers and 6Layers respectively.

Scenario1, SNR=30dB

| MCS | 4Layer (Throughput(Mbps)) | 6Layer (Throughput(Mbps)) |
| --- | --- | --- |
| 19 | 227.3072 | 267.3760 |
| 20 | 245.2032 | 283.4923 |
| 21 | 269.2544 | 221.4845 |
| 22 | 287.3728 | 60.5730 |
| 23 | 311.3568 | 2.356 |
| 24 | 335.3408 | 0 |
| 25 | 358.8018 | 0 |
| 26 | 280.0417 | 0 |
| 27 | 88.6423 | 0 |

It can be observed that when SNR=30dB, the optimal MCS for 4Layer is 25, the optimal MCS for 6 Layers is 20, the maximum throughput of 4Layers is 26.4% higher than that of 6Layers.

Scenario1, SNR=40dB

| MCS | 4Layer (Throughput(Mbps)) | 6Layer (Throughput(Mbps)) |
| --- | --- | --- |
| 19 |  | 267.3760 |
| 20 |  | 295.4750 |
| 21 |  | 295.1654 |
| 22 |  | 187.7885 |
| 23 | 311.3568 | 32.0194 |
| 24 | 335.3408 | 0.5919 |
| 25 | 359.1216 | 0 |
| 26 | 363.3708 | 0 |
| 27 | 260.8754 | 0 |

It can be observed that when SNR=40dB, the optimal MCS for 4Layer is 26, the optimal MCS for 6 Layers is 20, the maximum throughput of 4Layers is 23.0% higher than that of 6Layers.

It can be observed that for scenario 1(Non-aligning DMRS overhead for 4Layers and 6Layers), even at extremely high SNR, 4Layers outperform 6Layers with considerable gain. So it can be concluded that for scenario 1, 4 Layers ourperforms much better than 6Layers.

Observation 3: For scenario 1(Non-Aligning DMRS overhead for 4Layers and 6Layers),  4 Layers outperforms much better than 6Layers, at least 23.0% throughput gain can be observed.

Scenario2, SNR=20dB (TDLA30-5)

| MCS | 4Layer(1+1) (Throughput(Mbps)) | 6Layer(2+0) (Throughput(Mbps)) |
| --- | --- | --- |
| 14 |  | 235.9256 |
| 15 |  | 238.5002 |
| 16 |  | 137.5957 |
| 17 |  | 227.5918 |
| 18 |  | 121.1695 |
| 19 | 227.3072 | 5.5566 |
| 20 | 245.2032 |  |
| 21 | 269.1438 |  |
| 22 | 227.4256 |  |
| 23 | 204.8531 |  |
| 24 | 13.5450 |  |
| 25 | 0 |  |
| 26 | 0 |  |
| 27 | 0 |  |

It can be observed that when SNR=20dB, the optimal MCS for 4Layer is 21, the optimal MCS for 6 Layers is 15, the maximum throughput of 4Layers is 12.8% higher than that of 6Layers.

Scenario2, SNR=25dB

| MCS | 4Layer (Throughput(Mbps)) | 6Layer (Throughput(Mbps)) |
| --- | --- | --- |
| 16 |  | 282.2546 |
| 17 |  | 283.2384 |
| 18 |  | 300.8843 |
| 19 | 227.3072 | 323.6557 |
| 20 | 245.2032 | 232.1224 |
| 21 | 269.2544 | 25.8820 |
| 22 | 287.3728 | 0.62 |
| 23 | 311.2922 | 0 |
| 24 | 332.3483 | 0 |
| 25 | 288.0605 | 0 |
| 26 | 9.4168 | 0 |
| 27 | 0 | 0 |

It can be observed that when SNR=25dB, the optimal MCS for 4Layer is 24, the optimal MCS for 6 Layers is 20, the maximum throughput of 4Layers is 2.69% higher than that of 6Layers.

Scenario2, SNR=30dB

| MCS | 4Layer (Throughput(Mbps)) | 6Layer (Throughput(Mbps)) |
| --- | --- | --- |
| 19 | 227.3072 | 330.5888 |
| 20 | 245.2032 | 364.3406 |
| 21 | 269.2544 | 343.0084 |
| 22 | 287.3728 | 195.8174 |
| 23 | 311.3568 | 11.9645 |
| 24 | 335.3408 | 0 |
| 25 | 358.4821 | 0 |
| 26 | 263.7251 | 0 |
| 27 | 65.7500 | 0 |

It can be observed that when SNR=30dB, the optimal MCS for 4Layer is 25, the optimal MCS for 6 Layers is 20, the maximum throughput of 6Layers is 1.69% higher than that of 4Layers.

Scenario2, SNR=35dB

| MCS | 4Layer (Throughput(Mbps)) | 6Layer (Throughput(Mbps)) |
| --- | --- | --- |
| 19 | 227.3072 | 330.5888 |
| 20 | 245.2032 | 365.8656 |
| 21 | 269.2544 | 395.6848 |
| 22 | 287.3728 | 359.0034 |
| 23 | 311.3568 | 107.1570 |
| 24 | 335.3408 | 1.1027 |
| 25 | 359.1216 | 0 |
| 26 | 346.2262 | 0 |
| 27 | 195.2573 | 0 |

It can be observed that when SNR=35dB, the optimal MCS for 4Layer is 25, the optimal MCS for 6 Layers is 21, the maximum throughput of 6Layers is 10% higher than that of 4Layers.

It can be concluded that for scenario 2(Aligning DMRS overhead for 4Layers and 6Layers), 6layers outperforms 4layers when SNR is larger than 30dB, but the gain is very limited.

Observation 4: For scenario 2(Aligning DMRS overhead for 4Layers and 6Layers with low Doppler), 6layers only  outperforms 4layers when SNR is higher than 30dB, but the gain is quite limited.

Scenaro 3, SNR=35dB

| MCS | 4Layer (Throughput(Mbps)) | 6Layer (Throughput(Mbps)) |
| --- | --- | --- |
| 10 |  | 144.6144 |
| 11 |  | 162.4366 |
| 12 |  | 186.7251 |
| 13 |  | 176.8181 |
| 14 |  | 31.6563 |
| 15 |  | 7.3421 |
| 19 | 227.5630 |  |
| 20 | 245.2032 |  |
| 21 | 269.5853 |  |
| 22 | 287.8365 |  |
| 23 | 311.6435 |  |
| 24 | 264.3351 |  |
| 25 | 1.6183 |  |
| 26 | 0 |  |
| 27 | 0 |  |

It can be observed that when SNR=30dB, the optimal MCS for 4Layer is 23, the optimal MCS for 6 Layers is 12, the maximum throughput of 6Layers is 66.6% higher than that of 4Layers.

Observation 5: It can be concluded that for scenario 3(Aligning DMRS overhead for 4Layers and 6Layers with 100Hz Doppler), 4layers much bettter outperforms than 6layers even at high SNR (SNR=35dB)

- Discussions

UE type

According to WI[1], the requirements should target at handled UE and the specified requirements can be applicable to both UE and FWA devices.

6Rx for handheld and FWA UE - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

Hence, we propose to respect the WI that MIMO layers discussions for 6Rx should be limited at handled UE. FWA already support 8Rx with 8 layers and are available on market, it’s very meaningless to discuss the feasibility of 6 layers.

Observation 6: WI specifies that the target UE type for evaluation is handled UE.

Proposal 2: RAN4 shall focus the MIMO layers discussions on handled UE.

Performance gain

Based on the outcomes from observation 1~4, in most normal cases, 4Layers outperforms much better than 6Layers with considerable gain. 4 Layers outperforms 6Layers only when following conditions are met: 1) DMRS 2+0 for 6Layers, 2) very low doppler (5Hz), 3) extremely high SNR(>30dB), and the gain is quite limited, considering target UE type is handled UE, which is always in the moving state, we think these conditions are too ideal which can’t happen in the real network.

Observation 7: 4 layers outperforms 6layers only when following conditions are met: 1) DMRS 2+0 for 6Layers, 2) very low doppler (5Hz), 3) extremely high SNR(>30dB) and gain is quite limited.

Concerns on foldable UE

People may have wrong impression about that the foldable UE could have more space to accommodate 6Rx then lead to an imprudent feasibility conclusion. For the time being, we hold following concerns:

- One should note that the foldable phone is not always unfolded, which could depend on unpredictable factors e.g. user preference and hand blockage. When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- From receiving power perspective, if the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- Actually the hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

In conclusion, we have the following proposals for the foldable UE assumed for the 6 layers feasibility study.

Proposal 3: For the foldable UE assumed for the 6 layers feasibility study, RAN4 should look into the following aspects to proceed the evaluation:

- When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

Complexity analysis

Besides performance gain, baseband complexity analysis and power consumption should also be considered. To support 2CWs, , which means complexity of decoder is doubled. Besides, dimension of channel matrix is increased from 4x6 to 6x6, the complexity of MIMO detector is also much increased, which is the module with the highest complexity increase. Besides, complexity of channel estimation and DMRS based interference estimation is also increased by 1.5 times when number DMRS ports is increased from 4 to 6. So, we strongly encourage to take baseband processing into account in addition to consider performance gains. It is not worthwhile to implement 6Layers after trading off the complexity and performance gain.

Proposal 4: RAN4 shall analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

- Conclusion

In this contribution we provide our views on MIMO layers for 6Rx, the proposals are observations are:

For simulation assumptions:

Proposal 1: RAN4 should consider following assumptions for evaluation:

Low correlation at BS side

8Tx

Only Rank4 and 6

For simulations with adaptive MCS:

Observation 1: The performance difference between two UE correlation matrixes is negligible.

Observation 2: For both scenarios(Aligning DMRS overhead and Non-aligning DMRS overhead), 4 layers outperfomrs 6layers

For simulations with fixed MCS:

Observation 3: For scenario 1(Non-Aligning DMRS overhead for 4Layers and 6Layers),  4 Layers outperforms much better than 6Layers, at least 23.0% throughput gain can be observed.

Observation 4: For scenario 2(Aligning DMRS overhead for 4Layers and 6Layers with low Doppler), 6layers only  outperforms 4layers when SNR is higher than 30dB, but the gain is quite limited.

Observation 5: It can be concluded that for scenario 3(Aligning DMRS overhead for 4Layers and 6Layers with 100Hz Doppler), 4layers much bettter outperforms than 6layers even at high SNR (SNR=35dB)

Observation 6: WI specifies that the target UE type for evaluation is handled UE.

Proposal 2: RAN4 shall focus the MIMO layers discussions on handled UE.

Observation 7: 4 layers outperforms 6layers only when following conditions are met: 1) DMRS 2+0 for 6Layers, 2) very low doppler (5Hz), 3) extremely high SNR(>30dB) and gain is quite limited.

Proposal 3: For the foldable UE assumed for the 6 layers feasibility study, RAN4 should look into the following aspects to proceed the evaluation:

- When the phone is folded, the spatially overlapped antenna could be impacted which would lead to degraded receiving performance.

- If the diversity branches are not on the main board, there could be power imbalance issue since the insertion loss would be high for the path across the hinge.

- The hinge would partially occupy the edge of each screen in order to guarantee the structural stability, hence unlike phablet at least the occupied space cannot be used for antenna placement anymore. Then adding more antenna could make the antenna correlation worse.

Proposal 4: RAN4 shall analyse the increased baseband complexity and power consumption for 6Layers compared to 4Layers including LDPC decoder, channel estimation, DMRS based interference estimation and MIMO detector.

- Reference

[1] R4-2414285 WF on 6Rx UE requirements.

[2] RP-240828 New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4

- Appendix

Table 6-1: Feasibility Study Link-Level Simulation Assumptions

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz or 100MHz |
| SCS |  | 30 |
| Antenna configuration |  | Option 1: 16Tx, 6Rx Option 2: 8Tx, 6Rx |
| Propagation channel |  | Option 1: TDLA30-5 Option 2: TDLA30-10 Option 3: TDLC300-100 |
| Rank |  | Fixed rank [4,5,6] |
| MCS |  | Option 1: Adaptive MCS (target BLER 10%) Option 2: MCS13, MCS17 from Table 1 Other MCS options are not precluded |
| PDSCH configuration | Mapping type | Type A |
|  | Starting symbol | 2 |
|  | Length | 12 |
|  | PRB bundling size | 2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | Precoding | Rel-15 Type I |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | DMRS Configurations | Option 1: [1+1(4L), 2+0(6L)] Option 2: [2+2(4L), 2+2(6L)] Option 3: [1+1 (4L), 2+2 (6L)] |
|  | Precoding | Random (further check) |
| UE Correlation matrix |  | Option 1: Option 2: 1.000.100.101.000.300.10 0.300.150.100.141.000.12 0.130.120.300.200.200.120.150.140.130.300.120.20 0.121.000.200.110.120.30 0.110.301.000.100.101.00 Option 3: Other options are not precluded. |